3GPP TSG RAN WG1 Meeting #91																R1-1721376

Reno, Nevada, USA, November 27th — December 1st, 2017

Source: 	Intel Corporation

Title:	Summary of Wednesday offline discussion for NR Radio Link Monitoring

Agenda item:	7.1.5.2

Document for:	Discussion and Decision

# Introduction

This contribution tries to summaries Wednesday’s offline discussion on the NR RLM topic during RAN1 #91.

# Summary of Offline Discussions

## 2.3 Signaling of RLM-RS

Signaling methods for the SS/PBCH Block for RLM was discussed in the last meeting. It was suggested that maximum number of configured RLM-RS should be taken into account for the design. The following are list of proposals from companies who provided views.

Following options were discussed in the contributions:

- Indicated based on actual transmitted SS blocks via bitmap (variable bitmap from 1 to X) ZTE, Huawei,  OPPO,

- Indicated based on bitmap (bitmap of either 4, 8, or 64 bits, depending on frequency): AT&T, Docomo

- Indicated based on SS/PBCH block index: Intel

The following was agreed in RAN1 #91:

Agreements: - For value of X: - For below 3GHz: X = 2 - For above 3GHz and below 6GHz: X = 4 - For above 6GHz: X = [8]

The working assumption is we have at most 8 RLM-RSs configured. If we need to index SS/PBCH block individually we would end up with maximum 8*6 bits (8 times SS/PBCH block index) + 3 bits (for number of indices). Therefore, it is suggested to adopt the following:

| Parameter Name | Description | Value Range |
| --- | --- | --- |
| RLM-RS-List | Contains the list of RLM-RSs | Up to 8 entries of { RLM-RS-Type, RLM-SSB or RLM-CSIRS} tuple |
| RLM-RS-Type | Type of one RLM-RS | CHOICE{RLM-SSB,RLM-CSIRS} For each RLM-RS |
| RLM-SSB | SSB block index of the cell-defining SSB to give which SSB is used for RLM | 0,1,2,3,…,63 |
| RLM-CSIRS | CSI-RS resource ID used for RLM | 0,1,2,3,…,7 |

The following are RRC signaling needed to support RLM-CSI-RS configurations.

| Parameter Name | Description | Value Range |
| --- | --- | --- |
| RLM-CSI-RS-ResourceConfig | RLM CSI-RS resource configuration |  |
| RLM-CSI-RS-ResourceConfigId | RLM-CSI-RS resource configuration ID | 0, …, 7 |
| Pc-hypothetical-PDCCH | EPRE offset of RLM CSI-RS RE to hypothetical PDCCH DMRS (EPRE of CSI-RS [dB] – ERPRE of hypothetical PDCCH DMRS [dB]) | 3 bits FFS: values. e.g. {-6, -4.77, -3, 0, +3, +4.77, +6}dB |
| RLM-CSI-RS ScramblingID | Scrambling ID | Adopt the parameter values agreed for CSI-RS |
| RLM-CSI-RS Numerology | Numerology for RLM-CSI-RS | Adopt the parameter values agreed in BM Common value across all RLM-CSI-RS |
| RLM-CSI-RS-Density | Density of CSI-RS resource measured in RE/port/PRB | {1, 3, 6, 12} |
| RLM-CSI-RS-FreqBand | Measurement PRB start and end value of RLM-CSI-RS | Adopt the parameter values agreed in BM with following exception: Minimum number of PRB is limited to 24. |
| RLM-CSI-RS-ResourceMapping | Include parameters to capture OFDM symbol location(s) in a slot and subcarrier occupancy in a PRB of the CSI-RS resource | Adopt the parameter values agreed in BM CSI-RS |
| RLM-CSI-RS-timeConfig | Contains periodicity and slot offset for periodic/semi-persistent CSI-RS | Periodicity, P: {5ms, 10ms, 20ms, 40ms} Slot offset: {0, …, Ps-1} slots Where Ps is number of slots within period P in the CSI-RS numerology Common value across all configured RLM-CSI-RS |

# References

- R1-1721369, “Summary of Discussion for NR Radio Link Monitoring,”	Intel Corp.