3GPP TSG-RAN WG1 #91	R1-1721383

Reno, USA, November 27th – December 1st, 2017

Source:	Ericsson

Title:	Sequence initialization for DMRS and CSI-RS

Agenda Item:	7.2.3.8

Document for:	Discussion and Decision

# Introduction

In RAN1 NR Ad Hoc #3, the following agreements were made:

Agreements (NR AH2):

- Support for CSI-RS sequence design:

- (Working assumption) PN sequence at least for CSI-RS for CSI acquisition

- UE specifically configured CSI-RS sequence seed

- FFS, sequence design, e.g. PN sequence, ZC sequence, M sequence, etc., for other use cases such as beam management and fine time/frequency tracking

Agreements:

- Confirm the WA with the following revision

  - Support PN sequence for CSI-RS for CSI acquisition and beam management

- FFS detailed sequence generation and initialization methods

  - E.g. function of slot number, OFDM symbol number, CP length, UE ID, cell ID, virtual cell ID, function of PRB position of configured CSI-RS resource, etc.

Agreements (#90):

- DMRS sequence for CP-OFDM based PDSCH and PUSCH is resource-specific, and is a function of at least the following parameters: scrambling ID and offset

  - FFS how to define the offset parameter (e.g., via signalling, implicitly derived, etc.)

  - FFS the details of resource-specific (e.g., w.r.t to the wideband of a carrier, or a BWP, etc.)

  - FFS the values of scrambling ID

  - FFS other parameters

Agreement (#90bis):

For each of the following RSs, c_init used for sequence generation will at least depend on UE specifically configured scrambling ID by RRC configuration

- DL: DMRS, CSI-RS resource

  - FFS: Support of multiple IDs for DMRS

- UL: DMRS(for CP-OFDM), PTRS(for DFT-s-OFDM)

Agreement (#90bis):

For the PN sequence for CP-OFDM, QPSK modulated DMRS is derived from the

    - Alt. 1: length-31 Gold sequence (as in LTE)

    - Alt. 2: length-63 Gold sequence

In this contribution, we propose a detailed formulation of the sequence generation for CSI-RS and DMRS for CP-OFDM including a formula for computation of the seed. We also evaluate the proposal numerically. Specific formulas for DMRS and CSI-RS are also proposed in [1], and [2], respectively.

# Discussion

## Issues when reusing initialization of LTE PRBS generator for NR RSs

The LTE pseudo-random binary sequence generator use 31-bit initialization of the linear feedback shift registers in which the initialization formulas for CRS and CSI-RS can be expressed for normal CP as a product of the symbol counter t=7ns+l+1 and scrambling ID for most significant bits (MSBs) and the scrambling ID for least significant bits LSBs:

cinit=210t+72nID+118 bit (MSBs)+2nID+110 bit (LSBs),       t=1,2,…,140,  nID∈0,1,…,503.

As has been noticed previously, reusing this formula in conjunction with a length-31 PRBS will not be possible in NR as the range of the radio frame OFDM symbol counter t can be considerably larger.

It can be noticed that keeping the same wrap around duration of t in absolute time for NR with scaled numerology as was used in LTE (i.e. 10ms), will require a range of t of 1,2,…,140∙2n where n=0 corresponds to a 10-slot radio frame (15 kHz SCS) and n=5 to a 320-slot radio frame (480 kHz SCS). The corresponding bit range of t will then be 8,9,…,13. Hence, at least 13 bits is needed for the term t+7 if the same wrap around time is desired for NR as in LTE, which implies that the CRS/CSI-RS initialization approach of the LTE PRBS generator cannot be reused.

Another aspect that needs to be considered is that NR supports twice the number of physical cell IDs as LTE so the term 2nID+1 requires at least 11 bits. This implies that log2t+7 needs to be limited to 9 bits if cinit is to fit into the length-31 shift register.

Length-63 Gold sequences for PRBS based RS should only be considered if it is not possible to find any 31-bit initialization formula cinit=ft,nID,… that provides sequences with correlation properties matching LTE. We note that introducing length-63 Gold sequences would require a larger specification effort as both the polynomials defining the m-sequences and a new sequence initialization mechanism need to be specified.

However, based on the evaluations conducted in Section 2.5, we see no issues with keeping the length-31 Gold sequence generation defined for LTE. This is true also while increasing the bit width of the scrambling ID to 15 bits and keeping the 10ms wrap around duration of t as in LTE. We thus propose the following

Reuse the same length-31 Gold sequence generator as defined for LTE.

- The proposed 31-bit initialization formula to be used for NR is provided in Section 2.4.

## On common RS structure for PRBS initialization

Depending on configuration, reference signals of different types may be mapped to the same REs when transmitted from different transmission points, e.g., DMRS in one cell may collide with CSI-RS in another cell. In a synchronized network, this means that interference inter and intra RS type may not be randomized unless care is taken to ensure that reference signal sequence initialization differs between all transmission points that may interfere with each other for all RS that may occupy the same REs. Note that this may be a problem even if the overlap in terms of mapped REs is only partial (e.g. TRS may only use every 4th subcarrier but DMRS every 2nd) - this is because sequences are resource specific. To ensure that the inter RS type interference can be randomized, it is important to have a common structure of the initialization formulas; that will make sequence generation transparent and then the network can configure scrambling IDs to avoid malign interference. To that end we propose:

The PRBS generator use the same initialization formula structure at least for CSI-RS and DMRS

The above proposal allows creating pseudo orthogonal sequences across RS types through configuration by giving them different scrambling IDs. Note that even if the same structure is used, this will still be a complex coordination problem, as coordination of scrambling IDs needs to be done for all combinations of reference signals, not only per reference signal type. In order to avoid this configuration/coordination effort and also avoid reducing the set of available scrambling IDs (as the pool would be shared across CSI-RS and DMRS) we also propose:

Define cinit such that reference signals of different type always have different sequence initialization independent of the UE specifically configured ID

## On bit width of the UE specifically configured scrambling ID

From a MU-MIMO perspective, it is beneficial to support multiple scrambling IDs per cell for DMRS - this allows for intra cell semi-orthogonal DMRS assignment between users with high spatial separation. Accordingly, as the number of cell identities in NR is 1008, the width of the scrambling ID should then be greater than 10 bits. A larger bit width could simplify network planning as cells may be assigned a unique range of scrambling IDs from a larger set.

As an example, consider NR deployed on an existing LTE grid (that will have cell-IDs selected to optimize reuse), then it is desirable to assign a range of scrambling IDs to each NR site/sector, and the ranges should be such that if two LTE cells have different cell-IDs, then the range of NR scrambling IDs mapped to the corresponding sites/sectors should be non-overlapping.

Hence, we think that more than 10 bits in the UE specifically configured scrambling ID is beneficial for DMRS, and that this should also apply for CSI-RS (to enable a common structure of the sequence initialization between CSI-RS and DMRS as motivated further below). We also note that a bit width of 15 bits allows for many scrambling IDs per cell, assuming 210 unique physical cell identifiers: This is likely enough for the envisioned use cases.

The UE specifically configured scrambling ID for CSI-RS and DMRS use 15 bits

## The PRBS initialization formula

One objective for the design of the sequence initialization should be to ensure that the randomization properties of the interference from a reference signal are acceptable irrespective of which different scrambling ID that is used by the interfering reference signal. This means that the overall statistical properties of the cross-correlations should be independent of the scrambling ID when observed over a period of time. If this property is fulfilled, then planning of scrambling ID assignments need not to take cross-correlation properties of individual scrambling ID pairs into account.

Design the RS sequence initialization such that the statistical properties of the cross- correlations between any pair of distinct scrambling IDs are similar as measured across a sufficiently long-time period.

To randomize inter-cell interference across OFDM symbols carrying reference signals, a dependency between the symbol index and the RS sequence initialization can be introduced. This dependency will mitigate biased measurements due to systematic properties of the interference that may, e.g., lead to consistently poor RS cross-correlations. Note that for data scrambling, we suggest to not make the bit scrambling time dependent as there is serious consequences when using very late go/no go scheduling decisions as a large data packet must be prepared in advance to deciding on when to perform the transmission. This is especially important for unlicensed operation. For RS, this is not as critical as the transmitter complexity to generate a RS sequence is far less demanding than preparing a large transport block.

Define cinit for CSI-RS and DMRS such that it depends on a time counter

The Gold sequence generation mechanism in LTE is based on the modulo-2 additions of two m-sequences

ct,nID=x1+x2t,nID

where the m-sequence x1 is initialized with a fixed seed (independent of configuration) and where the initialization of the m-sequence x2 is a function of the symbol counter and the scrambling ID. In case the symbol counter t is linearly (in modulo 2 sense) combined with the ID, such as cinit=2log2max⁡(nID)t+7+nID, the second m-sequence can be expressed as

x2t,nID=x2(1)t+x2(2)nID

which follows by noting that m-sequences are linear functions of the initializer (in modulo-2 sense). We observe that if the symbol counter and the scrambling ID are combined linearly (in modulo 2 sense), we may then expect that cross-correlations between Gold sequences generated from two different scrambling IDs are consistent across time as

ct,nID1=x1+x2(1)t+x2(2)nID1

ct,nID2=x1+x2(1)t+x2(2)nID2

This is also confirmed using numerical evaluations in Section 2.5, with the  alternative for scrambling seed representing linearly combined symbol counter and ID. Evidently, this is an undesirable property.

Define cinit such that the scrambling ID combines non-linearly (in modulo 2 sense) with the time counter

Based on the aspects highlighted in this and the preceding sections, we propose the following structure of the PRBS initialization formula

Adopt cinit=2B∙t∙Zμ+nID+nRStype∙M+nID , where t has a sufficiently long wrap-around period, as the PRBS initialization for DM-RS and CSI-RS

where the addition is in standard sense, is the UE specifically configured scrambling ID with bit width B (proposed above to be 15 bits), is a constant specific for DMRS and CSI-RS, Zμ is a constant integer selected to make bit width of t∙Zμ match that of  (it needs to be numerology dependent and should also be prime), M is selected so that no bit in  can be fixed to zero, and t is a symbol counter with a wrap-around time of 10 ms.

The proposed mechanism is evaluated in Section 2.5 below, both for a bit width of 10 (to allow comparison to LTE) and with the proposed bit width of 15.

For B=15 bit wide UE specific scrambling ID we propose

In the case of t=14ns+l, we propose  Zμ = {233, 113, 53, 29, 13, 7} for subcarrier spacings {15 kHz, 30 kHz, 60 kHz, 120 kHz, 240 kHz, 480 kHz}, respectively

## On the range of the symbol counter and the number of available sequences

For the LTE CRS, the sequence initialization is a function of the slot counter, ns, and the symbol index within the slot. With four symbols used for CRS per slot (per port) this given 40 unique sequences for a given cell-ID. With NR DMRS in single symbol front loaded configuration and 15 kHz numerology, the same principle would result in only 10 unique sequences for a given scrambling ID. As seen in Section 2.6.4, this may lead to poor randomization of the RS sequences; some pairs of scrambling IDs will have high correlation for all time instances.

Basing the time part of sequence initialization on the slot and symbol index only may result in poor randomization properties for numerologies with small subcarrier spacing.

One approach to reworking the LTE sequence initialization mechanism to make it support the larger range of the symbol counter due to shorter symbols, but not larger bit-width of nID is to explicitly introduce a wrap-around to match the range of the symbol counter in of LTE, e.g.,

cinit=211modt,140+72nID+119 bit MSBs+2nID+111 bit LSBs,  nID∈0,1,…,1007

We note that this formulation can work with the Gold-31 sequence design, also with subcarrier spacings lager than 15kHz. However, consider the following example (with DMRS mapped to 3rd symbol and 60kHz subcarrier spacing):

t=14ns+l, ns=0…139×22, l=2

In this example, the time dependent factor of the first term will only take the values

modt,140= 2, 16, 30,44,58,72,86,100,114,128.

This means than for a given nIDwith single front loaded DMRS, only 10 sequences are used (irrespective of the numerology). This is clearly not good from an interference randomization perspective. Furthermore, an interfering cell with the same or partly overlapping DMRS configuration will have the same cycle, so the mutual correlation will be poor, at least in a statistical sense.

Another approach would be to introduce a wrap-around on the entire symbol index dependent term:

cinit=211mod(t+72nID+1,220)19 bit MSBs+2nID+111 bit LSBs,

where the wrap around range is selected to get the desired bit-width of the first term. Here we note that the wrap-around range will be dependent on the configured nID, i.e. the amount of randomization will depend on the scrambling ID which evidently is an undesirable property of an initialization design. Furthermore, there will be cases where different scrambling IDs, for same time instant t, results in the same value of the first term. This is evidently an undesirable property as discussed in the previous section.

The effect of the number of unique sequences per scrambling ID is demonstrated in Section 2.6.4.

Reducing the range of the time counter may result in fewer unique sequences per scrambling ID and hence poor randomization of the interference.

Consider including a dependency on the frame counter in the sequence initialization to increase the number of unique sequences used for any given scrambling ID.

## Simulation results

### Evaluation methodology

Sequences where generated using different seed generation formulas, including the one defined for LTE. Pairs of , (,) where drawn based from a uniform distribution covering the full range (0 to 1023 when evaluating 10-bit scrambling IDs, and 0-32767 when evaluating 15-bit scrambling ID). The correlation between the sequence (of length 48 in the presented examples, other lengths show the same behaviour) generated byand the sequence generated by was computed for the full range of the symbol counter t (1…140, when comparing with LTE, 1 … 4480 when not). The resulting correlation values where then plotted either as a CDF (across all time instances) per randomly picked pair of  , or as a histogram of the 95th percentile of the correlation (across all random pairs).

### Results comparing LTE + simple + proposed scheme for 10-bit scrambling ID

To allow comparison to the seed definition formula used in for LTE, we restrict the RS specific scrambling ID to the range 0…1023 (B = 10) (although we propose to use 15-bits with a correspondingly larger range), and the range of to 1…140 in this section. Three alternative formulations of the scrambling seed where tested:

- cinitproposed=2B∙t∙Z+nID+nRStype∙M+nID , where Z=7, M=997 and nRStype=1.

- cinitLTE=2B∙t+7∙2nID+1+2nID+1  (as in LTE CRS/CSI-RS and normal CP)

- cinitsimple=2B∙t+nID

Figure 1, shows CDFs across time for 15 randomly picked RS IDs using the three alternative scrambling seed formulations. Figure 2 shows a histogram of the 95th percentile correlation over 1000 such pairs. Alternatives and  show only marginal dependence on the scrambling ID pair selected in the sense that correlation is similar for all pairs in a statistical sense if evaluated over the full symbol counter range. The initialization, on the other hand, does not show this property: if a scrambling ID pair has a poor correlation at one-time instance it is likely to have poor correlation for all time instances (and the same is true for favourable cross correlation). This effect is readily explained by the linearity property of the m-sequences discussed in Section 2.4. From these results it may be concluded that the proposed scrambling seed formulation performs on par with LTE for 10-bit scrambling ID while it has the advantage of readily extending to 15-bits scrambling ID and to larger range of the symbol counter as evaluated in the next section.

Figure 1	CDF of correlation (across the symbol counter) for randomly picked scrambling ID pairs. 10-bit width of RS scrambling ID.

Figure 2	Histogram of 95th percentile of correlation across time between sequences generated using the alternative seed generation formulas. 10-bit width of RS scrambling ID.

The proposed scrambling seed formulation works as well as the LTE version in terms of cross correlation properties across time and may readily be extended to 15 bits.

### Results comparing simple + proposed scheme for 15-bit scrambling ID

In this section, corresponding evaluations are made using the proposed 15-bit width of the scrambling ID, i.e. B=15. Here the range of t is extended to 1…4480.  The LTE formula does not scale to 15-bits and is excluded from the comparison. Instead the following two formulations are used:

- cinitproposed=2B∙t∙Z+nID+nRStype∙M+nID , where Z=7, M=997 and nRStype=1.

- cinitsimple=2B∙t+nID

Figure 3  shows the CDF across individual pairs of IDs for the two alternative scrambling seed formulations and Figure 4 show histogram of the 95the percentile (similar to the previous section). The conclusions from the previous section carry-over also to the 15-bit width case, and the proposed scheme show that the correlation properties of random pairs of IDs are relatively constant if measured over time. This indicates that the proposed scheme allow extension to 15-bit scrambling ID, and that Gold-31 sequence generation works well for NR.

Figure 3	CDF of correlation (across the symbol counter) for randomly picked scrambling ID pairs. 15 bit width of RS scrambling ID.

Figure 4	Histogram of 95th percentile of correlation across time between sequence pairs generated using the alternative seed generation formulas. 15-bit width of RS scrambling ID. 1000 pairs where evaluated.

### Results comparing the proposed solution to a solution based on using the symbol counter with a modulus operation to reduce effective range

The following three sequence initialization mechanisms for 10 bit nID where evaluated at one constant OFDM symbol in each slot (corresponding the case experienced for example with a single front loaded DMRS):

- cinitproposed=2B∙t∙Z+nID+nRStype∙M+nID , where Z=7, M=997 and nRStype=1.

- cinitmodLTE=2B∙mod(t,140)+7∙2nID+1+2nID+1  (as discussed in Section 2.5)

- cinitsimple=2B∙t+nID

The range of t evaluated was 1…4480 (corresponding to 10 ms with 480kHz subcarrier spacing). Otherwise the evaluation methodology was the same as in the sections above. The resulting CDF (of a few random pairs of nID) and a histogram of the 95th percentile over a much larger set of random pairs are shown in Figure 5. The fact that the cinitmodLTE results in fewer unique sequences per nID is clearly resulting in more spread of the correlation properties. This is not desirable.

Figure 5	Top: CDF of correlation (across the symbol counter) for 60 randomly picked scrambling ID pairs. 10-bit width of RS scrambling ID. Bottom: Histogram of 95th percentile of correlation across time between sequences generated using the alternative seed generation formulas.

# Conclusions

We make the following proposals:

Proposal 1	Reuse the same length-31 Gold sequence generator as defined for LTE.

Proposal 2	The PRBS generator use the same initialization formula structure at least for CSI-RS and DMRS

Proposal 3	Define cinit such that reference signals of different type always have different sequence initialization independent of the UE specifically configured ID

Proposal 4	The UE specifically configured scrambling ID for CSI-RS and DMRS use 15 bits

Proposal 5	Design the RS sequence initialization such that the statistical properties of the cross- correlations between any pair of distinct scrambling IDs are similar, or constant, as measured across a sufficiently long-time period.

Proposal 6	Define cinit for CSI-RS and DMRS such that it depends on a time counter

Proposal 7	Define cinit such that the scrambling ID combines non-linearly (in modulo 2 sense) with the time counter

Proposal 8	Adopt cinit=2B∙t∙Zμ+nID+nRStype∙M+nID, where t has a sufficiently long wrap-around period, as the PRBS initialization for DM-RS and CSI-RS

Proposal 9	In the case of t=14∙ns+l, we propose  Zμ= {233, 113, 53, 29, 13, 7} for subcarrier spacings {15 kHz, 30 kHz, 60 kHz, 120 kHz, 240 kHz, 480 kHz }, respectively

Proposal 10	Consider including a dependency on the frame counter in the sequence initialization to increase the number of unique sequences used for any given scrambling ID.

# References

R1-1720736, “Remaining details on DMRS design”, 3GPP TSG-RAN WG1 #91, Ericsson, November 2017

R1-1720735, “Remaining details on CSI-RS design”, 3GPP TSG-RAN WG1 #91, Ericsson, November 2017