3GPP TSG RAN WG1 Meeting 91		R1-1721486

Reno, USA, November 27th – December 1st, 2017

Source:	CATT

Title:	On TBS quantization

Agenda Item:	7.4.1.2

Document for:	Discussion and Decision

Introduction

In RAN1#90bis meeting, the following agreements were reached on TBS determination.

TBS determination

Agreements:

- For every TB-level (re-)transmission, the UE is able to determine the TB size from the DCI information in that transmission only

Agreements:

- The TBS is determined based on the actual # of available REs compared with a plurality of reference # of REs

  - FFS the details, including the # of reference REs and other factors for TBS determination

Agreements:

- Calculate an “intermediate” number of information bits NRE∙υ∙Qm∙R where

  - υ is the number of layers,

  - Qm is the modulation order, obtained from the MCS index

  - R is the code rate, obtained from the MCS index

  - NRE is number of resource elements

  - NRE= Y * #PRBs_scheduled

- When determining NRE (number of REs) within a slot

  - Determine X =  12* #OFDM_symbols_scheduled – Xd – Xoh

    - Xd = #REs_for_DMRS_per_PRB in the scheduled duration

    - Xoh = accounts for overhead from CSI-RS, CORESET, etc. One value for UL, one for DL.

      - Xoh is semi-statically determined

  - Quantize X into one of a predefined set of values, resulting in Y

    - [8] values

      - Should allow for reasonable accuracy for all transmission durations

      - May depend on the number of scheduled symbols

    - FFS: floor, ceiling or some other quantization

    - Note: quantization may not be needed

    - FFS: Quantization step should ensure the same TB size can be obtained between transmission and retransmission, irrespective of the number of layers used for the retransmission. otherwise Xd has to be independent of the number of layers

- Obtain the actual TB size from the intermediate number of information bits according to the channel coding decisions

This contribution discusses TBS quantization from intermediate TB size to actual TB size.

Discussion

In [1], a TBS quantization method is proposed. The idea is to quantize the TBS such that the CB size including CB-CRC (if any) is one of the values in a predefined set where the values in the set are the information block sizes assumed in LDPC evaluation campaigns. One of the advantages of this quantization method is that all the code block sizes have been fully evaluated so that the channel coding performance can be guaranteed.

Let TBSinter be the “intermediate” number of information bitsNRE∙υ∙Qm∙R, the quantization steps should take channel coding agreements into account as illustrated in Figure 4. Different BGs lead to different maximum code block sizes.

Figure 4: Illustration of BG1/BG2 and number of CBs

Define Z=3840, if R≤1/48448, else

If TBSinter≤Z, C = 1

TBSquantized=minimum K such that K≥TBSinter, where K∈K1, K2,…,KN

Else

C=TBSinterZ-LCB-CRC.

TBSquantized=C×K-LCB-CRC,

Where K= minimum K such that TBSinter≤C×K-LCB-CRC and K∈K1, K2,…,KN,.

TBS=TBSquantized-LTB-CRC

The set of K is as listed below.

|  | 40<=K<=512 | 528<=K<=1024 | 1056<=K<=2048 | 2112<=K<=6144 | 6272<=K<=8448 |
| --- | --- | --- | --- | --- | --- |
| Step size | 8 | 16 | 32 | 64 | 128 |

We compare the above quantization method (denoted as ‘non-linear byte-aligned CBS’) with the other two methods in terms of scheduling flexibility, accuracy and overhead as follows. LTE MCS table as shown in Annex is assumed. For LTE TBS, Table 7.1.7.2.1-1 in 36.213 is assumed. For NR, the range of number of PRBs is 1 to 275.

The other two quantization methods are:

- TBSinter8*C*8*C-LTB-CRC (denoted as ‘step size of 8*C’ in the following)

- TBSinterlcm(8,C)*lcm(8,C)-LTB-CRC (denoted as ‘step size of lcm(8,C)’ in the following)

- Scheduling flexibility

In order to evaluate the scheduling flexibility, two metrics are considered [2][3] including:

- Number of combination of MCS and #PRBs vs. TB size

- Percentage of TB size with single combination of MCS and #PRBs

The results are shown in Figure 5 to Figure 8 with assumptions of Y=120 and Y=60 respectively.

Figure 5: Number of PRB/MCS combinations vs. TBS (Y=120)

Figure 6: Percentage of TB size with single combination of MCS and #PRBs (Y=120)

Figure 7: Number of PRB/MCS combinations vs. TBS (Y=60)

Figure 8: Percentage of TB size with single combination of MCS and #PRBs (Y=60)

- Quantization accuracy

In order to evaluate the quantization accuracy, we show the CDF of coding rate difference between effective coding rate and target coding rate where effective coding rate equals TBS+LTB-CRCNRE∙υ∙Qm∙NRB-R and R is the target coding rate derived from MCS table.

The results are shown in Figure 9 and Figure 10 assuming Y=120 and Y=60 respectively.

- Overhead

In order to evaluate the overhead, we show the overhead ratio of different quantization methods below where quantization ratio equals TBSk-(TBSk-1+8)TBS(k) [4][5].

Observation: Quantization method with non-linear byte-aligned CBS achieves much better scheduling flexibility than quantization step size of 8*C and lcm(8,C) with good quantization accuracy and low padding ratio.

Based on the above analysis, it is proposed to adopt the quantization method with non-linear byte-aligned CBS.

Conclusion

This contribution discussed TBS quantization from intermediate TBS to final TBS with the following proposal.

Proposal 1: Denote “intermediate” number of information bits as TBSinter, it is quantized as:

Define Z=3840, if R≤1/48448, else

If TBSinter≤Z, C = 1

TBSquantized=minimum K such that K≥TBSinter, where K∈K1, K2,…,KN

Else

C=TBSinterZ-LCB-CRC.

TBSquantized=C×K-LCB-CRC,

where K= minimum K such that TBSinter≤C×K-LCB-CRC and K∈K1, K2,…,KN

TBS=TBSquantized-LTB-CRC

The set of K is as listed below.

|  | 40<=K<=512 | 528<=K<=1024 | 1056<=K<=2048 | 2112<=K<=6144 | 6272<=K<=8448 |
| --- | --- | --- | --- | --- | --- |
| Step size | 8 | 16 | 32 | 64 | 128 |

Reference

- R1-1715824, PDSCH and PUSCH resource allocation, CATT, RAN1 NR AH#3

- R1-1719525, Remaining details of LDPC coding, ZTE, Sanechips, RAN1#91

- R1-1720479, On resource allocation for PDSCH and PUSCH in NR, Nokia, Nokia Shanghai Bell, RAN1#91

- R1-1719579, On TBS determination formula, MediaTek, RAN1#91

- R1-1719596, On TBS Determination and DL/UL Resource Allocation, Ericsson, RAN1#91

- Annex

Table 5: MCS Table

| MCS Index | modulation | coding rate x 1024 |
| --- | --- | --- |
| 0 | 2 | 120 |
| 1 | 2 | 157 |
| 2 | 2 | 193 |
| 3 | 2 | 251 |
| 4 | 2 | 308 |
| 5 | 2 | 379 |
| 6 | 2 | 449 |
| 7 | 2 | 526 |
| 8 | 2 | 602 |
| 9 | 2 | 679 |
| 10 | 4 | 340 |
| 11 | 4 | 378 |
| 12 | 4 | 434 |
| 13 | 4 | 490 |
| 14 | 4 | 553 |
| 15 | 4 | 616 |
| 16 | 4 | 658 |
| 17 | 6 | 438 |
| 18 | 6 | 466 |
| 19 | 6 | 517 |
| 20 | 6 | 567 |
| 21 | 6 | 616 |
| 22 | 6 | 666 |
| 23 | 6 | 719 |
| 24 | 6 | 772 |
| 25 | 6 | 822 |
| 26 | 6 | 873 |
| 27 | 6 | 910 |
| 28 | 6 | 948 |