3GPP TSG RAN WG1 Meeting #91								    R1-1721603

Reno, USA, 27th November - 1st December, 2017

Source:            ZTE, Sanechips

Title:		Remaining details of LDPC coding

Agenda item:   7.4.1.2

Document for: Discussion/Decision

# Introduction

At the 3GPP TSG RAN1 NR #90bis meeting, the following agreement about TBS determination have been achieved [1]:

Agreements:

- Calculate an “intermediate” number of information bits where

  - is the number of layer,

  - is the modulation order, obtained from the MCS index

  - is the code rate, obtained from the MCS index

  - is number of resource elements

  - =Y*#PRBs_scheduled

- When determining (number of REs) within a slot

- Determine X = 12*#OFDM_symbols_scheduled—Xd—Xoh

    - Xd = #REs_for_DMRS_per_PRB in the scheduled duration

    - Xoh = accounts for overhead from CSI-RS, CORESET, etc. One value for UL, one for DL

      - Xoh is semi-statically determine

- Quantize X into one of a predefined set of values, resulting in Y

    - [8] values

- Should allow for reasonable accuracy for all transmission durations

- May depend on the number of scheduled symbols

- FFS: floor, ceiling or some other quantization

- Note: quantization may not be needed

- FFS: Quantization step should ensure the same TB size can be obtained between transmission and retransmission, irrespective of the number of layers used for the retransmission. otherwise Xd has to be independent of the number of layers

- Obtain the actual TB size from the intermediate number of information bits according to the channel coding decisions.

- For both slot and mini-slot, the scheduling DCI can provide an index into a UE-specific table giving the OFDM symbols used for the PDSCH (or PUSCH) transmission

- starting OFDM symbol and length in OFDM symbols of the allocation

- For every TB-level (re-)transmission, the UE is able to determine the TB size from the DCI information in that transmission only

- The TBS is determined based on the actual # of available REs compared with a plurality of reference # of REs

  - FFS the details, including the # of reference REs and other factors for TBS determination

Agreement:

- TBSs are byte-aligned

Agreement:

The first Working Assumption from RAN1#90 AI 6.1.4.1.2 and the first Working Assumption from NR AH#3 AI 6.4.1.3 are combined and agreed as modified below:

- For initial transmissions with code rate Rinit> 1/4, BG2 is not used when TBS>3824

  - If the FFS on UE capabilities w.r.t. support of both BGs is resolved such that it is possible that a UE does not support BG1, then the above bullet only applies if the UE supports BG1.

- BG2 is used for initial transmissions with code rate Rinit<= ¼ for all TBS supported at that code rate

  - For BG2 with TBSs larger than 3824, the TB is segmented into CBs no larger than 3840

- TBS determination for all code rates shall ensure that no zero padding is necessary with BG1 segmentation; TBS determination shall also strive to achieve no zero padding also with BG2 segmentation; any special cases are only permitted for BG2.

  - If needed for BG2 segmentation, zero padding is added during segmentation, with the padding being placed at the beginning of the first code block prior to CB-CRC calculation; padding bits are transmitted.

Agreement:

The first Working Assumption from RAN1#90 AI 6.1.4.1.2 and the first Working Assumption from NR AH#3 AI 6.4.1.3 are combined and agreed as modified below:

- TBS determination for all code rates shall ensure that no zero padding is necessary with BG1 segmentation; TBS determination shall also strive to achieve no zero padding also with BG2 segmentation; any special cases are only permitted for BG2.

  - If needed for BG2 segmentation, zero padding is added during segmentation, with the padding being placed at the beginning of the first code block prior to CB-CRC calculation; padding bits are transmitted.

FFS: Byte- or something-alignment of CB sizes.

At the 3GPP TSG RAN1 #90bis meeting, the following agreement and conclusion about RV sequences have been achieved [1]:

Agreement:

- The default RV order (sequence) is {0,2,3,1} for cases where RV index is not explicitly signalled or otherwise specified and there is no ambiguity about which instance of a transmission occurred, for both BG1 and BG2

Conclusion for other cases, e.g. grant-free and unlicensed:

- The respective session should determine the requirements (ambiguity, number of repetitions, self-decodability, existence of configuration signalling) and RV(s) should then be determined accordingly in the channel coding session.

Furthermore, a working assumption on RV sequences for UL grant free from email discussion has been reached as follow:

Working assumption:

•        For UL transmission without UL grant, for a TB transmission with K repetitions

–     The repetitions follow an RV sequence and it is configured by UE-specific RRC signalling to be one of the following:

•     Sequence 1: {0, 2, 3, 1}

•     Sequence 2: {0, 3, 0, 3}

•     Sequence 3: {0, 0, 0, 0}

In this contribution, the actual TB size determination from the intermediate number of information bits for NR is discussed. And, the RV sequences in working assumption for UL grant free are also discussed.

# Consideration on TBS determination

2.1 Discussion

In the previous meeting, it is agreed that BG determination rule depends on code rate and TBS as shown in figure 1 and intermediate TBS is introduced in the first time. As we all know, BG determination rule for initial transmission have been discussed in the previous three meetings, and during these meeting there isn’t intermediate number of information bits concept and TBS only means actual TBS. That is to say, the previous agreement that BG determination depends on actual TBS and code rate means that BG determination should be decided after TBS determination.

Figure 1: Illustration of BG1/BG2 region

Observation 1: It is agreed in the previous meeting that BG determination rule for initial transmission depends on code rate and actual TBS.

Proposal 1: Actual TBS rather than intermediate number of information bits should be used for BG determination.

According to the rule of TBS determination being independent on BG determination above, there are three ways for TBS determination:

In [2], A BG determination method based on the number of intermediate information bits is proposed to avoid circular relations between TBS determination and base graph selection. Obviously, there is discrepancy with the agreement we achieved in the previous meetings. In fact, either of the following 2 principles can be considered to get actual TBS with no circular relations problem as well.

Principle 1: Actual TBS determination only considers the requirements of BG1, like equal CBS, no zero padding, CBS byte-alignment etc.

Principle 2: TBS determination only considers the requirements of both BG1 and BG2, like equal CBS, no zero padding, CBS byte-alignment etc.

In the next section, we propose an actual TBS determination method  based on the principle 2.

2.2 Actual TBS determination

In LTE, the transport block size is inferred from the MCS index and the resource allocation indicated in the DCI. This is done via look-up tables specified in TS 36.213.These tables are applied to both downlink and uplink. In NR, an intermediate TB sizes can be calculated by the product of modulation order, code rate, the number of layers and the total number of REs. The total number of REs is equal to Y*#PRBs_scheduled, where Y is the reference value of REs per PRB. However, there are still some channel coding constraints listed below need to be considered to obtain the actual TB size from the intermediate TB size:

- Actual TBSs are byte-aligned;

(2) TBS determination for all code rates shall ensure that no zero padding is necessary with BG1 segmentation;

(3) TBS determination for all code rates should strive to achieve no zero padding also with BG2 segmentation;

(4) The same TB size can be obtained between transmission and retransmission;

(5) equal CBS for BG1&BG2;

(6) equal and byte-aligned CBS for BG1&BG2.

To fulfill the 6 channel coding constraints list above, we need to quantize the intermediate number of information bits to get actual TBS。In this section, we propose that actual TBS can be acquired by mapping the intermediate number of information bits to the entries in a provisioned TBS sequence. The mapping rule is described as follows:

Step1: calculate an intermediate information bits TBS_temp by the scheduled resources parameters:

TBS_temp=, where TBS_temp with TB CRC bits.

Step2: quantize this intermediate information bits to obtain the actual TBS:

find the minimum value of actual TBS in TBS sequence, such that .

·Qm is the scheduled modulation order,

·R is the code rate,

·υ is the number of layers the codeword is mapped onto,

·NRE is the total number of resource elements,

·LTB,CRC is the size of the TB CRC, if  TBS ≤ 3824, LTB,CRC = 16; else LTB,CRC = 24,

Table 1 gives a TBS sequence, which is updated based on the sequence 2 in [6] without TBS size of 328.

Table 1 TBS sequence (Updated TBS sequence 2 in [6] by removing TBS of 328)

| index | TBS | index | TBS | index | TBS | index | TBS | index | TBS | index | TBS | index | TBS | index | TBS | index | TBS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 8 | 31 | 304 | 61 | 1160 | 91 | 2792 | 121 | 8232 | 151 | 24336 | 181 | 73416 | 211 | 181608 | 241 | 501576 |
| 2 | 16 | 32 | 320 | 62 | 1192 | 92 | 2856 | 122 | 8472 | 152 | 25288 | 182 | 74856 | 212 | 194664 | 242 | 543376 |
| 3 | 24 | 33 | 336 | 63 | 1224 | 93 | 2976 | 123 | 8760 | 153 | 26184 | 183 | 76136 | 213 | 197160 | 243 | 567368 |
| 4 | 32 | 34 | 352 | 64 | 1256 | 94 | 3104 | 124 | 9096 | 154 | 27240 | 184 | 77256 | 214 | 205176 | 244 | 571176 |
| 5 | 40 | 35 | 368 | 65 | 1288 | 95 | 3240 | 125 | 9480 | 155 | 28328 | 185 | 78936 | 215 | 206776 | 245 | 585176 |
| 6 | 48 | 36 | 384 | 66 | 1320 | 96 | 3368 | 126 | 9864 | 156 | 29288 | 186 | 81816 | 216 | 208976 | 246 | 626976 |
| 7 | 56 | 37 | 408 | 67 | 1352 | 97 | 3496 | 127 | 10248 | 157 | 30504 | 187 | 84984 | 217 | 228456 | 247 | 638376 |
| 8 | 64 | 38 | 432 | 68 | 1384 | 98 | 3624 | 128 | 10680 | 158 | 31656 | 188 | 87008 | 218 | 232264 | 248 | 668776 |
| 9 | 72 | 39 | 456 | 69 | 1416 | 99 | 3752 | 129 | 11064 | 159 | 32808 | 189 | 88680 | 219 | 246456 | 249 | 710576 |
| 10 | 80 | 40 | 480 | 70 | 1480 | 100 | 3880 | 130 | 11432 | 160 | 33816 | 190 | 92376 | 220 | 248136 | 250 | 752376 |
| 11 | 88 | 41 | 504 | 71 | 1544 | 101 | 4008 | 131 | 11816 | 161 | 35096 | 191 | 96072 | 221 | 250776 | 251 | 794176 |
| 12 | 96 | 42 | 528 | 72 | 1608 | 102 | 4136 | 132 | 12200 | 162 | 36536 | 192 | 99336 | 222 | 272424 | 252 | 797976 |
| 13 | 104 | 43 | 552 | 73 | 1672 | 103 | 4264 | 133 | 12552 | 163 | 37816 | 193 | 101064 | 223 | 281768 | 253 | 835976 |
| 14 | 112 | 44 | 576 | 74 | 1736 | 104 | 4392 | 134 | 12936 | 164 | 39136 | 194 | 104808 | 224 | 285576 | 254 | 856776 |
| 15 | 120 | 45 | 608 | 75 | 1800 | 105 | 4568 | 135 | 13480 | 165 | 40456 | 195 | 108552 | 225 | 292576 | 255 | 877776 |
| 16 | 128 | 46 | 640 | 76 | 1864 | 106 | 4760 | 136 | 14024 | 166 | 42120 | 196 | 112536 | 226 | 295752 | 256 | 885288 |
| 17 | 136 | 47 | 672 | 77 | 1928 | 107 | 4952 | 137 | 14600 | 167 | 43752 | 197 | 114552 | 227 | 319176 | 257 | 892920 |
| 18 | 144 | 48 | 704 | 78 | 1992 | 108 | 5160 | 138 | 15208 | 168 | 45288 | 198 | 119016 | 228 | 334376 | 258 | 997672 |
| 19 | 152 | 49 | 736 | 79 | 2024 | 109 | 5352 | 139 | 15816 | 169 | 46776 | 199 | 122736 | 229 | 355672 | 259 | 1001480 |
| 20 | 160 | 50 | 768 | 80 | 2088 | 110 | 5544 | 140 | 16376 | 170 | 48648 | 200 | 125376 | 230 | 359456 | 260 | 1142376 |
| 21 | 168 | 51 | 808 | 81 | 2152 | 111 | 5736 | 141 | 16896 | 171 | 50600 | 201 | 129896 | 231 | 363240 | 261 | 1331760 |
| 22 | 176 | 52 | 848 | 82 | 2216 | 112 | 5976 | 142 | 17496 | 172 | 52616 | 202 | 134760 | 232 | 376176 | 262 | 1335576 |
| 23 | 184 | 53 | 888 | 83 | 2280 | 113 | 6200 | 143 | 18216 | 173 | 54576 | 203 | 137064 | 233 | 417976 | 263 | 1339392 |
| 24 | 192 | 54 | 928 | 84 | 2344 | 114 | 6456 | 144 | 18936 | 174 | 56256 | 204 | 140872 | 234 | 422664 | 264 | 1427976 |
| 25 | 208 | 55 | 968 | 85 | 2408 | 115 | 6712 | 145 | 19704 | 175 | 58216 | 205 | 145992 | 235 | 438816 | 265 |  |
| 26 | 224 | 56 | 1000 | 86 | 2472 | 116 | 6968 | 146 | 20520 | 176 | 60648 | 206 | 151976 | 236 | 442632 | 266 |  |
| 27 | 240 | 57 | 1032 | 87 | 2536 | 117 | 7224 | 147 | 21336 | 177 | 63080 | 207 | 155776 | 237 | 446448 | 267 |  |
| 28 | 256 | 58 | 1064 | 88 | 2600 | 118 | 7480 | 148 | 22152 | 178 | 65640 | 208 | 159576 | 238 | 459776 | 268 |  |
| 29 | 272 | 59 | 1096 | 89 | 2664 | 119 | 7728 | 149 | 22872 | 179 | 68376 | 209 | 165416 | 239 | 464552 | 269 |  |
| 30 | 288 | 60 | 1128 | 90 | 2728 | 120 | 7992 | 150 | 23664 | 180 | 71112 | 210 | 171336 | 240 | 478776 | 270 |  |

The comparison of the total number of MCS index supported by each TBS between TBS sequence in table 1 and LTE with evaluation assumptions in figure 2. The evaluation assumptions are listed in table 2.

Table 2	Evaluation Assumptions

| MCS Table | In Appendix |
| --- | --- |
| BG Determination | BG1: 292<TBS≤3824 for R>2/3, TBS>3824 for R>1/4 BG2: Others |
| NRe | 6,12,18,36,72,108,144 |
| Y #PRB | 1:1:275 |
| Layers | 1:1:4 |

Figure 2. the number of MCS indices supported by TBSs in table 1 compared with LTE's TBSs

Figure 3 compares that the overhead ratio for TBS sequence in table 1, calculated as (TBSj+1 – TBSj – 8)/ TBSj+1 where TBSj is the j-th entry in the sequence, and the TBSs for LTE. We can see that the TBS overhead ratio for the proposed TBS sequence is comparable to that of LTE's TBS set.

Figure 3 the overhead ratio for TBSs < 171336 in table 1 and LTE TBSs

Observation 2: the overhead ratio of TBS sequence in table 1 is less than 4% which is comparable to that of LTE's TBSs.

In Figure 4, we compare the total number of MCS index supported by each TBS using different methods from [2], [3], [4], [5] and the TBS sequences in table 1 by using the evaluation condition in table 2. Note that a quantization set of {6,12,18,36,72,108,144} REs per PRB as in [5] has been taken into account for comparison.

Figure 4. the total number of MCS indices supported by TBSs in table 1 compared with methods in [2][3][4][5] for the TBS < 10000

Figure 5 shows the overhead ratio of TBS sequence in table 1 and the methods in [2][3][4][5].

Figure 6 gives the percentages of TBSs which can be scheduled by a particular numbers of MCS index for initial transmission.  The top left part in the figure can be regarded as "bad" case, since large portion of TBSs can only be scheduled by a small number of MCS levels.  From the figure, we can see that the proposed TBS sequence in table 1 has the least restriction in MCS level compared with other methods, which means TBS sequence in table 1 outperforms other methods in terms of scheduling flexibility at initial transmission.

Figure 6 	PDF of TB size with different number of MCS for initial transmission

Figure 7. PDF of TB size with different number of MCS for retransmission for BG1

Figure 8. PDF of TB size with different number of MCS for retransmission for BG2

Figure 7 and Figure 8, respectively related to BG1 and BG2,  give the PDF of TBSs which can be scheduled by a particular numbers of MCS index for re-transmission compared with [2][3][4][5]. It is also observed that the proposed TBS sequence in table 1 has the least restriction in MCS level compared with other methods in re-transimssion, which means TBS sequence in table 1 outperforms other methods in terms of scheduling flexibility for re-transmission as well.

Figure 9  the total number of MCS indices supported by TBSs for re-transmission for BG1 in table 1 compared with methods in [2][3][4][5]

Figure 10  the total number of MCS indices supported by TBSs for re-transmission for BG2 in table 1 compared with methods in [2][3][4][5]

Figure 9 and Figure 10 give the total number of available MCS indices for the TBS less than 3824 respectively compared with [2][3][4][5], corresponding to BG1 and BG2. It is also observed that the proposed TBS sequence in table 1 has the best MCSs scheduling flexibility particularly for TBS < ~1000. Zooming in the TBS less than 2000 for comparison with [2] as shown in Figure 11, for BG1, the proposed method has 0 TBSs for the number MCS of 1 and 2 TBSs for the number MCS of 2, while the method in [2] has 11 points for the number MCS of 1 and 14 TBSs for the number MCS of 2. It means that the proposed quantization method using the proposed TBS sequence has the most flexibility for MCSs scheduling for re-transmission. For the method in [2], it has only one MCS option for the retransmission which is the same as initial transmission.

Figure 11 Comparison with [2] for the TBS less than 2000

Observation 3: For TBS determination by sequence, TBSs is superior in terms of scheduling flexibility when compared to LTE TBS and other TBS determination methods provided in [2],[3],[4] and [5].

Proposal 2: Intermediate TBSs should be quantized by a predetermined TBS sequence to obtain the actual TBS for transmission and re-transmissions.

Proposal 3: TBS sequence in Table 1 should be adopted to quantize the intermediate number of information bits into actual TBS.

# RV Sequences for Grant Free UL

When full rate matching buffer is used, fix RVs {0,1,2,3} for BG1 are set at {0,17,33,56}*Z and fix RVs {0,1,2,3} for BG2 are set at {0,13,25,43}*Z.

# 3.1  Performance Evaluation

According to the working assumption from email discussion, there are 3 candidates for RV sequences: {0,2,3,1}, {0,3,0,3} and {0,0,0,0}. And, the RV sequences candidates of {0,3,0,3} and {0,0,0,0} are simulated with simulation assumptions in Table 3.The normalized throughput performances results are shown in Figure 12 to Figure 19. The normalized throughput is calculated by the following expression:

Wherein,  is modulation order (2 for QPSK, 4 for 16QAM, 6 for 64QAM and 8 for 256QAM),  is code rate,  is block error rate and  is average number of transmission times for a new data.

Table 3	Simulation Assumptions

| Channel | AWGN |
| --- | --- |
| Modulation | QPSK, 16QAM, 64QAM, 256QAM |
| TBS | 1024 |
| Code Rate | 1/5, 1/3, 2/5, 1/2, 2/3, 3/4, 5/6, 8/9 |
| RV Sequences | {0,3,0,3} and {0,0,0,0} |
| Decoding Aalgorithm | Flooding BP, Max iterations =50 |
| Codeword Length | Same Length for Each Tx |

Figure 12Rate=1/2& QPSK			Figure 13Rate=3/4& QPSK

Figure 14Rate=1/3& 16QAM			Figure 15Rate=2/5 & 16QAM

Figure 16Rate=5/6& 64QAM			Figure 17Rate=8/9& 64QAM

Figure 18Rate=5/6&256QAM			Figure 19Rate=8/9&256QAM

According to the simulations, it is observed that RV sequenceof {0, 3, 0, 3} has better performance compared with RV sequenceof {0, 0, 0, 0}

Observation 4: RV sequence of {0, 3, 0, 3} has better performance compared with RV sequence of {0, 0, 0, 0}.

# 3.2 Processing Latency Analysis

For the case that the 1st transmission is missed as shown in Figure 20, RV sequence of {0, 2, 3, 1} needs 3 times transmissions because RV2 is not self-decodable. However, for RV sequence of {0, 3, 0, 3} and RV sequenceof {0, 0, 0, 0}, they need only 2 times transmissions since their 2nd transmission (RV3 or RV0) are self-decodable. Therefore, for the case as in Figure 22, RV sequence of {0, 2, 3, 1} has longer latency than RV sequence of {0, 3, 0, 3} and RV sequenceof {0, 0, 0, 0}.

Figure 20	Example of Case of 1st Transmission Missed

Observation 5:  Processing latency of RV sequenceof {0, 0, 0, 0} and {0, 3, 0, 3} outperform  processing latency of RV sequenceof  {0, 2, 3, 1} when 1sttransmission is missed .

Proposal 4: For UL transmission without UL grant, RV sequences of {0, 2, 3, 1}, {0, 3, 0, 3} and {0, 0, 0, 0} which are configured by UE-specific RRC signaling in working assumption should be confirmed.

# Conclusion

In this contribution, we compared scheduling flexibility of TBS determined by formula and TBS sequence. We also evaluated the RV sequences for UL grant free. In summary, we have the following observations and proposals.

Observation 1: It is agreed in the previous meeting that BG determination rule for initial transmission depends on code rate and actual TBS.

Observation 2: the overhead ratio of TBS sequence in table 1 is less than 4% which is comparable to that of LTE's TBSs.

Observation 3: For TBS determination by sequence, TBSs is superior in terms of scheduling flexibility when compared to LTE TBS and other TBS determination methods provided in [2],[3],[4] and [5].

Observation 4: RV sequence of {0, 3, 0, 3} has better performance compared with RV sequence of {0, 0, 0, 0}.

Observation 5:  Processing latency of RV sequenceof {0, 0, 0, 0} and {0, 3, 0, 3} outperform  processing latency of RV sequenceof  {0, 2, 3, 1} when 1st  transmission is missed .

Proposal 1: Actual TBS rather than intermediate number of information bits should be used for BG determination.

Proposal 2: Intermediate TBSs should be quantized by a predetermined TBS sequence to obtain the actual TBS for transmission and re-transmissions.

Proposal 3: TBS sequence in Table 1 should be adopted to quantize the  intermediate number of information bits into actual TBS.

Proposal 4: For UL transmission without UL grant, RV sequences of {0, 2, 3, 1}, {0, 3, 0, 3} and {0, 0, 0, 0} which are configured by UE-specific RRC signaling in working assumption should be confirmed.

# References

- Chairman's Notes RAN1 90 bis final

- R1-1721447, “TBS Determination With LDPC Considerations,” Ericsson, RAN1 NR #91

- R1-1721544, On TBS determination formula, MediaTek Inc., RAN1 NR #91

- R1-1721390	Discussion on TBS and MCS designs	Huawei, HiSilicon, RAN1 NR #91

- R1-1721470	TBS and Base-graph Determination	 	Qualcomm Incorporated, RAN1 NR #91

- R1-1721406  Remaining details of LDPC coding  ZTE, Sanechips

- R1-1720687  Remaining Details on DL/UL Resource Allocation   Qualcomm

# Appendix

#### MCS table for example

| MCS Index | Modulation Order | code rate x 1024 | TBS Index |
| --- | --- | --- | --- |
| 0 | 2 | 120 | 0 |
| 1 | 2 | 157 | 1 |
| 2 | 2 | 193 | 2 |
| 3 | 2 | 251 | 3 |
| 4 | 2 | 308 | 4 |
| 5 | 2 | 379 | 5 |
| 6 | 2 | 449 | 6 |
| 7 | 2 | 526 | 7 |
| 8 | 2 | 602 | 8 |
| 9 | 2 | 679 | 9 |
| 10 | 4 | 340 | 9 |
| 11 | 4 | 378 | 10 |
| 12 | 4 | 434 | 11 |
| 13 | 4 | 490 | 12 |
| 14 | 4 | 553 | 13 |
| 15 | 4 | 616 | 14 |
| 16 | 4 | 658 | 15 |
| 17 | 6 | 438 | 15 |
| 18 | 6 | 466 | 16 |
| 19 | 6 | 517 | 17 |
| 20 | 6 | 567 | 18 |
| 21 | 6 | 616 | 19 |
| 22 | 6 | 666 | 20 |
| 23 | 6 | 719 | 21 |
| 24 | 6 | 772 | 22 |
| 25 | 6 | 822 | 23 |
| 26 | 6 | 873 | 24 |
| 27 | 6 | 910 | 25 |
| 28 | 6 | 948 | 26/26A |
| 29 | 2 | reserved |  |
| 30 | 4 |  |  |
| 31 | 6 |  |  |