3GPP TSG-RAN WG1 Meeting #91 	R1-1721703

27th November – 1st December 2017

Reno, USA

Agenda item:	7.3.3.2

Source: 	Qualcomm Incorporated

Title: 	Summary of DL/UL scheduling and HARQ management

Document for:	Discussion/Decision

# Introduction

This document provides a summary of the issues addressed by the submissions [2]-[23] to RAN1 #91 in 7.3.3.2 which can be essentially grouped along the following topics.

- DL/UL Scheduling

- UE Processing Time

- HARQ-ACK Feedback Timing

- Maximum number of HARQ processes

- Other aspects of HARQ management

Note that HARQ HARQ-ACK multiplexing and bundling was also discussed in the contributions but those aspects are combined in the summary for 7.3.4.2 Other Aspects on Carrier Aggregation. This includes proposals which address both CA and CBG-based re-transmission, in addition to non-CA non-CBG semi-static and dynamic TDD and FDD.

Some aspects related to scheduling and HARQ management discussed here are also related to topics in 7.3.3.1.

This is a 4th revision updating R1-1721652.

# DL/UL Scheduling

Throughout this summary, different aspects related to slot-based scheduling and non-slot (or mini-slot) based scheduling are discussed. It is therefore important to establish some of the terminology and properties related to these before proceeding. The following summary is provided based on [9] with some modification reflecting other contributions.

Table 1. Slot-based and non-slot based scheduling

| Slot-based scheduling | Non-slot (mini-slot) based scheduling |
| --- | --- |
| CORESET used for PDCCH scheduling PDSCH/PUSCH is restricted to first three symbols. | CORESET used for PDCCH scheduling PDSCH/PUSCH can be beyond the first three symbols of a slot. |
| CORESET monitoring periodicity is in integer multiples of 14 symbols. | CORESET monitoring periodicity can be < 14 symbols. |
| PDSCH starting symbol is within first [4] symbols of a slot, spans more than [7] symbols up to the entire slot. First DMRS for PDSCH occurs in 3rd or 4th symbol of a slot (DL-DMRS-typeA-pos). | PDSCH starting symbol can be beyond first four symbols of a slot, and spans 2/4/7 symbols. First DMRS for PDSCH occurs in 1st symbol of PDSCH, and 1 additional DMRS if PDSCH spans 7-symbols (DL-DMRS-typeB-pos). |

It is clear from various contributions that the flexibilities associated with non-slot based scheduling might be be considered as an optional UE capability in general [9], or supported only in a very limited fashion for all UEs in Rel 15 [5]. The contribution [2] suggests further reduction in specification scope to benefit implementation.

The following discussion between 7.3.3.1/7.3.3.2 arrived at the following proposal, but this was later captured in a another agreement for 7.3.3.1.

Offline Proposal: For unicast scheduling, a UE can be RRC configured with DMRS type A, or A and B.

- FFS whether explicit bit is used in RRC

(No consensus) Proposal 1: There should be at least one UE capability for which the minimum gap between the starting point of two PDCCH monitoring occasions across all search space sets is no less than [125] us.

Views/comments are welcome below.

| Company | Comment |
| --- | --- |

# UE Processing Time

In this section, we survey processing time updates since RAN1 #90bis, along with additional considerations related to UCI multiplexing, carrier aggregation, mixed uplink and downlink numerology, and non-slot-based scheduling.

## Slot-based Scheduling

Here we update the UE processing times related to slot-based scheduling taking into account carrier aggregation. No UCI multiplexing with data is assumed for any these cases, but processing time discussion for these aspects is provided in Section 3.3. Contributions [2],[3],[17] provide updated values addressing carrier aggregation, although some are conditioned upon the progress in other topics such as cross-carrier scheduling and control channel search space.

### Survey of Capability #1

Table 2. UE Processing Time and HARQ Timing (Capability #1)

| Configuration | Reference | HARQ Timing Parameter | Units | 15 KHz SCS | 30 KHz SCS | 60 KHz SCS | 120 KHz SCS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Front-loaded DMRS only | [1] | N1 | Symbols | [8] | [10] | [14] | [14-21] |
|  | [2]1 | N1 | Symbols | N/C | N/C | N/C | [21] |
|  | [3]2 | N1 | Symbols | N/C | N/C | [17] | [21] |
|  | [9] | N1 | Symbols | No Proposal | No Proposal | No Proposal | No Proposal |
|  | [11]3 | N1 | Symbols | N/C | N/C | N/C | [18] |
|  | [17]4 | N1 | Symbols | N/C | N/C | N/C | N/C |
| Front-loaded + additional DMRS | [1] | N1 | Symbols | [13] | [13] | [17] | [21] |
|  | [2]1 | N1 | Symbols | N/C | N/C | N/C | N/C |
|  | [3]2 | N1 | Symbols | N/C | [14.5] | [20] | [24] |
|  | [9] | N1 | Symbols | No Proposal | No Proposal | No Proposal | No Proposal |
|  | [11]3 | N1 | Symbols | N/C | N/C | N/C | N/C |
|  | [17]4 | N1 | Symbols | N/C | N/C | N/C | N/C |
| Frequency-first RE-mapping | [1] | N2 | Symbols | [9] | [11] | [17] | [31] |
|  | [2]1 | N2 | Symbols | N/C | N/C | N/C | N/C |
|  | [3]2 | N2 | Symbols | [11] | [19] | [35] | [45] |
|  | [9] | N2 | Symbols | No Proposal | No Proposal | No Proposal | No Proposal |
|  | [11]3 | N2 | Symbols | N/C | N/C | N/C | N/C |
|  | [17]4 | N2 | Symbols | N/C | N/C | N/C | N/C |

- [2] This is also extended to CA operation up to 4 CCs.

- [3] This is also extended to CA operation. The N2 also extends to “Simple aperiodic CSI” cases (i.e., WB CQI/RI/PMI w/ 2 CSI-RS ports; location of CSI-RS at symbol#x, x<=6”). Other configurations for aperiodic CSI reporting require 20, 24, 44, and 52 symbols across the respective SCS.

- This only applies to non-CA case.

- N2 should be increased by 1 if any data REs are at the start of the scheduled transmission. Further clarifications on timeline provided in case of UCI multiplexing.

Upon review of the details underlying the different estimates, the following table is proposed below with some compromises considered. First, at the higher sub-carrier spacings the higher N1 processing times were assumed to provide some margin when the slot-durations were smaller. This was not however taken in the case of N2 since some relaxations may be associated also with CSI reporting, and here we instead propose to de-couple these times in Section 3.3. N2 was incremented by 1 since PUSCH may have DMRS FDM’d in the 1st symbol with data, or DMRS may come after the 1st PUSCH symbol altogether. Note that in some cases e.g., [2],[9],[17], conditions on number of supported carriers or number of supported blind decodes (and channel estimations related to blind decodes) were associated with support of the values below. Thus, agreement of this table below should take into account progress along these other design aspects.

The following was discussed online and captured. Please see Chairman’s notes.

Offline Consensus (Proposal 2): The baseline UE processing time capability in NR Release 15 for slot-based scheduling, including CA case with no cross-carrier scheduling and with single numerology for PDCCH, PDSCH, and PUSCH and no UCI multiplexing, is given by Table 2-1 below.

- FFS whether processing times can be supported also for cross-carrier scheduling

- FFS if front-loaded DMRS occurs beyond 4th symbol of slot, add 1 symbol to N1 in table

Table 2-1. UE Processing Time and HARQ Timing (Capability #1)

| Configuration | HARQ Timing Parameter | Units | 15 KHz SCS | 30 KHz SCS | 60 KHz SCS | 120 KHz SCS |
| --- | --- | --- | --- | --- | --- | --- |
| Front-loaded DMRS only | N13 | Symbols | 8 | 10 | 17 | 20 |
|  |  | us | 571 | 357 | 303 | 178 |
| Front-loaded + additional DMRS | N1 | Symbols | 13 | 13 | 20 | 24 |
|  |  |  | 928 | 464 | 357 | 214 |
| Frequency-first RE-mapping | N21 | Symbols | 10 | 12 | 23 | 36 |
|  |  |  | 71.4 | 35.7 | 17.9 | 8.9 |

- If 1st symbol of PUSCH is data-only or FDM data with DMRS, then add 1 symbol to N2 in table.

Views/comments are welcome below.

| Company | Comment |
| --- | --- |

### Survey of Capability #2

Table 3. UE Processing Time and HARQ Timing (Capability #2)

| Condition | Reference | HARQ Timing Parameter | Units | 15 KHz SCS | 30 KHz SCS |
| --- | --- | --- | --- | --- | --- |
| Front-loaded DMRS only | [1] | N1 | Symbols | [2.5-4] | [2.5-6] |
|  | [2]1 | N1 | Symbols | 3 | 5 |
|  | [3]2 | N1 | Symbols | No Proposal | No Proposal |
|  | [9] | N1 | Symbols | No Proposal | No Proposal |
|  | [11] | N1 | Symbols | 3 | 5 |
|  | [17] | N1 | Symbols | No Proposal | No Proposal |
| Front-loaded + additional DMRS | [1] | N1 | Symbols | [12] | [12] |
|  | [2]1 | N1 | Symbols | 13 | 13 |
|  | [3]2 | N1 | Symbols | No Proposal | No Proposal |
|  | [9] | N1 | Symbols | No Proposal | No Proposal |
|  | [11] | N1 | Symbols | N/C | N/C |
|  | [17] | N1 | Symbols | No Proposal | No Proposal |
| Frequency-first RE-mapping | [1] | N2 | Symbols | [2.5-4] | [2.5-6] |
|  | [2]1 | N2 | Symbols | 4 | 6 |
|  | [3]2 | N2 | Symbols | No Proposal | No Proposal |
|  | [9] | N2 | Symbols | No Proposal | No Proposal |
|  | [11] | N2 | Symbols | 5.5 | 5.5 |
|  | [17] | N2 | Symbols | No Proposal | No Proposal |

- [2] Proposes to remove UE capability differentiation on front-loaded + additional DMRS since there was no significant change from baseline.

- [3] Further restrictions were recommended for this processing time

  - Further restrict of FDMing DMRS and PUSCH due to UE processing timeline. If FDMing DMRS and PUSCH is allowed, one additional OFDM symbol is added to N2.

  - Only CP-OFDM is allowed for PUSCH.

  - Reporting aggressive UE capability for N1 (in case of front-loaded DMRS only) along with support for 0%, 20% or 100% of max PRB number for DL. This is also suggested by [6].

  - Indicate support of 1,2, or 4 CC.

For this second case, there seemed to be less convergence on values and more conditions associated with supporting this capability. Therefore, the more relaxed set of values are suggested for the final proposal regarding the processing times. As with the baseline UE processing time proposal, some of these are contingent upon progress in other design aspects.

Proposal 3: The aggressive UE processing time capability in NR Release 15 for slot-based scheduling, including CA case with no cross-carrier scheduling and with single numerology for PDCCH, PDSCH, and PUSCH, is given by Table 3-1 below.

- FFS whether processing times can be supported also for cross-carrier scheduling

- FFS whether two additional parameters can be carried to indicate the support 20% or 100% of max PRB number (respectively for UL and DL) for the aggressive UE processing time.

Table 3-1. UE Processing Time and HARQ Timing (Capability #2)

| Configuration | HARQ Timing Parameter | Units | 15 KHz SCS | 30 KHz SCS |
| --- | --- | --- | --- | --- |
| Front-loaded DMRS only | N1 | Symbols | 3 | 5 |
|  |  | us | 214 | 178 |
| Front-loaded + additional DMRS | N1 | Symbols | 12 | 12 |
|  |  | us | 214 | 178 |
| Frequency-first RE-mapping | N21 | Symbols | [4] | 6 |
|  |  | us | 285 | 214 |

- If 1st symbol of PUSCH is data-only or FDM data with DMRS, then add 1 symbol to N2 in table.

Views/comments are welcome below.

| Company | Comment |
| --- | --- |

## CSI Reporting and UCI Multiplexing

Recall that the original framework for UE processing time was based on simplified assumptions which did not include UCI multiplexing among ACK or CSI-reports or Data. The notion of extending this was expressed in [7], and addressed quantitatively in [3] and [17] while other contributions did not explicitly provide any adjustments. Here, we first provide a proposal from [17] to first establish the processing time terminology in this context, and then use this to extend to UCI multiplexing.

Proposal 4: NR should specify UE capability for processing time N3 for reporting CSI in the case of no UCI multiplexing, associated with additional conditions (e.g., CSI parameter, number of antenna ports, sub-carrier spacing, CSI-RS location), in terms of symbols. If the Y indicated in DCI for CSF reporting exceeds the capability of the UE then the UE is not expected to report CSI, otherwise the UE reports CSI.

- FFS on capabilities, e.g., Table 5 below (taken from [3]).

Table 5. UE processing time for Aperiodic CSI (ACSI) reporting

| Configuration | Units | 15 KHz SCS | 30 KHz SCS | 60 KHz SCS | 120 KHz SCS |
| --- | --- | --- | --- | --- | --- |
| Simple ACSI1 | Symbols | [11] | [19] | [35] | [45] |
| Non-simple ACSI | Symbols | [20] | [24] | [44] | [52] |

- Conditions for simple ACSI are WB CQI/RI/PMI w/ 2 CSI-RS ports; location of CSI-RS at symbol#x, x<=6.

In [17], a set of conditions to extend the current processing time were presented. These are proposed below slightly modified to address a few other combinations not originally included.

The following was discussed and captured. Please see Chairman’s notes.

(Offline consensus) Proposal 5:  In the case of multiplexing HARQ-ACK with uplink data on PUSCH

- N1’ the number of OFDM symbols required for UE processing from the end of PDSCH to the earliest possible start of the corresponding ACK/NACK transmission on PUSCH from UE perspective

  - N1’ ≥ N1 + d where N1 is based on the UE capability for ACK-only

- N2’ the number of OFDM symbols required for UE processing from the end of PDCCH containing the UL grant reception to the earliest possible start of the corresponding the same PUSCH transmission from UE perspective

  - N2’ ≥ N2 + d where N2 is based on the UE capability for sending data-only on PUSCH

- d = [0 or 1] symbols

- UE is not expected transmit the HARQ-ACK multiplexed with uplink data if the network set the values of K1 and/or K2 without leaving sufficient time for UE processing

- FFS: how to much time is needed to multiplex CSI reports, depending on outcome from MIMO session.

The following two agreements were made and one proposal is given to ensure consistency.

Agreements:

In the case of multiplexing HARQ-ACK with uplink data on PUSCH

- N1’ the number of OFDM symbols required for UE processing from the end of PDSCH to the earliest possible start of the corresponding ACK/NACK transmission on PUSCH from UE perspective

  - N1’ ≥ N1 + d where N1 is based on the UE capability for ACK-only

- N2’ the number of OFDM symbols required for UE processing from the end of PDCCH containing the UL grant reception to the earliest possible start of the corresponding the same PUSCH transmission from UE perspective

  - N2’ ≥ N2 + d where N2 is based on the UE capability for sending data-only on PUSCH

- d = [1] symbols

- UE is not expected transmit the HARQ-ACK multiplexed with uplink data if the network set the values of K1 and/or K2 without leaving sufficient time for UE processing

- FFS: how to much time is needed to multiplex CSI reports, depending on outcome from MIMO session.

Agreement:

- For mixed numerology case, the UE processing time needed between the end of PDSCH reception (with SCS u1) and start of PUCCH transmission (with SCS u2) is max(T1, T2), where

  - T1 = N1_1 x S1, N1_1 is the processing time in the single numerology with SCS u1 and S1 is the symbol duration of SCS u1;

  - T2 = N1_2 x S2, N1_2 is the processing time in the single numerology with SCS u2 and S2 is the symbol duration of SCS u2;

- For mixed numerology case, the UE processing time needed between the end of PDCCH reception  (with SCS u1) and start of PUSCH transmission (with SCS u2) is max(T1, T2), where

  - T1 = N2_1 x S1, N2_1 is the processing time in the single numerology with SCS u1 and S1 is the symbol duration of SCS u1;

  - T2 = N2_2 x S2, N2_2 is the processing time in the single numerology with SCS u2 and S2 is the symbol duration of SCS u2;

Proposal 5’ (Offline consensus): In the case of multiplexing HARQ-ACK with uplink data on PUSCH

- In the case of mixed numerology between the UL and DL, the UE processing times for N1’, N2’ apply according to the lowest subcarrier spacing between the UL and DL numerologies

Companies commented that the above should have a set of d values (or candidates) defined for completeness. Also, N3 needs to be provided for further extension to multiplexing with CSI report.

Views/comments are welcome below.

| Company | Comment |
| --- | --- |

## Mixed Numerologies between UL and DL

In handling mixed numerologies, there can be some complication since different aspects of processing are split between the uplink and downlink numerologies. Two approaches were provided in [2] and [17] to address this by taking N1 and N2 and applying only one of the existing numerologies. Below is a modified proposal which take the more relaxed timing, as can be seen in earlier Table 2-1 and 3-1.

Note this was agreed in separate session.

Offline Consensus (Proposal 6): In the case of mixed numerology between the UL and DL, the UE processing times for N1 and N2 apply according to the lowest subcarrier spacing between the UL and DL numerologies.

Views/comments are welcome below.

| Company | Comment |
| --- | --- |

## Mini-slot Scheduling

Regarding non-slot scheduling, which in Section 2 is regarded as any supported flexibility in Table 1, there are additional timeline considerations related to the fact that the pipeline depth cannot be leveraged when the transmissions durations are a very small number of symbols. This was nicely illustrated in [2], and given the focus from several other contributions on slot-based scheduled e.g., [3], [17], this is somewhat a commonly observed limitation. The one exception comes from [9], which suggests that the minimum UE-processing times and HARQ timings be the same.

At least upon review of [2] and [9], it is possible to reconcile these views based on the observations. For instance, if one examines the proposed values in [2], the only framework which needs extension upon the slot based scheduling cases are (1) 7-symbol distributed DMRS case (since channel interpolation need only span 7 symbols), and (2) the 60kHz case which is not specified aggressively.

For now we consider a starting point for the baseline capability to address the points raised in [2] and [9].

Offline consensus (Proposal 7) for working assumption (bracketed values are targets):  The starting point for baseline UE processing time capability in NR Release 15 for non-slot-based scheduling, including CA case with no cross-carrier scheduling and with single numerology for PDCCH, PDSCH, and PUSCH and no UCI multiplexing, is given by Table 7-1 below.

- FFS whether processing times can be supported also for cross-carrier scheduling

- FFS whether additional dependence on time-domain allocation length should be given

- FFS (for N1) regarding front-loaded DMRS location

- FFS (for N1) processing times in relation to CORESET configuration where UE finds scheduling DCI

- FFS if there is a second lower latency UE capability for non-slot based scheduling

Table 7-1. UE Processing Time and HARQ Timing for Non-Slot

| Configuration | HARQ Timing Parameter | Units | 15 KHz SCS | 30 KHz SCS | 60 KHz SCS | 120 KHz SCS |
| --- | --- | --- | --- | --- | --- | --- |
| Front-loaded DMRS only | N11 | Symbols | [8] | [10] | [17] | [20] |
| Front-loaded + additional DMRS | N11 | Symbols | [13] | [13] | [20] | [24] |
| Frequency-first RE-mapping | N22 | Symbols | [10] | [12] | [23] | [36] |

- FFS whether additional dependence on time-domain allocation length should be given.

- If 1st symbol of PUSCH is data-only or FDM data with DMRS, then add 1 symbol to N2 in table.

Views/comments are welcome below.

| Company | Comment |
| --- | --- |

## Other Comments

Two remaining areas of discussion were raised in [2] and [9], respectively addressing the default processing time assumptions and UE assumptions when there are switches. Slightly modified proposal from [2] is included below, where the restriction to initial access and re-transmission is not included since this would need more discussion. For instance, in the case of initial access there may be some aspects of L2 packet inspection which are not including in the processing time evaluation.

The default processing time proposal from [2] is combined in a later section on HARQ-ACK feedback timing.

Needs discussion (Proposal 8): UE is not expected to transmit anything on a symbol if it follows within T us from a DL symbol being received by that UE on unpaired spectrum for a given serving cell.

- Note that the exact value of T may not be in RAN1 specification.

Views/comments are welcome below.

| Company | Comment |
| --- | --- |

# HARQ-ACK Feedback Timing

In this section, several aspects for HARQ-ACK feedback timing are discussed including timing indication, semi-static operation, and default timing.

## Timing Indication

Regarding the timing indication, open issues are related to what the indication represents and how many bits to signal the indication. Upon reviewing proposals among companies, we have the following survey.

- Slot-based scheduling

  - 2-bits in DCI signal slot which contains the K1 slot-timing

    - [2], [10] (separate from ACK resource field), [4] (shared with ACK resource field), [20]

  - 3-bits in DCI signal slot which contains the K1 slot-timing

    - [13] (separate from ACK resource)

  - Support also 0-bit semi-static operation in TDD and FDD

    - [4], [6], [20] (fixed and slot-dependent)

- Mini-slot based scheduling

  - 2-bits in DCI signal slot which contains the K1 slot-timing (and not the resource within the slot)

    - [2] (separate configuration from slot-based)

  - 3-bits in DCI signal timing delay between PDSCH and ACK in symbols

    - [13]

  - Two separate timing parameters in DCI representing both slots and symbols respectively

    - [4]

In addition to the above, [7] did not provide specific proposals but generally shared the motivation of reducing DCI overhead and configuring timing separately for slot-based and non-slot based operation. [12] provided design criteria which seemed to be supported already from the above framework. Therefore, at least it seems the following proposal below would be essential in enabling both semi-static TDD and dynamic TDD, while also supporting FDD; and this seems shared by almost all of the proposals put forth in contributions.

Note that the following was discussed and agreed. Please see Chairman’s notes for final wording.

Offline Proposal: [2 or 3]-bits are used to indicate K1 slot-timing in DCI

- RRC configures the set of K1 values to be indexed by these bits

Note about PUCCH offline that consensus was for separate ACK resource field (which includes symbol location w/in slot)

Regarding non-slot based operation, more discussion is needed on how to signal the relative timing for the ACK. Note that this may also depend on the symbol timing discussions also in PUCCH design sessions (e.g., 7.3.2.1).

## Default Timing

Finally, some discussion related to the default HARQ timing indication was provided in [4],[10],[11],[20], when the UE has not been configured with the HARQ timing values to be signaled in DCI. This is also combined with the notion of default UE processing time proposal given in [2].

The following was discussed and captured. Please see Chairman’s notes.

Proposal 10: For the case when RRC connection has not yet been established, the UE processing time should be assumed to be the maximum values among all conditions under the same SCS.

- FFS for conditions other than slot-based scheduling where the maximum is applied

Proposal 11: For the case when RRC connection has not yet been established and the timings are unknown to UE

- A default values for the timing indication of the HARQ-ACK relative to the PDSCH can be predefined or configured by system information

- This values is mapped to one of the values which are indicated by a bit field in DCI.

Views/comments are welcome below.

| Company | Comment |
| --- | --- |

# Maximum number of HARQ processes

Regarding the maximum number of HARQ processes, the survey of positions at this meeting is given below.

- 16: [2],[13],[16],[20]

- 8: [5],[10],[23]

Additionally, [9] encourage using smaller values such as 8 and [17] also encourage reducing the maximum number in light of latency improvements, reduced overhead and complexity. It should be noted that the configurability on the maximum number of HARQ processes was agreed at RAN1 NR AdHoc #3 (Nagoya).

Agreements:

- Maximum number of HARQ processes for unicast PDSCH is configured per cell for a UE

  - FFS impact on DCI design

Moreover, since the soft buffer size at the UE is de-coupled from the specification, the maximum number supported in specification does not mean the UE must support the largest possible HARQ RTT with the largest bandwidth. Therefore, the following proposal is considered for discussion at this meeting.

This was discussed and captured in Chairman’s notes.

Offline consensus (Proposal 11): The maximum number of DL HARQ processes per carrier that can be signalled in DCI is 16. The maximum number of UL HARQ processes per carrier that can be signalled in DCI is 16.

Views/comments are welcome below.

| Company | Comment |
| --- | --- |

# Other aspects of HARQ management

A few other important aspects for HARQ management were discussed in this agenda and are covered here.

First, [17],[20] both discuss certain aspects related to dynamic HARQ timing. [17] generally raises awareness of out-of-order possibilities, while [20] discuss details related to the specific case of re-transmission before ACK on downlink. [2] proposes to put a lower limit of 125us between the two closest scheduling instances (i.e., in terms of separate PDCCH monitoring occasions) in specification.

With regard to HARQ management, [23] discusses how the HARQ process pool should be unified between non-slot and slot-based scheduling. This is also a consequence of the proposal for mandatory support of non-slots by [5].

Since several of these proposals serve to reduce the scope of specification for Rel-15, they are stated below. However, the topic of ordering with dynamic HARQ timing needs more discussion and some conditions should also be added to reduce the scope.

The following proposal has been updated to a conclusion for this meeting.

Proposal 12: From the UE Perspective, transport blocks should not cross slot boundaries.

In RAN1 NR AdHoc#2, there was the following agreement:

Agreements:
 For grant-based DL or UL, transmissions where a TB spans multiple slots or mini-slots can be composed of repetitions of the TB:

- The repetitions follow an RV sequence

- FFS for grant-based DL or UL transmissions, if a TB can span multiple slots without repetitions

Conclusion:

There has been no consensus that TB can span multiple slots without repetitions for grant-based DL or UL transmissions

Proposal 13: A single HARQ process pool is configured per carrier for DL (and also for UL). This single DL pool is shared by all DL transmissions on a carrier irrespective of CORESET configuration. The single UL pool is shared by grant-free and grant-based UL transmissions.

Views/comments are welcome below.

| Company | Comment |
| --- | --- |

# Additional comments

Additional views/comments are welcome below.

| Company | Comment |
| --- | --- |

# References

- RAN1 #90bis Chairman’s Notes, Prague, Czech Republic

- R1-1719401, “Remaining issues on HARQ”, Huawei, HiSilicon

- R1-1719552, “HARQ-ACK and UL Scheduling Timing Relationship”, MediaTek, Inc.

- R1-1719614, “Discussion on HARQ-ACK codebook and HARQ feedback timing”, Fujitsu

- R1-1719648, “Remaining details on DL/UL scheduling and HARQ management”, AT&T

- R1-1719794, “Remaining issues on DL/UL scheduling and HARQ”, Vivo

- R1-1719930, “HARQ process and feedback for NR”, LG Electronics

- R1-1719993, “Discussion on HARQ-ACK transmission”, Guangdong OPPO Mobile Telecom

- R1-1720095, “On DL/UL Scheduling and HARQ management”, Intel Corporation

- R1-1720203, “Discussion on HARQ management and HARQ-ACK feedback”, CATT

- R1-1720340, “HARQ Management and Feedback”, Samsung

- R1-1720368, “HARQ-Ack multiplexing and bundling”, ZTE, Sanechips

- R1-1720480, “On remaining details of HARQ procedure”, Nokia, Nokia Shanghai Bell

- R1-1720498, “HARQ-ACK codebook for CBG-based transmission”, Panasonic

- R1-1720554, “HARQ-ACK codebook with dynamic timing indication”, InterDigital, Inc.

- R1-1720561, “Number of HARQ processes for co-existence with TD-LTE”, NEC

- R1-1720688, “DL/UL Scheduling, Processing Time and HARQ management”, Qualcomm, Incorporated

- R1-1720712, “HARQ ACK multiplexing for NR”, HTC

- R1-1720749, “HARQ-ACK transmission for DL transmission in NR”, CATR

- R1-1720822, “DL/UL scheduling and HARQ management”, NTT DOCOMO, INC.

- R1-1720876, “Discussion on HARQ-ACK multiplexing and bundling for NR”, WILUS Inc.

- R1-1720913, “UE-driven HARQ-ACK bundling for NR”, Sequans communications

- R1-1721013, “On HARQ Management”, Ericsson

# Appendix

The following Table A.1 from R1-1716865 is provided for reference. The purpose of this table originally was to provide reasonable scope for understanding processing times achievable in NR. The current proposals in this summary considering widening the scope such that slot-based scheduling is covered in the set of operations, which may mean additional changes the such as the CORESET configuration.

Table A.1. Candidate factors for UE processing time (N1,N2)

|  | N1 | N2 |
| --- | --- | --- |
| Nominal assumptions | Single carrier / Single BWP / Single TRP - Full range of MCS and multi-layer support up to the 4-layer MIMO and 256-QAM - Up to 3300 active subcarriers2 PDCCH - Same numerology / BWP as PDSCH - Single grant monitored for PDSCH - 44 blind decodes, single symbol CORESET PDSCH - PDSCH does not precede PDCCH - 14-symbol slot-based scheduling - Frequency-first RE-mapping, no time-interleaving of CBs across TB PUCCH - Short formats for HARQ-ACK | Single carrier / Single BWP / Single TRP - Full range of MCS and multi-layer support up to the 2-layer MIMO and 64-QAM - Up to 3300 active subcarriers PDCCH - Same numerology / BWP as PUSCH - Single grant monitored for PUSCH - 44 blind decoding, single symbol CORESET PUSCH - 14-symbol slot-based scheduling - No time-interleaving of CBs across TB - DFTsOFDM or OFDM - Front loaded DMRS for low latency4 - No UCI multiplexing |
| Candidate factors | - SCS - DMRS configuration3 - [Percentage of peak rate] - [RE-mapping1] | - SCS - RE-mapping (depending on specification)1 - [Percentage of peak rate] |

1Preferred RE-mappings may be specified in cases where decisions are pending.
2Some consideration can also be given to N1 when the 3300 active subcarriers are achieved with carrier aggregation. 
3Front loaded and distributed patterns are assumed. For front loaded, the 3rd and 4th symbols have DMRS. 
4N2 is measured from the start of DMRS (since front-loaded assumption is made). One DMRS is TDM with PUSCH.