3GPP TSG RAN WG1 Meeting #91	R1-1721713

Reno, USA, November 27th- December 1st, 2017

Agenda Item:	7.2.3.1

Source:	Huawei, HiSilicon

Title:	Summary of RS multiplexing further remaining issues

Document for:	Discussion and decision

Introduction

In this contribution, the views of RS multiplexing is summarized from companies’ Tdocs. Some suggestions are also provided.

Key remaining issues for RS multiplexing

Key issue-1: Multiplexing for CSI-RS

- Issue 1-a: CSI-RS and PDCCH (CORESET)

[Agreement] A CSI-RS resource can be configured on RBs outside CORESET in CORESET symbols from UE perspective;

Above applies at least for the case where PDCCH and CSI-RS are spatially QCL-ed;

Above both applies for non-slot and slot based cases.

The remaining issues include the additional restriction on the use cases:

| - CSI-RS and CORESET in the same OFDM symbol(s), when the CSI-RS and CORESET are NOT spatially QCL-ed |  |
| --- | --- |
| Can be used (including UE behavior): | Samsung, Huawei, HiSilicon, ZTE, Sanechips, vivo |
| Can not be: | LGE (Even for multi-panel UE, UE can still use the multi-panels to combine RF signals. It is not desirable mandating UE implementation in a specific way (different Rx beam per panel for receiving CORESET and CSI-RS, respectively), so that it is safe to keep the previous agreement shown above (i.e., same spatial QCL condition) at least in NR Phase I), Qualcomm |
| - Whether there is a BW restriction for CSI-RS? |  |
| No restriction: | Samsung, ZTE, Sanechips, vivo, Huawei, HiSilicon, Ericsson |
| Restricted: (details) | Qualcomm (more than 20 RBs) |
| - Whether there is a BW restriction for CSI reporting, i.e., wideband or subband? |  |
| No restriction | Samsung, ZTE, Sanechips, vivo, Huawei, HiSilicon, CATT |
| Restricted (details) |  |

Proposal 3:  In the case of A CSI-RS resource configured on RBs outside CORESET in CORESET symbols from UE perspective:

- There is no BW restriction for CSI reporting, i.e., wideband or subband

- Also can be applied for the case of CSI-RS and CORESET are NOT spatially QCL-ed

- There is no BW restriction for CSI-RS resource

- Issue 2-b: CSI-RS and CORESET (TDM)

If DMRS is from 3rd symbol and CORSET symbol is 1st, then CSI-RS can be in 2nd OFDM symbol.

Option 1: Agree:

Option 2: Do not agree:

| Companies | Views |
| --- | --- |
| Qualcomm | Option-1 |
| Samsung | We think this is a non-critical issue, which should be deprioritized in Reno taking into account the time schedule. |
| CATT | This is not an essential issue, shouldn’t be spending time discussing it. |
| Ericsson | We already have agreements that CSI-RS can be in the 1st, 2nd, and 3rd symbols (in addition to the other agreed symbol locations) |

Possible conclusions in this meeting:

Proposal 3:  In the case of A CSI-RS resource configured on RBs outside CORESET in CORESET symbols from UE perspective:

- There is no BW restriction for CSI reporting, i.e., wideband or subband

- Also can be applied for the case of CSI-RS and CORESET are NOT spatially QCL-ed

- There is no BW restriction for CSI-RS resource

- Agreements in previous meetings

In RAN1#91 meeting:

Agreement:

Proposal 2: Only support TDM between SRS and PUSCH/UL DMRS/UL PTRS/Long PUCCH in Rel-15 from UE perspective.

Agreement:

Symbol location for CSI-RS:

{0,1,2,3,4,5,6,7,8,9,10,11,12,13}, where 2 is supported only when DL-DMRS-typeA-pos equals 3

- UE is not expected to receive CSI-RS and DMRS on overlapping REs

Agreement:

UE does not expect any DMRS RE to collide with SSB REs on the 4 symbols occupied by SSB

Agreement

- A CSI-RS resource can be configured on RBs outside PBCH RBs in the symbols containing SS block from UE perspective.

- Above applies for the case where SS block and CSI-RS are spatially QCL-ed

- Note: CSI-RS BW discussion should be taken into account. If beam management is agreed, the requirement on minimum BW for CSI acquisition and beam management may be different.

- Above applies at least for the case where the same subcarrier spacing is used for SS block and CSI-RS

- Above applies for the cases: CSI-RS only used for beam management

In RAN1#90bis meeting:

Agreement: on page 7 in R1-1718998:

The PT-RS according to the mapping pattern is not transmitted in OFDM symbols that contains PDSCH/PUSCH DMRS

The PT-RS according to the mapping pattern is not transmitted in RE that overlaps with a configured CORESET

Agreement:

When SS block and PDSCH are scheduled in the same symbols, DMRS and SS block can be in a same symbol.

Above applies at least for the case where DMRS and SS block are not overlapping in the frequency domain

Above applies at least for the case where SS block and DMRS are spatially QCL-ed

Above applies at least for the case where the same subcarrier spacing is used for SS block and DMRS

Agreement:

Multiplexing schemes for TRS with DMRS/PDSCH/PDCCH/SS block follow the multiplexing schemes for CSI-RS with DMRS/PDSCH/PDCCH/SS block.

Agreement:

A CSI-RS resource can be configured on RBs outside CORESET in CORESET symbols from UE perspective.

Above applies at least for the case where PDCCH and CSI-RS are spatially QCL-ed, and FFS for multi-panel UEs

FFS: Use case when the above is applicable (ex: CSI reporting for wideband and partial band)

Note: CSI-RS BW discussion should be taken into account

Above at least applies for non-slot based cases

Above feature is supported for slot-based transmissions as well

Agreement:

A CSI-RS resource can be configured on RBs outside PBCH RBs in the symbols containing SS block from UE perspective.

Above applies at least for the case where SS block and CSI-RS are spatially QCL-ed, FFS for multi-panel UEs.

FFS: If non-QCLed, study UE’s behavior

Note: CSI-RS BW discussion should be taken into account. If beam management is agreed, the requirement on minimum BW for CSI acquisition and beam management may be different.

Above applies at least for the case where the same subcarrier spacing is used for SS block and CSI-RS

Down select following alternatives:

Alt.1 Above applies for the cases: CSI-RS used for beam management and CSI acquisition

Alt.2 Above applies for the cases: CSI-RS only used for CSI acquisition

Alt.3 Above applies for the cases: CSI-RS only used for beam management

Agreement:

In the case of collision of SRS and short PUCCH carrying only CSI report/beam failure recover request, support the prioritization rules in the table below:

The channel listed in the entries below are prioritized

|  | Aperiodic SRS | Semi-persistent SRS | periodic SRS |
| --- | --- | --- | --- |
| sPUCCH with aperiodic CSI report only | No rule** | sPUCCH | sPUCCH |
| sPUCCH with semi persistent CSI report only | SRS | sPUCCH | sPUCCH |
| sPUCCH with periodic CSI report only | SRS | sPUCCH | sPUCCH |
| sPUCCH with beam failure recover request* | sPUCCH | sPUCCH | sPUCCH |

In case SRS is dropped, dropping can be partial in time domain, i.e., only those OFDM symbols that collide with short PUCCH

*If short PUCCH is supported for beam failure recovery request and collision between short PUCCH with beam failure recovery request and aperiodic/semi persistent/periodic SRS occurs, prioritize short PUCCH

** UE can assume that this collision will not occur

In RAN1 NR Ad-Hoc#3 meeting:

Agreement:

- Down-select among the following two options

  - Option 1-1: From a UE perspective, CSI-RS is not multiplexed on SS block OFDM symbol(s)

  - Option 1-2: From a UE perspective, CSI-RS can be multiplexed on SS block symbol(s)

    - FFS on the conditions when multiplexing is allowed

- Down-select among the following two options

  - From a UE perspective, for REs that is in the same OFDM symbol of configured CORESET but outside of the CORESET,

    - Option 2-1: NZP CSI-RS can be multiplexed

      - FFS: Whether or not a UE needs to be aware of the CORESET of another UE, e.g., through configuration of a ZP-CSI-RS

      - FFS: Whether or not PBCH is affected for common CORESET configuration, e.g., through a configuration of ZP-CSI-RS

    - Option 2-2: NZP CSI-RS is not multiplexed

- Down-select among the following two options

  - Option 3-1: Within a BWP, a UE is not expected to be configured with NZP-CSI-RS in OFDM symbols for which it is configured to receive DMRS

    - E.g., from a UE perspective in the slot with scheduled PDSCH, CSI-RS can be transmitted on the potential additional DMRS OFDM symbol(s), when the additional DMRS does not exist in the OFDM symbol(s).

    - Note: In Option 3-1, CSI-RS is not multiplexed on the potential front-loaded DMRS OFDM symbol(s)

  - Option 3-2: From a UE perspective, CSI-RS can be multiplexed on all any potential DMRS OFDM symbol locations(s)

  - Note: Additional restriction from cell-/UE-group perspective will be further discussed based on the down-selection above

In RAN1#90 meeting:

Conclusion:

Regarding CSI-RS and TRS multiplexing/sharing, discuss after the TRS design is mature (target later this week)

Agreements:

- For UE’s perspective,

  - For REs that is configured as a CORESET,

    - NZP CSI-RS is not multiplexed.

  - For REs that is in the same OFDM symbol of configured CORESET but outside of the CORESET,

    - FFS: Whether NZP CSI-RS can be multiplexed or not

Agreements:

- NR supports FDM between DMRS and PDSCH for CP-OFDM at least for some cases

  - FFS the conditions for this feature

- NR supports FDM between DMRS and PUSCH for CP-OFDM at least for some cases

  - FFS the conditions for this feature

- NR supports signaling for PDSCH rate matching in DMRS symbols

  - FFS details

Agreements:

- When one or more of PT-RS RE(s) is overlapped with CSI-RS

  - The one or more overlapping PT-RS RE(s) is punctured

Agreements:

- For collision avoidance between short PUCCH and SRS, from a UE perspective, NR supports at least the following two options on a given carrier

  - Collision is defined whenever SRS and PUCCH are transmitted in the same symbol, regardless of whether there are overlapped REs or not

  - Option 1-1: symbol level TDM

  - (Working assumption) Option 2: Prioritize SRS or short PUCCH transmission, i.e., drop SRS or short PUCCH in case of collision

    - FFS whether to have one prioritization rule, or configurable prioritization

    - Examples of prioritization rules

      - Example 1

        - Always prioritize PUCCH over SRS

      - Example 2

        - If PUCCH contains ACK/NACK, prioritize PUCCH

        - Otherwise prioritize SRS

  - FFS the case of FDM SRS and short PUCCH

In RAN1 NR Ad-Hoc#2 meeting:

Agreements:

- In NR, for a given BWP, support the case where CSI-RS for CSI acquisition and beam management is always transmitted with the same numerology as the PDSCH of the UE if PDSCH is present

  - FFS the case when PDSCH is not present

Agreements:

- Down-select among the following two options in the next meeting

  - Option 1-1: From a UE perspective, CSI-RS is not multiplexed on SS block OFDM symbol(s)

  - Option 1-2: From a UE perspective, CSI-RS can be multiplexed on SS block symbol(s)

Agreements:

- Down-select among the following two options in the next meeting:

  - Option 2-1: From a UE perspective, CSI-RS is not multiplexed on PDCCH OFDM symbol(s) for a slot

  - Option 2-2: From a UE perspective, CSI-RS can be multiplexed on PDCCH OFDM symbol(s) for a slot

    - Note: PDCCH decoding behavior at UE side will not be changed by Option 2-2.

  - Note: up to each company to define PDCCH OFDM symbols based on the max possible number of symbols or the configured PDCCH OFDM symbols for the slot

Agreements:

- Study further aspects related to DMRS and data multiplexing in DL and UL considering 14 and 7 symbol slots/mini-slots, 1 vs. 2 front loaded DM-RS symbols, additional DM-RS, etc.

- Study further aspects related to possibly power boosting DM-RS (performance, complexity, spec impact)

Agreements:

- Study further how to handle DM-RS and SS block collision (if any)

  - E.g., changing DM-RS symbol(s) position in time domain, no PDSCH transmission on the collided PRBs, dropping the DM-RS symbol in collision, etc.

Agreements:

- Study further how to handle PT-RS collision with CSI-RS

- References

RAN1-89 Chairman Notes.

RAN1 NR-AH2 Chairman Notes.

RAN1-90 Chairman Notes.

RAN1 NR-AH3 Chairman Notes.

RAN1-90b Chairman Notes.

R1-1715447	Discussion on multiplexing of different RSs	ZTE, Sanechips

R1-1715470	Multiplexing different types of RSs for DL and UL	Huawei, HiSilicon

R1-1715511	On DL RS multiplexing	Spreadtrum Communications

R1-1715621	On multiplexing of different types of RSs	vivo

R1-1715705	Discussion on RS multiplexing	Guangdong OPPO Mobile Telecom.

R1-1715805	Consideration on RS multiplexing	CATT

R1-1715863	On multiplexing of different types of RSs	LG Electronics

R1-1715965	Discussion on DL/UL RS multiplexing	Samsung

R1-1716051	Discussion on multiplexing of different types of RSs	CMCC

R1-1716170	Multiplexing different type of RSs	AT&T

R1-1716227	On multiplexing of different reference signals	MediaTek Inc.

R1-1716298	On multiplexing of different RS types	Intel Corporation

R1-1716370	On Multiplexing of RS types	Ericsson

R1-1716404	On multiplexing of different types of RSs	Qualcomm Incorporated

R1-1716507	On multiplexing of different RS types	Nokia, Nokia Shanghai Bell

R1-1717304      Multiplexing different types of RSs for DL and UL, Huawei, HiSilicon

R1-1717371     On multiplexing of different RS types Intel Corporation

R1-1717431     On multiplexing of different RS types ZTE, Sanechips

R1-1717475     On multiplexing of different types of RSs vivo

R1-1717628     Discussion on DL/UL RS multiplexing Samsung

R1-1717744     Consideration on DL RS multiplexing Spreadtrum Communications

R1-1717816     Consideration on RS multiplexing CATT

R1-1717881     Discussion on multiplexing of different types of RSs CMCC

R1-1717944     On multiplexing of different types of RSs LG Electronics

R1-1718027     Discussion on RS multiplexing Guangdong OPPO Mobile Telecom

R1-1718350     On RS multiplexing MediaTek Inc.

R1-1718391     Multiplexing different type of RSs AT&T

R1-1718446     On multiplexing of RS types Ericsson

R1-1718514     On multiplexing of different RS types Nokia, Nokia Shanghai Bell

R1-1718545     On multiplexing of different types of RSs Qualcomm Incorporated

R1-1719437	Multiplexing RSs and other signals	Huawei, HiSilicon

R1-1719540	Remaining details on RS Multiplexing	ZTE, Sanechips

R1-1719635	Remaining details on Multiplexing of different types of RSs	AT&T

R1-1719691	Remaining issues on RS multiplexing	Spreadtrum Communications

R1-1719772	Remaining details on multiplexing of different types of RSs	vivo

R1-1719910	On multiplexing of different types of RSs	LG Electronics

R1-1720074	On multiplexing of DM-RS and SS block	Intel Corporation

R1-1720184	Remaining details on RS multiplexing	CATT

R1-1720309	Remaining details on DL/UL RS multiplexing	Samsung

R1-1720588	Discussion on multiplexing of different types of RSs	CMCC

R1-1720666	On multiplexing of different types of RSs	Qualcomm Incorporated

R1-1720738	On multiplexing of RS types	Ericsson