TSG-RAN WG1 #91	R1-1721719

Reno, NV, US, November 27 – December 1, 2017

Source:	Ericsson

Title:	Outcome of offline discussion on 7.3.3.1

Agenda Item:	7.3.3.1

Document for:	Discussion and Decision

# Default entries in the time-domain allocation table

Agreements:

- One table for UL, one table for DL configured by RRC in Rel-15

  - Each table is up to 16 rows

- In the table, each row is configured by RRC with

  - K0 using 2 bits (for DL table),  K2 using 3 bits (for UL table)

  - an index (6-bit) into a table/equation in RAN1 specs capturing valid combinations of start symbol and length (jointly encoded)

  - PDSCH mapping type A or B

- The reference point for starting OFDM symbol:

  - No RRC impact (e.g., slot boundary, start of CORESET where the PDCCH was found, or part of the table/equation in RAN1 specs. FFS details)

- Aggregation factor (1, 2, 4, 8 for DL or UL) is semi-statically configured separately (i.e. not part of table)

  - No additional RRC impact how to use the aggregation factor along with the tables

Discussion:

Example DL table

| Index | K0 | Resource index | PDSCH mapping type |
| --- | --- | --- | --- |
| 0 | 0 | <DL symbol 2-11> | A |

Example UL table

| Index | K2 | Resource index |
| --- | --- | --- |
| 0 | 1 | <UL symbol 2-11> |

Open issues:

- How to encode the resource index?

Offline proposal: Encode OFDM symbol start and length into the resource index RIV according to

L = length

S=start

if (L-1)<7 then

RIV=14(L-1)+S

else

RIV=14(14-L+1)+(14-1-S)

- Reference point for time-domain information?

# RBG sizes

From previous offline session:

- Wishlist:

  - Fill only one column as starting point

  - Select values to get bit sizes similar to LTE (for the same bandwidth)

  - Select switch points to reasonably well match RAN4 spectrum allocations at different subcarrier spacings

  - Alignment between subband sizes in the MIMO session and RBG sizes is beneficial

From MIMO session:

- Agreement:

- Refine subband sizes as follows:

| Carrier bandwidth part (PRBs) | Subband Size (PRBs) |
| --- | --- |
| < 24 | N/A |
| 24 – 72 | 4,8 |
| 73 – 144 | 8,16 |
| 145 – 275 | 16,32 |

Discussion:

- If the MIMO session is taken as starting point

  - With change of 23 to 24 based on the LS from RAN4

Offline proposal: Working assumption is the table below.

| BWP size | RBG size, config 1 | RBG size, config 2 |
| --- | --- | --- |
| - 24 | 2 |  |
| 25 – 72 | 4 |  |
| 73 – 144 | 8 |  |
| 145 – 275 | 16 |  |

Note: Config 2 may use different switch points.