3GPP TSG RAN WG1 Meeting 91	R1-1721726 Attachment

RENO, USA, NOV 27th  –  DEC 1st, 2017

Source:          	CATT

Title:  	Working Assumptions for RMSI CORESETs

Agenda Item:     	7.1.2.2

Document for:   	Discussion and Decision

# Introduction

The document provides the working assumption on RMSI CORESET and RMSI timing configuration agreed in RAN1#91.

# Working Assumptions for RMSI CORESETs

- The frequency offset in PRB level between RMSI CORESET and SS/PBCH block in the table is define as the frequency difference from the lowest PRB of RMSI to the lowest PRB of SS/PBCH block

  - Note: The offset in subcarrier level between the edge of SS/PBCH block PRB and RMSI CORESET PRB grid is indicated by PRB grid offset (SSB-subcarrier-offset) in PBCH (5 bits for below-6GHz and 4 bits for above-6GHz)

- For each of the following combination of SS/PBCH SCS and RMSI CORESET SCS, the multiplexing patterns between SS/PBCH block and RMSI (CORESET and PDSCH), RMSI CORESET BW, RMSI CORESET duration, and PRB-level offset are jointly coded in a table using 4 bits of RMSI configuration in NR-PBCH.

  - {SSB SCS, RMSI SCS} = {15, 15}, {15, 30}, {30, 15}, {30, 30}, {120, 60}, {120,120},{240, 60}, {240, 120}} kHz

- Definition for the multiplexing pattern tables SS/PBCH SCS and RMSI CORESET SCS

  - “Pattern 1” refers to the multiplexing pattern that SS/PBCH block and RMSI CORESET occur in different time instances, and SS/PBCH block TX BW and the initial active DL BWP containing RMSI CORESET overlap

  - “Pattern 2” refers to the multiplexing pattern that SS/PBCH block and RMSI CORESET occur in different time instances, and SS/PBCH block TX BW and the initial active DL BWP containing RMSI CORESET do not overlap

  - “Pattern 3” refers to the multiplexing pattern that SS/PBCH block and RMSI CORESET occur in the same time instance, and SS/PBCH block TX BW and the initial active DL BWP containing RMSI CORESET do not overlap

  - Note: The following figure is for information purpose only.

Table 1 {SSB SCS, RMSI SCS} = {15, 15}kHz

| Configuration Index | Multiplexing Pattern | CORESET BW in PRB | Number of Symbols for CORESET | Frequency Offset in PRB of RMSI CORESET numerology |
| --- | --- | --- | --- | --- |
| 1 | Pattern 1 | 24 | 2 | 0 |
| 2 | Pattern 1 | 24 | 2 | 2 |
| 3 | Pattern 1 | 24 | 2 | 4 |
| 4 | Pattern 1 | 24 | 3 | 0 |
| 5 | Pattern 1 | 24 | 3 | 2 |
| 6 | Pattern 1 | 24 | 3 | 4 |
| 7 | Pattern 1 | 48 | 1 | 12 |
| 8 | Pattern 1 | 48 | 1 | 16 |
| 9 | Pattern 1 | 48 | 2 | 12 |
| 10 | Pattern 1 | 48 | 2 | 16 |
| 11 | Pattern 1 | 48 | 3 | 12 |
| 12 | Pattern 1 | 48 | 3 | 16 |
| 13 | Pattern 1 | 96 | 1 | 38 |
| 14 | Pattern 1 | 96 | 2 | 38 |
| 15 | Pattern 1 | 96 | 3 | 38 |
| 16 | Reserved |  |  |  |

Table 2 {SSB SCS, RMSI SCS} = {15, 30}kHz

| Configuration Index | Multiplexing Pattern | CORESET BW in PRB | Number of Symbols for CORESET | Frequency Offset in PRB of RMSI CORESET numerology |
| --- | --- | --- | --- | --- |
| 1 | Pattern 1 | 24 | 2 | 6 |
| 2 | Pattern 1 | 24 | 2 | 7 |
| 3 | Pattern 1 | 24 | 2 | 8 |
| 4 | Pattern 1 | 24 | 3 | 6 |
| 5 | Pattern 1 | 24 | 3 | 7 |
| 6 | Pattern 1 | 24 | 3 | 8 |
| 7 | Pattern 1 | 48 | 1 | 18 |
| 8 | Pattern 1 | 48 | 1 | 20 |
| 9 | Pattern 1 | 48 | 2 | 18 |
| 10 | Pattern 1 | 48 | 2 | 20 |
| 11 | Pattern 1 | 48 | 3 | 18 |
| 12 | Pattern 1 | 48 | 3 | 20 |
| 13 | Reserved |  |  |  |
| 14 | Reserved |  |  |  |
| 15 | Reserved |  |  |  |
| 16 | Reserved |  |  |  |

Table 3 {SSB SCS, RMSI SCS} = {30, 15}kHz

| Configuration Index | Multiplexing Pattern | CORESET BW in PRB | Number of Symbols for CORESET | Frequency Offset in PRB of RMSI CORESET numerology |
| --- | --- | --- | --- | --- |
| 1 | Pattern 1 | 48 | 1 | 2 |
| 2 | Pattern 1 | 48 | 1 | 6 |
| 3 | Pattern 1 | 48 | 2 | 2 |
| 4 | Pattern 1 | 48 | 2 | 6 |
| 5 | Pattern 1 | 48 | 3 | 2 |
| 6 | Pattern 1 | 48 | 3 | 6 |
| 7 | Pattern 1 | 96 | 1 | 28 |
| 8 | Pattern 1 | 96 | 2 | 28 |
| 9 | Pattern 1 | 96 | 3 | 28 |
| 10 | Reserved |  |  |  |
| 11 | Reserved |  |  |  |
| 12 | Reserved |  |  |  |
| 13 | Reserved |  |  |  |
| 14 | Reserved |  |  |  |
| 15 | Reserved |  |  |  |
| 16 | Reserved |  |  |  |

Table 4 {SSB SCS, RMSI SCS} = {30, 30}kHz

| Configuration Index | Multiplexing Pattern | CORESET BW in PRB | Number of Symbols for CORESET | Frequency Offset in PRB of RMSI CORESET numerology |
| --- | --- | --- | --- | --- |
| 1 | Pattern 1 | 24 | 2 | 0 |
| 2 | Pattern 1 | 24 | 2 | 1 |
| 3 | Pattern 1 | 24 | 2 | 2 |
| 4 | Pattern 1 | 24 | 2 | 3 |
| 5 | Pattern 1 | 24 | 2 | 4 |
| 6 | Pattern 1 | 24 | 3 | 0 |
| 7 | Pattern 1 | 24 | 3 | 1 |
| 8 | Pattern 1 | 24 | 3 | 2 |
| 9 | Pattern 1 | 24 | 3 | 3 |
| 10 | Pattern 1 | 24 | 3 | 4 |
| 11 | Pattern 1 | 48 | 1 | 12 |
| 12 | Pattern 1 | 48 | 1 | 14 |
| 13 | Pattern 1 | 48 | 1 | 16 |
| 14 | Pattern 1 | 48 | 2 | 12 |
| 15 | Pattern 1 | 48 | 2 | 14 |
| 16 | Pattern 1 | 48 | 2 | 16 |

Table 5 {SSB SCS, RMSI SCS} = {120, 60}kHz

| Configuration Index | Multiplexing Pattern | CORESET BW in PRB | Number of Symbols for CORESET | Frequency Offset in PRB of RMSI CORESET numerology |
| --- | --- | --- | --- | --- |
| 1 | Pattern 1 | 48 | 1 | 0 |
| 2 | Pattern 1 | 48 | 1 | 8 |
| 3 | Pattern 1 | 48 | 2 | 0 |
| 4 | Pattern 1 | 48 | 2 | 8 |
| 5 | Pattern 1 | 48 | 3 | 0 |
| 6 | Pattern 1 | 48 | 3 | 8 |
| 7 | Pattern 1 | 96 | 1 | 28 |
| 8 | Pattern 1 | 96 | 2 | 28 |
| 9 | Pattern 2 | 48 | 1 | [-42] if PRG grid not aligned and [-41] if PRG grid aligned |
| 10 | Pattern 2 | 48 | 1 | [49] |
| 11 | Pattern 2 | 48 | [2] | [-42] if PRG grid not aligned and [-41] if PRG grid aligned |
| 12 | Pattern 2 | 48 | [2] | [49] |
| 13 | Pattern 2 | [96] | 1 | [-42] if PRG grid not aligned and [-41] if PRG grid aligned |
| 14 | Pattern 2 | [96] | 1 | [97] |
| 15 | Pattern 2 | [96] | [2] | [-42] if PRG grid not aligned and [-41] if PRG grid aligned |
| 16 | Pattern 2 | [96] | [2] | [97] |

Note: Configurations 13, 14, 15, and 16 are supported only when carrier bandwidth is larger than 100MHz

Table 6 {SSB SCS, RMSI SCS} = {120, 120}kHz

| Configuration Index | Multiplexing Pattern | CORESET BW in PRB | Number of Symbols for CORESET | Frequency Offset in PRB of RMSI CORESET numerology |
| --- | --- | --- | --- | --- |
| 1 | Pattern 1 | 24 | 2 | 0 |
| 2 | Pattern 1 | 24 | 2 | 4 |
| 3 | Pattern 1 | 48 | 1 | 14 |
| 4 | Pattern 1 | 48 | 2 | 14 |
| 5 | Pattern 3 | 24 | 2 | [-21] if PRG grid not aligned and [-20] if PRG grid aligned |
| 6 | Pattern 3 | 24 | 2 | 24 |
| 7 | Pattern 3 | [48] | 2 | [-21] if PRG grid not aligned and [-20] if PRG grid aligned |
| 8 | Pattern 3 | [48] | 2 | [48] |
| 9 | Reserved |  |  |  |
| 10 | Reserved |  |  |  |
| 11 | Reserved |  |  |  |
| 12 | Reserved |  |  |  |
| 13 | Reserved |  |  |  |
| 14 | Reserved |  |  |  |
| 15 | Reserved |  |  |  |
| 16 | Reserved |  |  |  |

Note: Configurations 7 and 8 are supported only when carrier bandwidth is larger than 100MHz

Table 7 {SSB SCS, RMSI SCS} = {240, 60}kHz

| Configuration Index | Multiplexing Pattern | CORESET BW in PRB | Number of Symbols for CORESET | Frequency Offset in PRB of RMSI CORESET numerology |
| --- | --- | --- | --- | --- |
| 1 | Pattern 1 | 96 | 1 | 0 |
| 2 | Pattern 1 | 96 | 1 | 16 |
| 3 | Pattern 1 | 96 | 2 | 0 |
| 4 | Pattern 1 | 96 | 2 | 16 |
| 5 | Reserved |  |  |  |
| 6 | Reserved |  |  |  |
| 7 | Reserved |  |  |  |
| 8 | Reserved |  |  |  |
| 9 | Reserved |  |  |  |
| 10 | Reserved |  |  |  |
| 11 | Reserved |  |  |  |
| 12 | Reserved |  |  |  |
| 13 | Reserved |  |  |  |
| 14 | Reserved |  |  |  |
| 15 | Reserved |  |  |  |
| 16 | Reserved |  |  |  |

Table 8 {SSB SCS, RMSI SCS} = {240, 120}kHz

| Configuration Index | Multiplexing Pattern | CORESET BW in PRB | Number of Symbols for CORESET | Frequency Offset in PRB of RMSI CORESET numerology |
| --- | --- | --- | --- | --- |
| 1 | Pattern 1 | 48 | 1 | 0 |
| 2 | Pattern 1 | 48 | 1 | 8 |
| 3 | Pattern 1 | 48 | 2 | 0 |
| 4 | Pattern 1 | 48 | 2 | 8 |
| 5 | Pattern 2 | 24 | 1 | [-42] if PRG grid not aligned and [-41] if PRG grid aligned |
| 6 | Pattern 2 | 24 | 1 | [25] |
| 7 | Pattern 2 | 24 | [2] | [-42] if PRG grid not aligned and [-41] if PRG grid aligned |
| 8 | Pattern 2 | 24 | [2] | [25] |
| 9 | Pattern 2 | [48] | 1 | [-42] if PRG grid not aligned and [-41] if PRG grid aligned |
| 10 | Pattern 2 | [48] | 1 | [49] |
| 11 | Pattern 2 | [48] | [2] | [-42] if PRG grid not aligned and [-41] if PRG grid aligned |
| 12 | Pattern 2 | [48] | [2] | [49] |
| 13 | Reserved |  |  |  |
| 14 | Reserved |  |  |  |
| 15 | Reserved |  |  |  |
| 16 | Reserved |  |  |  |

Note: Configurations 9, 10, 11, and 12 are supported only when carrier bandwidth is larger than 100MHz