3GPP TSG RAN WG1 #92																				R1-1802633

Athens, Greece, February 26th – March 2nd, 2018

Agenda item:	    7.4.1

Source: 	InterDigital Inc.

Title:	On IDMA Transmitter Processing

Document for:	Discussion

# Introduction

NOMA studies in 3GPP started with the Rel. 14 NOMA SI where different schemes were investigated. Leading up to RAN1 #86b meeting, preliminary NOMA simulations were performed and the schemes were shown to provide significant benefits in throughput, overloading, and handling large packet arrival rates. The results were summarized in [1] and captured in TR38.802 along with the simulation assumptions. At RAN #75, a new Study Item on NOMA was approved and further revised at RAN #76 [2].

In this contribution, we discuss implementation aspects and some details related to the transmitter signal processing of Interleave Division Multiple Access (IDMA).

# NOMA Schemes

Given the importance of NOMA, a wide range of studies were conducted during Rel-14 SI to identify the best solution for use in NR. Thus far, at least 16 different transmission mechanisms have been identified for potential consideration in NR. Table 1 shows a catalogue of the proposed schemes. The proposed schemes can be categorized from different angles. However, at a high level, the main principle for user discrimination and detection for nearly all the proposed schemes can be considered as a special form of spreading. The spreading mechanisms could be linear or non-linear, and its length could be short or long [3-5].

| Table 1 – Proposed NOMA Schemes |  |  |
| --- | --- | --- |
| Scheme | Name | Proposer |
| PDMA | Pattern division multiple access | CATT |
| SCMA | Sparse code multiple access | Huawei |
| MUSA | Multi-user shared access | ZTE |
| LCRS | Low code rate spreading | Intel |
| FDS | Frequency domain spreading |  |
| NCMA | Non-orthogonal coded multiple access | LG |
| NOMA | Non-orthogonal multiple access (power) | NTT Docomo |
| RSMA | Resource spreading multiple access | Qualcomm |
| IGMA | Interleave-grid multiple access | Samsung |
| LDS-SVE | Low density spreading with signature vector extension | Fujitsu |
| LSSA | Low code rate and signature based shared access | ETRI |
| NOCA | Non-orthogonal coded access | Nokia & ALU |
| IDMA | Interleave division multiple access | Nokia & ALU, InterDigital |
| RDMA | Repetition division multiple access | MTK |
| GOCA | Group orthogonal coded access |  |
| WSMA | Welch sequence multiple access | Ericsson |

In evaluation and selection of NOMA schemes, it is crucial to consider the actual use case, and the expected performance of the system. Most of the proposed schemes have demonstrated very reasonable multiplexing capabilities, however often they were not set up to achieve a specific target BLER or overloading capability. Therefore, it is important to study and evaluate all proposed schemes in a balanced and meaningful way.

# IDMA Transmitter Processing

Earlier studies on spread spectrum multiple access systems indicate that the optimal capacity of a multiple access channel can be achieved if the spreading is entirely realized by employing a very low rate error correction code per user [6]. As such, the capacity of a NOMA system can be optimized by proper design of the spreading mechanism for a given number of users. Under the paradigm of employing very low code rate, users may be perceived using orthogonal code, and at the same time take advantage of a strong error control mechanism for robust detection. Having said that, per user definition of very low rate codes is not straightforward, and in fact a very challenging task in terms of error correcting code design. Besides, due to extra receiver complexity, it would not be an attractive solution from implementation perspective. Therefore, a common way for such realization is by combining a very low rate error correcting code with user-specific interleaving and/or scrambling.

IDMA is considered the main representative of very low code rate NOMA schemes [7]. In fact, IDMA can be viewed as a CDMA system where the spreading mechanism is a function of the actual payload. The main feature of IDMA principle lies in that it does not treat the observed interference as an additive noise. During the detection process, the a-priori LLR’s are continually enhanced by updating relevant statistics of the received signal and the interference. More details on the receiver implementation of IDMA are provided in our companion contribution [9].There have been some basic evaluations of IDMA in the literature [7-8], however IDMA did not receive sufficient interest for implementation and adoption during 3G and 4G developments. Given the new NR use cases as well as more demanding spectral efficiency requirements, very low code rate NOMA schemes such as IDMA seem very relevant.

Figure 1 shows the basic structure of an IDMA-based multiple access system with K number of active users. At the transmitter, each UE payload is coded and then randomized prior to transmission in the shared channel. The randomization operation is user-specific, and it can be implemented through a combination of interleaving/scrambling functions.

Figure 1 A conceptual model of IDMA transmission

Figure 2 exhibits a more detailed presentation of the IDMA transmitter processing. To allow more flexibility in design, as well as in selection of desired Spectrum Efficiency (SE), a relatively low code rate encoder combined with a repetition mechanism (R) is employed. As such, it would be straight forward to vary the spreading rate (R) to achieve a desired SE without committing to any change in the hardware. In other words, in an exemplary implementation, a same error correction mechanism can be used for all use cases. Then, the repetition rate R can be reduced or increased to achieve a lower or higher capacity, and/or a better or worse target BLER per requirement. Hence, we could potentially accommodate different use cases of NOMA with a same transmission mechanism and hardware.

The coded stream ci is interleaved according to its designated interleaving function to facilitate user separation at the receive end. To reduce the impact of intercell interference, a cell based randomization may also be employed that will be common for all configured NOMA UEs in the cell.

As demonstrated in Figure 2, an IDMA transmitter operates with a standard OFDM modulator as defined in NR. As such, there is no impact on the waveform, alleviating any potential concern about PAPR and PA non-linearity behaviour.

Based on the discussion and provided details on an IDMA transmitter, following observations can be made;

Observation 1: The transmitter of an IDMA receiver does not require much extra complexity and can be easily integrated with the existing NR transmitter signal chain.

Observation 2: An IDMA transmitter can be easily reconfigured to enable operation in various SE and required target BLER.

Observation 3: An IDMA transmitter employs the existing modulation formats for its transmission, as its transmission does not require any manipulation of the transmit symbols. As a result, any impact on PAPR is not expected.

Figure 2 Detail presentation of an IDMA transmitter

# Conclusions

In this contribution, we have provided further details and description of an IDMA transmitter. Based on the discussion, the following observations are made:

Observation 1: The transmitter of an IDMA receiver does not require much extra complexity and can be easily integrated with the existing NR transmitter signal chain.

Observation 2: An IDMA transmitter employs the existing modulation formats for its transmission, as its transmission does not require any manipulation of the transmit symbols. As a result, any impact on PAPR is not expected.

Observation 3: An IDMA transmitter can be easily reconfigured to enable operation in various SE and required target BLER.

# References

- R1-1611251, “Summary of simulation results for NR MA”, Huawei, HiSilicon, 3GPP TSG RAN WG1 Meeting #87, November 2016, Lisbon, Portugal

- RP-171043, “Revision of Study on 5G Non-Orthogonal Multiple Access”, RAN#76, West Palm Beach, USA, June 5-8, 2017.

- R1‑1608852, “Categorization and analysis of MA schemes,” 	Huawei, HiSilicon, 3GPP TSG RAN WG1 Meeting #86, October 2016, Lisbon, Portugal

- R1-165018, “Overview of the proposed non-orthogonal MA schemes,” Nokia/Alcatel-Lucent, 3GPP TSG RAN WG1 Meeting #85, May 2016 Nanjing, China

- R1-167445, “Classification of candidate UL non-orthogonal MA schemes,” China Telecom, 3GPP TSG RAN WG1 Meeting #86, August 2016, Gothenburg, Sweden

- A. J. Viterbi, “Very low rate convolutional codes for maximum theoretical performance of spread spectrum multiple-access channels,” IEEE Selected Areas in Communications, Vol. 8, August 1999

- L. Ping, L. Liu, K. Wu and W.K. Leung, “Interleave-Division Multiple-Access,” IEEE Transaction on Wireless Communications, Vol. 5, April 2006

- Y. Chen, F. Schaich and T. Wild, “Multiple Access and Waveforms for 5G: IDMA and Universal Filtered Multi-Carrier,” IEEE 79th Vehicular Technology Conference (VTC Spring), 2014, Seoul, South Korea.

- R1-1802634, “IDMA Receiver Processing”, InterDigital Inc., 3GPP TSG RAN WG1 Meeting #92, February 2018, Athens, Greece.