3GPP TSG RAN WG1 Meeting #92			R1-1801307

Athens, Greece, February 26th – March 2nd, 2018

3GPP TSG RAN WG2 Meeting #100	R2-1714015

Reno, USA, 27th Nov – 1st Dec 2017

Title:	Reply LS on System acquisition time reduction for Rel-15 LTE MTC

Response to:	R1-1709474

Release:	Rel-15

Work Item:	LTE_eMTC4-Core

Source:	RAN2

To:	RAN1, RAN4

Cc:

Contact Person:

#### Name:	Brian Martin

Tel. Number:

E-mail Address:	brian(dot)alexander(dot)martin(at)huawei.com

1. Overall Description:

RAN2 thanks RAN1 on their LS and informs RAN1 and RAN4 of the following:

RAN2 considers that most of the enhancements listed in the RAN1 LS are primarily within the scope of RAN1 and/or RAN4 and not in the scope of RAN2 and therefore have no feedback at this time.

For the enhancements in the scope of RAN2, the following has been agreed:

Accumulation across SIB1-BR modification periods:

- RAN2 confirms that SIB1-BR accumulation across SIB1-BR modification period is possible since it has been introduced in Rel-13, and it is left to UE implementation.

Accumulation across SI modification periods:

- RAN2 confirms that SI message accumulation across SI windows and modification period is possible since it has been introduced in Rel-13, and it is left to UE implementation.

New mechanism allowing to skip SIB1-BR reading:

- Indicate in MIB whether there has been or not a system information change for a certain period of time. It is FFS how such period of time is provided.

- The indication in MIB is provided with 1 bit.

- The indication for EAB is FFS.

2. Actions:

To RAN1 and RAN4:

ACTION: 	RAN2 respectfully asks RAN1 and RAN4 to take into account the RAN2 agreements.

3. Date of Next TSG-RAN2 Meetings:

TSG-RAN2 Meeting #101	26 Feb - 2 Mar 2018		Athens, Greece

TSG-RAN2 Meeting #101bis	16 - 20 Apr 2018		China