3GPP TSG RAN WG1 Meeting #92			R1-1801315

Athens, Greece, February 26th – March 2nd, 2018

3GPP TSG-RAN WG4 Meeting #85	R4-1714417

- Reno, US, 27 November - 1 December 2017

Title:	LS on transmission timing adjustments in sTTI

Reply to:

Release:	Rel-15

Work Item:	LTE_sTTIandPT-Core

Source:	RAN WG4

To:	RAN WG1

Cc:

Contact Persons:

#### Name:	Han Jing

#### E-mail Address:	hw.hanjing@huawei.com

Attachments:

1. Overall Description:

RAN4 has discussed the transmission timing adjustment delay in sTTI and sPT.  RAN4 agreed that when 1ms TTI and 3 subframe HARQ processing or sTTI is used and the TA command is received at subframe/slot/subslot n, the timing advance adjustment delay is shown in Table1. The UE shall adjust the uplink timing at the first subframe boundary following the time shown in Table 1.

Table 1 : Timing advance adjustment delay requirement

| sTTI/processing time reduction scheme | Requirement to update timing | Units |
| --- | --- | --- |
| 1ms TTI with 3 subframe HARQ processing | n+5 | Subframe |
| Slot TTI | [n+8] | Slot |
| Subslot TTI with 4 subslot HARQ processing | [n+16] | Subslot |
| Subslot TTI with 6 subslot HARQ processing | [n+18] | Subslot |
| Subslot TTI with 8 subslot HARQ processing | [n+20] | Subslot |

2. Actions:

RAN4 respectfully asks RAN1 to take the above agreements into consideration.

3. Date of Next TSG RAN WG4 Meetings:

TSG RAN WG4 Meeting #86	26 Feb - 2 Mar 2018			Athens , GR

TSG RAN WG4 Meeting #86bis	16 - 20 Apr 2018   			Melbourne , AU