3GPP TSG RAN WG1 Meeting #92bis                                      R1-1803802

Sanya, China, April 16th – 20th, 2018

Source:				ZTE, Sanechips

Title:						Discussion on PDCCH repetition for URLLC

Agenda item:		7.2.3

Document for:	Discussion and Decision

# Introduction

As described in [1], a general URLLC reliability requirement for one transmission of a packet is 1-10-5 for 32 bytes with a user plane latency of 1ms.

The repetition in slot level for eMBB services has been supported. However, the mechanism is mainly targeted for extreme large coverage, and only the data transmission is repeated. In URLLC, ultra reliability including DL scheduling reliability is also to be taken into account. Therefore, PDCCH repetition is proposed to be studied in RAN plenary #78 meeting [2].

In this contribution, we discuss the repetition of the DL control channel, with focus on the same or multiple PDCCH monitoring occasion(s) in the same or multiple Search Spaces/CORESETs.

# PDCCH repetition

## Necessity of PDCCH repetition

Figure 1 Performance of PDCCH repetition

Potential methods for improving reliability include higher aggregation levels, compact DCI and PDCCH repetition, which may be used independently or together with each other. As observed in [3], about 0.5dB gain can be obtained by a compact DCI with 10 bits payload reduction from a normal DCI size of 40bits. Whether higher aggregation levels can be employed depends on availability of the resources. CCE occupies 6 PRBs in one symbol. There are not enough resource for using AL16 (Aggregation Levels = 16 CCE) in some cases, e.g.1-OS CORESET in10MHz BWP with 15 kHz SCS or 3-OS CORESET in 20 MHz BWP with 60 kHz SCS. In other words, AL8 would be more practical and desirable in some cases. However, in companion contribution [3], we find that the 1e-5 reliability cannot be fulfilled by AL8 with compact DCI even at a reference SINR -2dB. Considering the relatively smaller gain of the compact DCI, PDCCH repetition seems an indispensable solution. In addition, using repetition can avoid PDCCH blocking, which would impact latency.

As shown in Figure 1, about 3dB can be obtained by a 2-times repetition compared with a single transmission from BLER of 1e-2 to 1e-5. Simulation assumptions are provided in Annex.

Proposal 1: PDCCH repetition shall be supported to improve reliability of PDCCH for URLLC services.

## Implementation of PDCCH repetition

Table-1 below lists some possible cases for PDCCH repetition within the same or across multiple PDCCH monitoring occasion(s) of the same or multiple Search Spaces (SS)/CORESETs. Cases 1-3 can be classified as repetition in the frequency domain, while Cases 4-6 can be seen as repetition in the time domain or time + frequency domain.

Table 1 - Possible cases for PDCCH repetition

| Possible cases | Same or multiple PDCCH monitoring occasion(s) of the same or multiple Search Spaces/CORESETs | Scheme of repetition | Pros and Cons Pros and cons |
| --- | --- | --- | --- |
| Case 1 | Same occasion Same CORESET Same SS | Repetition among different candidates of one aggregation level | Low latency; Less scheduling flexibility; Equal to larger aggregation level. |
| Case 2 | Same occasion Same CORESET Multiple SSs | Repetition among multiple SSs with one or more candidates of one aggregation level used in each SS | Low latency; A little better than Case1 on scheduling flexibility. SSs have to be associated with the same CORESET |
| Case 3 | Same occasion Multiple CORESETs Multiple SSs | Repetition among multiple SSs within more than one CORESET. The candidates used belong to one aggregation level. | Low latency; Better than Case2 on scheduling flexibility |
| Case 4 | Multiple occasions Same CORESET Same SS | Repetition among multiple occasions configured to same SS. Multiple starting symbols are configured for the SS. Using same candidate and aggregation level among multiple occasions. | More latency than Case 1/2/3. Better than Case3 on scheduling flexibility. |
| Case 5 | Multiple occasions Same CORESET Multiple SSs | Repetition among multiple occasions configured to multiple SSs. One or more start symbols are configured for each SS and all the SSs belong to one CORESET. Using same candidate and aggregation level among Multiple occasions. | More latency than Case 1/2/3. Better than Case4 on scheduling flexibility. SSs have to be associated with same CORESET |
| Case 6 | Multiple occasions Multiple CORESETs Multiple SSs | Repetition among multiple occasions configured to multiple SSs. One or more start symbols are configured for each SS and the multiple SSs belong to multiple CORESETs. Using same candidate and aggregation level among Multiple occasions. | More latency than Case 1/2/3. Better than Case5 on scheduling flexibility. |

From the low latency point of view, Case 1-3 are better since the PDCCH repetition can be finished in one occasion, and the remaining time can be used for data repetition and HARQ timing. But this is not beneficial for multiplexing different UEs simultaneously, i.e. less scheduling flexibility. And, it is difficult to use frequency repetition for the UE without the capability of a wide-BWP. For a UE with the wide-BWP capability, Case 3 is better because SS blocking is less serious than for Case 1 and 2.

For Case 4-6, if each transmission of repetition is only in one occasion, then this equals to traditional repetition in the time domain. We could take Case 4 as an example. If the duration of the CORESET is 1 symbol and the starting symbols for the SS are configured with consecutive symbols, then repetition can be performed symbol by symbol. This can also achieve a low latency scheme and at the same time multiple users can be more easily multiplexed in the frequency domain. Case 5 and 6 are more complex than Case 4 which can keep the independency between SS and CORESET.

Overall, in our view, Case 3 and Case 4 from Table 1 are the most suited schemes to perform PDCCH repetition.

Proposal 2: At least one of two following schemes for PDCCH repetition is supported.

- PDCCH repetition within the same PDCCH monitoring occasion across multiple Search Spaces on different CORESETs.

- PDCCH repetition across multiple PDCCH monitoring occasions of the same Search Space and CORESET.

## DL transmission in case of PDCCH repetition

DL transmission with both PDCCH repetition and PDSCH repetition can be achieved by 2 possible alternatives. Alt.1: PDSCH repetition can only starts at the end of PDCCH repetition, which is shown in Figure 2. As for Alt 1a, PDCCH repetition only occurs in the time domain and can be supported by configuring multiple first symbols of the CORESET. Both time and frequency domain are used for PDCCH repetition in Alt.1b and this may be used in case the time resource for a CORESET is not very sufficient.

Alt.2: PDCCH repetition and PDSCH repetition start simultaneously, which is elaborated in Figure 3. For Alt.2, repetition for PDCCH and repetition for PDSCH can be started at the same TTI (e.g. 2OS mini-slot). When ACK is received by gNB, both PDCCH repetition and PDSCH repetition are terminated. When NACK is received by gNB, PDCCH repetition is terminated and PDSCH repetition is continued until ACK is received by gNB.

Figure 2 PDSCH repetition starts at the end of PDCCH repetition

Figure 3 PDCCH repetition and PDSCH repetition start simultaneously

Alt 1 can save processing resource for UE, as it only starts decoding PDSCH after received all the PDCCHs. However, it would introduce too much latency. This can easily exceed the 1ms boundary, since most relevant case is low frequency band with 15~60 kHz SCSs. Considering the processing resource is not limited for URRLC traffic, which is not for extremely high throughput, Alt.2 is slightly preferred.

Proposal 3: It is supported that repetition for PDCCH and repetition for PDSCH start simultaneously in time for URLLC.

Overall, if any starting symbol/occasion is supported in case of PDCCH repetition, the complexity of UE processing will be increased significantly. Thus, starting symbol/occasion should be limited for PDCCH repetition. Although this would lead to additional latency, a small number of repetition such as 2 or 3 can be used. In such case, the 1ms latency can be also achieved with alt.2. Note the repetition is the only way to achieve URLLC in case of SCS = 15 kHz, while HARQ based retransmission can be used for larger SCS. As a result, repetition within 1ms can be used for URLLC with all potential SCS, including SCS=15 kHz.

Proposal 4: Considering the different scenarios and performance benefit, all potential SCS can be used for URLLC.

# Conclusion

According to the analysis given above, we have the following proposals:

Proposal 1: PDCCH repetition shall be supported to improve reliability of PDCCH for URLLC services.

Proposal 2: At least one of two following schemes for PDCCH repetition is supported.

- PDCCH repetition within the same PDCCH monitoring occasion across multiple Search Spaces on different CORESETs.

- PDCCH repetition across multiple PDCCH monitoring occasions of the same Search Space and CORESET.

Proposal 3: It is supported that repetition for PDCCH and repetition for PDSCH start simultaneously in time for URLLC.

Proposal 4: Considering the different scenarios and performance benefit, all potential SCS can be used for URLLC.

# References

- 3GPP TR38.913 V14.3.0 Study on Scenarios and Requirements for Next Generation Access Technologies.

- 3GPP TSG RAN#79 Meeting,RP-172817, NR High-Reliability URLLC scope for RAN1/RAN2, Ericsson.

- 3GPP, R1-1803801,NR compact DCI format for URLLC, ZTE, Sanechips

# Annex

Table A-1Simulation Assumptions

| Parameter | Value |
| --- | --- |
| DCI payload (excluding 24bits CRC) | 40bits |
| System bandwidth | 20MHz |
| Carrier Frequency | 700MHz |
| Number of symbols for CORESET | 1 |
| CORESET BW (contiguous PRB allocation) | 20MHz |
| Subcarrier spacing | 30 kHz, 15 kHz |
| Aggregation level | 8 |
| Transmission type | Interleaved(R=3 for 3OS,others,R=2) |
| REG bundling size | 6 |
| Modulation | QPSK |
| Channel coding | Polar code (DCI) |
| Transmission scheme | 1-port precoder cycling |
| Channel estimation | Realistic |
| Channel model | TDL-C (delay spread: 300ns) |
| UE speed | 3 km/h |
| Number of BS antennas | 2Tx |
| Number of UE antennas | 4Rx for 4G, 2Rx for 700MHz |
| Residual target BLER | 10^-5 |
| Deployment | Urban macro as listed in 3GPP 38.802 |
| SINR target | PDCCH Repetition study: look at link curves directly |