3GPP TSG RAN WG1 Meeting #92bis	R1-1804285

Sanya, China, April 16- April 20, 2018

Agenda Item:	7.9

Source:	Huawei, HiSilicon

Title:	On coexistence between LTE and NR

Document for:	Discussion and decision

Introduction

NB-IoT is a key IoT technique introduced in LTE. Recently, NB-IoT has been widely deployed all over the world, as an important technique for machine type communication. Benefiting from the elaborate design based on the narrow bandwidth (1 RB, 180 kHz), it provides deep coverage and is expected to have a battery life longer than ten years.

In 3GPP Rel-15, NR is designed to coexist with LTE within LTE bands, which are also important deploying bands for NB-IoT. It can be foreseen that NB-IoT and NR will coexist within the same frequency band in the coming years. As has been evaluated, NB-IoT fulfills all of the requirements of 5G Low Power Wide Area (LPWA) machine type communication, and is a critical component of 5G. In RAN#79, it was agreed that LPWA use cases will continue to be addressed by evolving LTE-M (eMTC) and NB-IoT [1], and that no NR based solution will be studied or specified for LPWA use cases in Rel-16.

In this contribution, we provide an overview of coexistence between LTE and NR, where the scenarios of coexistence of NB-IoT and NR are discussed and analyzed. We also share our views on potential issues for future study.

Discussion

Frequency shift and subcarrier alignment

Similar to LTE, the UL carrier of NB-IoT has a 7.5 kHz shift, while NR may or may not be able to configure a 7.5 kHz shift depending on the operating band. In [2], we have analyzed the performance loss when both NR and NB-IoT use 15 kHz SCS on the uplink but with a 7.5 kHz frequency shift. It is observed that both NR and NB-IoT have a 1~2 dB performance loss. To mitigate the interference between them, a 1-RB-wide guard band is required, which increases the deployment overhead in the coexistence scenario between NR and NB-IoT.

In conclusion, 7.5 kHz frequency shift should be supported in NR FDD for uplink sub-carrier alignment with NB-IoT. Note that RAN1 has already agreed that 7.5 kHz shift is supported in the UL carrier of NR FDD in RAN1# AH1706. In the case when SUL is configured for NR TDD carrier, RAN4 has already supported 7.5 kHz shift for SUL to align with LTE UL carrier.

In RAN4#86, RAN4 has agreed that 7.5 kHz shift in UL carrier is mandatory in Bands n1, n2, n3, n5, n7, n8, n20, n28 and n66 [3].  However, the NB-IoT can also be deployed in Bands n70, n71 and n74, where NR should also support 7.5 kHz shift in UL. This may also benefit eMTC, since n71and n74 are also bands where eMTC can be deployed.

Therefore, we propose to support 7.5 kHz shift in UL carrier in Bands n70, n71 and n74.

Proposal 1: Support 7.5 kHz frequency shift in UL carrier in NR FDD for subcarrier alignment with NB-IoT in Bands n70, n71 and n74.

Numerology issue

For NB-IoT, two different subcarrier spacing (SCS) are supported: 15 kHz and 3.75 kHz. Meanwhile, only normal CP is supported. For NR, when working in FR1 (i.e., below 6 GHz), 15 kHz, 30 kHz and 60 kHz SCS are defined. In particular, for 60 kHz, extended CP is supported.

From point of view of simplicity, when both NB-IoT and NR use 15 kHz subcarrier spacing and normal CP, we can focus on the alignment of subcarrier and RB boundary between NB-IoT and NR, and possibly effective resource reservation method for NB-IoT. But note that NB-IoT with 3.75 kHz SCS is also an important deployment scenario, and 1.25 kHz NPRACH may be introduced in NB-IoT in Rel-15 for coverage improvement. However, this is the same scenario as coexistence with 15 kHz subcarrier spacing LTE. NR should also support efficient coexistence with NB-IoT with different SCS to fully exploit their advantages in different scenarios.

For the case of different numerologies, inter-carrier interference (ICI) occurs, and thus guard band or ICI cancelling scheme will be required. Moreover, problems may arise due to the misalignment of symbol/slot length in time domain. Possible issues, for example, BWP/carrier segmentation, additional baseband/radio filter, CORESET overlapping, required bandwidth of guard band between NB-IoT and NR for different NR SCS, performance loss of these two systems, could be considered. Mechanisms for managing ICI caused by difference of numerology could be further studied.

Observation 1: Mechanisms for managing ICI caused by difference of numerologies between NB-IoT and NR could be further studied.

Scenarios of coexistence between NB-IoT and NR

Coexistence with NB-IoT with different operation modes

Generally, there are three operation modes in NB-IoT: Inband, Guardband and Standalone mode. For Inband mode, it can be further divided as Inband-SamePCI and Inband-DifferentPCI mode. Regarding the time-frequency resource, these operation modes have different characteristics. Some of the differences are concluded in the following table:

Table 1 Difference between different operation modes

| Operation Mode | Raster Offset | Starting symbol in a subframe[1] | Utilization of LTE CRS | Mapping in LTE CRS REs |
| --- | --- | --- | --- | --- |
| Inband-SamePCI | Yes | 2nd, 3rd or 4th | Yes | No |
| Inband-DifferentPCI | Yes | 2nd, 3rd or 4th | No | No |
| Guardband | Yes | 1st | No | Yes |
| Standalone | No | 1st | No | Yes |

[1] This is for data channel.

There are two cases to be considered: (1) NB-IoT has already been deployed, and NR is going to be deployed in the same frequency band. (2) After the NR is deployed, NB-IoT is going to be deployed in/adjacent to the NR bandwidth.

NR coexists with already deployed NB-IoT

For the already deployed Inband and Guardband NB-IoT, the operation mode of NB-IoT depends on the relationship between NB-IoT and LTE. To align with the LTE RB boundary, for anchor carrier, a mandatory {±2.5, ±7.5} kHz raster offset is introduced and indicated in MIB-NB.

The scenario of coexistence with already deployed NB-IoT with different operation modes should be considered by NR, as illustrated in Figure 1. As can be seen from the figure, both Inband and Guardband NB-IoT may be located in NR frequency band. Similarly, though Standalone mode mainly focuses on re-farming GSM frequency band, it is still possible and allowed to be deployed within the LTE band.

Figure 1 NR coexist with already deployed NB-IoT with different operation mode

NB-IoT is going to be deployed in/adjacent to NR frequency band

When NB-IoT is going to be deployed in/adjacent to the already existed NR system frequency band, it may operate in Inband or Guardband mode, as illustrated in Figure 2.

Figure 2 NR coexist with going-to-deploy NB-IoT with different operation mode

In this case, for Inband mode anchor carrier, the issue of RB boundary alignment should be considered. In the DL carrier, only a small part of the NR RBs (about 20%) can be aligned with the NB-IoT anchor carrier candidates. This can be obtained from the fact that the least common multiple of NB-IoT raster (100 kHz) and NR RB bandwidth (180 kHz) is 900 kHz, so 1 out of 5 contiguous RBs of NR satisfies the requirement of NB-IoT raster, which is similar to the LTE case. If the NB-IoT anchor carrier still has to align with NR RB boundary, the allowable deployed location may be quite limited. Therefore, NR may consider supporting efficient coexistence with NB-IoT anchor carrier that only with aligned subcarriers but without aligned RB boundary.

For Guardband deployment, note that the guard band of NR has largely reduced compared to LTE, due to the improvement of spectrum efficiency. The minimum guard band of different SCS and channel bandwidth is captured in TS 38.101. For 15 kHz SCS, the minimum bandwidth of guard band is 242.5 kHz. However, to ensure that at least one NB-IoT anchor carrier can be deployed in the NR guard band, the bandwidth of NR guard band should satisfy:

With the restriction of 100 kHz raster, very limited number of NB-IoT anchor carrier can be deployed in the guard band of NR. In the worst case, it is possible that no NB-IoT anchor carrier can be deployed. This again emphasizes that NR should consider coexistence of multiple anchor carriers of NB-IoT within (rather than adjacent to) its frequency band.

In summary, the NB-IoT with Inband, Guardband or Standalone mode can be located within NR system band. NR may study efficient resource sharing mechanisms for multiple NB-IoT carriers, where the resource occupations of different operation modes, alignment of subcarriers and alignment of RB boundary are taken into consideration.

Observation 2: NR will coexist with multiple anchor and non-anchor NB-IoT carriers, which can be operated in Inband, Guardband and Standalone mode.

Interference avoidance between NB-IoT and NR

In this subsection, we focus on the coexistence of NB-IoT and NR in DL. Currently, NR supports two kinds of rate-matching schemes: RB-symbol level and RE level. Two kinds of schemes can be considered: semi-static resource sharing and dynamic resource sharing. In the following discussion, we focus on the case of same numerology and assume that the subcarriers of NB-IoT and NR are aligned.

Semi-static resource sharing

For semi-static resource sharing, NR will not use the subcarriers occupied by NB-IoT, even if no DL data is transmitted in NB-IoT within a period of time.

The current RB-symbol level rate-matching scheme based on the combined bitmaps can be applied for NB-IoT carrier reservation. However, if the NB-IoT anchor carrier is not align with the NR RB boundary, two RBs will have to be reserved in frequency domain, leading to a two times resource cost of NB-IoT, as illustrated in Figure 3(a). Also note that RB level granularity resource reservation may be realized by resource scheduling and can be transparent to NR UE, but it still leads to the same overhead in the coexistence scenario. NR may further consider mechanism of resource reservation to improve the efficiency, which is illustrated in Figure 3(b).

Figure 3 Illustration of semi-static resource sharing

Dynamic resource sharing

Dynamic resource sharing can be further considered based on the semi-static resource sharing. It allows higher spectrum utilization since (at least most of) the resource of the overlapping area can be used either by NR or NB-IoT. This requires ideal backhaul between the BS of NR and BS of NB-IoT, so it is more likely to be realized in the co-located scenario.

In NB-IoT, narrowband reference signal (NRS) are assumed to always be transmitted in the effective subframes in DL anchor carrier. From NB-IoT’s perspective, the NB-IoT UE can use NRS to estimate the DL channel at any time, even if no DL data is receiving. From NR’s perspective, it is reasonable to reserve the REs for NRS transmission, when dynamic sharing is applied.

Figure 4 Illustration of dynamic resource sharing, assuming REs for NRS are reserved

As illustrated in Figure 4, if dynamic resource sharing is adopted, the REs for NRS (and other possible signals) should also be reserved. When considering the RE level resource reservation for NB-IoT, the starting symbol in a subframe of NB-IoT can also be taken into consideration, but may bring higher complexity since the starting symbols are different for data channel, broadcasting channel and synchronization signals.

In addition, NR UE is not expected to handle the case where PDSCH DMRS are overlapping, even partially, with any RE(s) as reserved. For both 1 port and 2 ports NRS, it locates at the 5th, 6th, 12th and 13th symbols within a slot (the symbols are numbered from 0th to 13th). According to TS 38.211, in NR, the location of DMRS for PDSCH in time domain depends on the PDSCH duration, configuration type and mapping type. It is possible that NRS and NR DMRS are located in the same symbol, e.g. the 6th symbol within a slot. But the collision is not inevitable, and can be handled by implementation.

Conclusion

In this contribution, coexistence of LTE, NB-IoT and NR is discussed. The observation and proposal are concluded as follows:

Observation 1: Mechanisms for managing ICI caused by difference of numerologies between NB-IoT and NR could be further studied.

Observation 2: NR will coexist with multiple anchor and non-anchor NB-IoT carriers, which can be operated in Inband, Guardband and Standalone mode.

Proposal 1: Support 7.5 kHz frequency shift in UL carrier in NR FDD for subcarrier alignment with NB-IoT in Bands n70, n71 and n74.

- References

Chairman’s summary slides, Chennai, India, March 19 – 22, 2018.

R1-1802715, “On coexistence between LTE and NR”, Huawei, HiSilicon, RAN1#92, Athens, Greece, February 26 – March 2, 2018.

R4-1803564, “LS on UE feature list”, NTT DoCoMo, Intel, Athens, Greece, February 26 – March 2, 2018.