3GPP TSG RAN WG1 Meeting #92bis                                         R1-1804397

Sanya, China, Apr. 16th – Apr. 20th, 2018

Agenda Item:		    7.4.2

Source:			Samsung

Title:		   Considerations on receiver types for NoMA schemes

Document for:		Discussion and Decision

# Introduction

In RAN #75 meeting, the study item on non-orthogonal multiple access for NR has been approved [1].

This study will further progress on the evaluation of non-orthogonal multiple access schemes focusing on uplink, and provide recommendation on the non-orthogonal multiple access scheme(s) to be specified later.

Agreements, observations and evaluation assumption in Rel-14 study shall be the starting point. The detailed objectives are to study the following:

1 non-orthogonal multiple transmission scheme

- Transmitter side signal processing schemes for non-orthogonal multiple access [RAN1]:

  - Modulation and symbol level processing, including spreading, repetition, interleaving, new constellation mapping, etc.

  - Coded bit level processing including interleaving and/or scrambling, etc.

  - Symbol to resource element mapping, sparse or not, etc.

  - Demodulation reference signal. Other signal is not excluded.

- Receivers for non-orthogonal multiple access: [RAN1, RAN4]

  - MMSE receiver, successive/parallel interference cancellation (SIC/PIC) receiver, joint detection (JD) type receiver, combination of SIC and JD receiver, or other receivers

  - The study should consider performance, receiver complexity, etc.

- Procedures related to the non-orthogonal multiple access  [RAN1]

  - UL transmission detection

  - HARQ, including transmission scheme, feedback scheme, and combining scheme

  - Link adaptation MA signature allocation/selection

  - Synchronous and asynchronous operation

  - Adaptation between orthogonal and non-orthogonal multiple access

- Link and system level performance evaluation or analysis for non-orthogonal multiple access continued from performance metrics identified from Rel-14. The benchmark for comparison is OFDM contention based multiple access. Realistic modelling of Tx/Rx impairment including potential PAPR issue, channel estimation error, power control accuracy, collision, etc. should be considered. [RAN1]

  - Traffic model and Deployment scenarios of eMBB (small packet), URLLC and mMTC

  - Device power consumption

  - Coverage (link budget)

  - Latency and signalling overhead

  - BLER reliability, capacity and system load

  - Physical abstraction (link-to-system mapping model)

Note: targeting common solution for mMTC, URLLC and eMBB small packet.

In this contribution, the receivers for NoMA schemes are discussed.

# Discussion on receiver type for NoMA schemes

For NoMA schemes, receivers have large impacts on overall detection performance. The multi-user interference caused by the super-positioned transmission is required to be canceled by advanced receiver. Here several types of receivers are discussed. Generally two types of advanced receivers can be applied, namely parallel interference cancelation (PIC) and serial interference cancelation (SIC).

## Parallel interference cancelation (PIC) detectors

As for PIC, iterative detection and decoding scheme is usually used and its basic structure is demonstrated in Fig. 1.

Fig. 1. Basic structure of iterative detection and decoding (PIC)

In Fig. 1, the multi-user detector and decoders exchanges information on decoded bits to improve the detection performance. The complexity is mainly casued by the multi-user detector. Here two types of detectors are introduced.

### Maximum a posterior Probability (MAP) detector

This type of detectors tries to maximize the a posterior probability of received symbols, which leads to near-optimal performance. By assuming low correlation between adjacent REs and Gaussian noise, the MAP receiving algorithm can be simplified to chip-by-chip receiver. NoMA schemes with bit/symbol-level interleaver or scrambler can reduce the correlation between REs and can facilitate the chip-by-chip MAP receivers. Although MAP receiver has theoretically best performance, the complexity is also considerable. This is caused by the fact that the MAP receiver will calculate the distance between received signal and all the possible combinations of transmitted symbols from all UEs. For example, if QPSK is used for a 6-UE case, the number of all potential combinations of transmitted symbols is 46=4096. As can be observed, the complexity of MAP algorithm is proportional to QK, where Q denotes modulation order and K denotes number of UEs.

For NoMA schemes with sparse mappings, since the number of UEs super-positioned on the same REs will be reduced, the complexity of applying MAP receiver can be reduced with nearly the same detection performance. For example, if sparse mapping with density 0.5 is used for 6-UE case with QPSK, the number of possible combinations of transmitted symbols is 43=64, which is reduced significantly. This means that for sparse mapping based NoMA schemes, MAP-based detector can be applied to enhance the detection performance with some level of increased complexity.

Observation 1: the sparsity can provide significantly complexity reduction to Chip-by-Chip MAP receivers.

Another similar detection algorithm is message-passing algorithm (MPA), which is used for codebook-based NoMA schemes, e.g. SCMA and PDMA. MPA is also a near-optimal receiver. However, since calculation of distance between received signal and all possible combinations of codewords from all UEs, the detection complexity is still proportional to the exponential of number of UEs.

### Elementary Symbol Estimator (ESE)

One possible way to reduce the detection complexity is ESE. Unlike chip-by-chip MAP which detects symbols from all UEs at the same times, to decode symbols for one UE, ESE treats signals from other UEs as noise by using central limit theorem. With the aid of Gaussian noise assumption, the ESE detection for one UE is simplified. With the iterative detection and decoding, channel decoding can help improve the detection reliability and the overall performance can be improved.

From descriptions above, we can observe that ESE detector can be regarded as single-user detector with Gaussian assumption. Only the estimation and calculation of mean and variance of interferences will cause additional operations. As a result, the detection complexity is much lower compared with chip-by-chip MAP detector. The complexity of ESE is proportional to the number of UEs and the modulation order.

It has been widely understood that the performance of ESE detector can approach that of MAP receiver at low coding rate region, that is to say, ESE is very suitable for low coding rate based NOMA schemes.

Observation 2: ESE detector with low complexity can approach the performance of MPA detector at low coding rate region, and could be the good candidate detector for low coding rate based NOMA schemes.

The detailed descriptions for ESE and chip-by-chip MAP detector can be found in Appendix.

## Serial interference cancelation (SIC) detectors

Besides PIC detectors, NOMA can use SIC receiver as well, whose basic structure is depicted in Fig. 2.

Fig. 2. Basic receiver structure of SIC.

MMSE-SIC is a conventional low complexity detection method which may achieve near-optimal performance in some suitable cases. Unlike above detection schemes, MMSE-SIC perform serial interference cancelation, which means that the detection is performed UE by UE. The MMSE detection is used for every single UE’s detection, which follows channel decoding. If the decoded data passes CRC, the corresponding signals are re-constructed and subtracted from the received signal, which will increase the SINR of signals from other UEs.

If the decoded data and the re-constructed signal are accurate enough, the residual interference is small and the SINRs of other UEs can be enhanced. Under this assumption, the performance of MMSE-SIC can approach that of MAP detectors. However, the well-known error propagation would be the major concern of this receiver type, due to the inaccuracy of reconstructed interference signals in practice. the accuracy of channel estimation may significantly affect the detection, since both initial detection and signal re-construction rely on the estimated channel. As a result, under real channel estimation, the loss of SIC detector may be relatively larger. Meanwhile, for large number of UEs, the cancellation among various UEs would suffer a lot from error propagation.

Hence, for SIC type detectors, the detection order is very important to the performance. Power differentiation among different UEs or channel ordering can benefit the detection performance in terms of both reliability (BLER) and complexity.

Observation 3: MMSE-SIC is also a low complexity detector capable for NOMA multi-user detection, but error propagation may limit the performance in practice, especially for large number of multiplexed user.

Proposal: Both complexity and BLER performance should be considered when study the receivers for NoMA.

# Conclusion

In this contribution, we discussed receiver types for NoMA schemes, and following observations and proposal are presented:

Observation 1: the sparsity can provide significantly complexity reduction to Chip-by-Chip MAP receivers.

Observation 2: ESE detector with low complexity can approach the performance of MPA detector at low coding rate region, and could be the good candidate detector for low coding rate based NOMA schemes.

Observation 3: MMSE-SIC is also a low complexity detector capable for NOMA multi-user detection, but error propagation may limit the performance in practice, especially for large number of multiplexed user.

Proposal: Both complexity and BLER performance should be considered when study the receivers for NoMA.

# References

RP-170829, “New Study Item proposal: Study on Non-orthogonal Multiple Access for NR”, ZTE, CATT, Intel, Samsung, March, 2017

TR38.802,” Study on New Radio Access Technology: Physical Layer Aspects”.

TR38.913, “Study on scenarios and requirements for next generation access technologies”.

# Appendix

## A.1 Detailed description for ESE and chip-by-chip MAP detector

A.1.1 ESE detector

Assume on the n-th RE, the received signal rn is the combination of all users’ signals plus the noise en:

rn=k=1Khkxk(n)+en, n=1,2,…,M,   	

in which n is the bit (chip) index and hk is the kth user’s equivalent frequency domain channel coefficient, which considers the OFDM demodulation procedure. When processing kth UE’s data, i.e.,xkn, the receiver will treat others’ signals as interference. Thus, the overall interference plus noise is denoted by δkn:

rn=hkxkn+δkn, 	                                 (2)

where

δkn=k'≠k,k'∈Gk(n)hk'xk'n+en,                                                                (3)

where Gk(n) denotes the set contains all the UEs superposed on the n-th chip according to the grid-mapping pattern.

Based on the central limit theorem (CLT), δknis approximated as a Gaussian variable, in which the obtained mean Eδknand variance Varδkn are given as:

Eδkn=Ern-hkExkn,                             (4)

Varδkn=Varrn-hk2Varxkn,                         (5)

Furthermore, based on (4) and (5), the conditional Gaussian probability density function of rn is derived based on

prnxkn=±1=               12πVarδknexp-rn-±hk+Eδkn22Varδkn.               (6)

- Based on (6), the extrinsic LLR for UE k can be calculated.

A.1.2 Chip-by-chip MAP detector

- The probability density function of rn conditioned on the bkn is calculated as

prnbkn=b=

12πσ2{x(n),bkn=b}exp-rm-i=1Khixi(n)22σ2,                             (7)

where the summation condition {x(n),bkn=b} denotes all the possible combinations of {xin,i=0,…,K-1} with bkn=b, taking the grid-mapping pattern into considerations.

For IGMA, the LLR for the k-th UE is calculated as

LLRkdn=lnprnbkn=+1prnbkn=-1.                                            (8)

- By using max-log-MAP approximation, the computational complexity of (8) can be further reduced.