3GPP TSG RAN WG1 Meeting #92bis 	R1-1804451

# Sanya, China, April 16th – 20th, 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | CRNum | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on PUCCH multiplexing in 38.213 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-04-16 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-15 (Release 15) |  |
| Reason for change: |  | In 9.2.3, the description of PUCCH multiplexing is not complete, according to the agreement in RAN1#88bis and RAN1#90bis: - Two NR-PUCCHs can be transmitted from one UE on the same slot in TDM manner. – The two NR-PUCCHs can be short-PUCCH. – The two NR-PUCCHs can be long-PUCCH + short-PUCCH. – FFS whether or not to have the two NR-PUCCHs as long-PUCCH + long-PUCCH – FFS: other multiplexing scheme(s) between the two NR-PUCCHs – FFS the case of more than 2 NR-PUCCHs in one slot from a UE (if more than 2, only short-PUCCHs) In RAN1#90bis, the periodicities of SR for one UE were agreed as 2 symbols and 7 symbols within a slot. According to the above agreement, one UE can transmit one or more PUCCH with the same and different PUCCH formats in different symbols in one slot, but can only transmit at most two long PUCCHs in different symbols in one slot. These restrictions should be captured. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add UE behavior restrictions in the description of PUCCH multiplexing in 9.2.3. |  |  |  |  |  |  |  |  |
| U |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE behavior is not clear and there is no restriction on PUCCH multiplexing for the same or different PUCCH format. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  | TS38.211 |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

### 9.2.3	UE procedure for reporting HARQ-ACK

A UE may transmit one or more PUCCHs with the same and different PUCCH formats, but at most two PUCCHs with format 1, 3 or 4, on a serving cell in different symbols within a slot of  symbols as defined in [4, TS 38.211]. With reference to slots for PUCCH transmissions, if the UE detects a DCI format 1_0 or a DCI format 1_1 scheduling a PDSCH reception or a DCI format 1_0 indicating a SPS PDSCH release over a number of symbols where the last symbol is within slot , the UE shall provide corresponding HARQ-ACK information in a PUCCH transmission within slot , where  is a number of slots and is indicated by the PDSCH-to-HARQ-timing-indicator field in the DCI format.

#### < Unchanged parts are omitted >