3GPP TSG RAN WG1 Meeting #92	bis                                        R1-1805441

Sanya, China, Apr. 16 – Apr. 20, 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.213 | CR |  | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of SRS antenna switching for 4 ports in TS36.213 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm, Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15 |  |  |  |  |  |  | Date: |  | 2018-04-03 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | At RAN1#91 meeting, it was agreed to support SRS antenna switching for 1T4R and 2T4R UE antenna configurations in Rel-15. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Support of SRS antenna switching for UE with 1T4R and 2T4R. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The SRS antenna swtiching for 1T4R and 2T4R are not supported. Then the agreement is not captured. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2, 8.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## 8.2	UE sounding procedure

< Unchanged parts are omitted >

For a TDD serving cell, SRS transmissions can occur in UpPTS and uplink subframes of the UL/DL configuration indicated by the higher layer parameter subframeAssignment for the serving cell.

When closed-loop UE transmit antenna selection is enabled for a given serving cell for a UE that supports transmit antenna, or for a UE that can be configured with SRS-Antenna-Switching-1T4R or SRS-Antenna-Switching-2T4R,

- When higher layer parameter ‘SRS-Antenna-Switching-1T4R’ is set to ‘on’ for a given serving cell, the index anSRS, of the UE antenna that transmits the SRS at time nSRS is given by

anSRS=nSRS mod 4, for both partial and full sounding bandwidth, and when frequency hopping is disabled (i.e., bhop≥BSRS),

anSRS=nSRS+nSRSmax⁡(4, K)+βnSRS4modmax⁡(4, K)4 mod 4,when K is evennSRS mod 4,                                                                                            otherwise

with β=1,if N1=2, N2=20,otherwise               , when frequency hopping is enabled (i.e., bhop<BSRS).

- When higher layer parameter ‘SRS-Antenna-Switching-2T4R’ is set to ‘on’ for a given serving cell for a UE configured with Λ UE antenna pairs, where Λ={2 or 3} is given by higher layer parameter ‘SRS-Antenna-Switching-2T4R-NrofPairs’, the index anSRS for the UE antenna pair as 2anSRS,  2anSRS+1 when Λ =2, or 0,  anSRS+1 when Λ =3 that transmits the SRS at time nSRS is given by

anSRS=nSRS mod Λ for both partial and full sounding bandwidth, and when frequency hopping is disabled (i.e., bhop≥BSRS),

anSRS=nSRS+nSRSΛ+βnSRSK mod Λ,when K mod Λ=0nSRS mod Λ,                                               otherwise

with β=1,if K mod Λ2=00,otherwise             , when frequency hopping is enabled (i.e., bhop<BSRS).

- Otherwise, the index , of the UE antenna that transmits the SRS at time nSRS is given by

, for both partial and full sounding bandwidth, and when frequency hopping is disabled (i.e.,  ),

,

when frequency hopping is enabled (i.e., ),

where values BSRS, bhop, Nb, and nSRS are given in Subclause 5.5.3.2 of [3], and (where  regardless of the  value), except when a single SRS transmission is configured for the UE. If a UE is configured with more than one serving cell, and for a group of cells belonging to bands that are signalled to be switched together in txAntennaSwitchUL the UE is not expected to transmit SRS on different antenna ports simultaneously. If a UE is configured with more than one serving cell, and for a group of cells belonging to bands that are signalled to be switched together in txAntennaSwitchUL the UE is not expected to transmit SRS and PUSCH on different antenna ports simultaneously.

When higher layer parameter ‘SRS-Antenna-Switching-1T4R’ is set to ‘on’ or ‘SRS-Antenna-Switching-2T4R’ is set to ‘on’ for a serving cell, a UE is not expected to be configured with more than two antenna ports for any uplink physical channel or signal on that serving cell.

< Unchanged parts are omitted >

## 8.7	UE transmit antenna selection

UE transmit antenna selection is configured by higher layers via parameter ue-TransmitAntennaSelection. The UE is not expected to be configured with either SRS-Antenna-Switching-1T4R or SRS-Antenna-Switching-2T4R simultaneously with ue-TransmitAntennaSelection.

A UE configured with transmit antenna selection for a serving cell is not expected to

- be configured with more than one antenna port for any uplink physical channel or signal for any configured serving cell, or

- be configured with trigger type 1 SRS transmission on any configured serving cell, or

- be configured with simultaneous PUCCH and PUSCH transmission, or

- be configured with demodulation reference signal for PUSCH with OCC for any configured serving cell (see [3], Subclause 5.5.2.1.1), or

- receive DCI Format 0 indicating uplink resource allocation type 1 for any serving cell, or

- be configured with a SCG.

If UE transmit antenna selection is disabled or not supported by the UE, the UE shall transmit from UE port 0.

If closed-loop UE transmit antenna selection is enabled by higher layers the UE shall perform transmit antenna selection for PUSCH in response to the most recent command received via DCI Format 0 in Subclause 5.3.3.2 of [4]. 
If a UE is configured with more than one serving cell, and for a group of cells belonging to bands that are signalled to be switched together in txAntennaSwitchUL the UE may assume the same transmit antenna port value is indicated in each PDCCH/EPDCCH with DCI format 0 in a given subframe.

If open-loop UE transmit antenna selection is enabled by higher layers, the transmit antenna for PUSCH/SRS to be selected by the UE is not specified.

< Unchanged parts are omitted >