3GPP TSG RAN WG1 Meeting #92bis																R1-1805575

Sanya, China, April 16th – 20th, 2018

Source: 	Intel Corporation

Title:	Summary of simultaneous transmission and reception of channels/signals

Agenda item:	    7.1.2.2.6

Document for:	Discussion and Decision

# Introduction

The following was concluded in RAN1#92:

- RAN1 to discuss at least the following combinations of DL physical channels and reference signals for the simultaneous reception assuming FR2 operation:

| Channels/RS in same OFDM symbol | Same CC / BWP | Different CCs |
| --- | --- | --- |
| SSB + PDSCH |  |  |
| CSI-RS + PDSCH (including DM-RS/PT-RS) - A-CSI-RS (below and above threshold), P CSI-RS, SP CSI-RS - BM CSI-RS, CSI-RS for CSI, CSI-RS for tracking - CSI-RS Repetition ‘ON’, ‘OFF’ - CSI-IM |  |  |
| PDCCH / CORESET+ PDSCH - Default CORESET (CORESET with lowest ID which is used for obtaining default QCL assumption) - Non default CORESET |  |  |
| CSI-RS + PDCCH / CORESET | Rx spatial QCL should be ensured by the NW configuration* |  |
| PDCCH / CORESET + PDCCH / CORESET | [N/A] |  |
| CSI-RS + SSB | Rx spatial QCL should be ensured by the NW configuration* |  |
| CSI-RS + CSI-RS |  |  |
| PDSCH + PDSCH with different RNTIs | TBD |  |
| SSB + SSB | N/A |  |
| PDSCH + PDSCH (C-RNTI + C-RNTI) | N/A |  |

*Note: Already agreed in the previous meetings

- RAN1 to discuss at least the following combinations of UL physical channels and reference signals combinations for the simultaneous transmission assuming FR2 operation:

| Channels/RS in same OFDM symbol | Same CC | Different CCs |
| --- | --- | --- |
| PUCCH + PUCCH | N/A |  |
| PUSCH + PUSCH | N/A |  |
| SRS + SRS |  |  |

- RAN1 to consider at least the following solutions:

  - Skipping transmission of reference signal / physical channel, e.g. QCL assumptions are not the same

  - Overriding QCL assumptions

  - Prioritization of the processing of one physical channel /signal over another

  - NW configuration should ensure the same QCL assumption for the physical channel /signal

  - Other solutions are not precluded

The views expressed on this topic have different levels of stability. Consider visiting sections 2.1, 2.4, 3.1, 3.2, 3.3 first.

# General issues

## 2.1 UE capability

Summary: It has been observed that the need for scheduling restrictions or rules for simultaneous transmission and reception of certain channels/RS is due to the limitation of (analog) RF beamforming of certain UEs due to non-frequency-selective beam weights. It has been expressed [E///, ITRI] that specification changes to benefit/allow such UE performance/function may be conditioned on UE capability to leave the broader RAN1 specification open to UEs that are not limited to such constraints.

Proposal (TBD):

- Consider UE capability signaling to indicate RF beamforming constraints to the gNB (considering intra-band contiguous synchronized CA scenario in FR2). RAN1 specification changes may be conditioned on UE capability.

Company views:

## 2.2 Multiple numerology

Summary: It has been observed that simultaneous multiplexing of physical channels and RS also includes the case where a UE may support multiple numerologies in different CCs [Huawei]. However this topic is not discussed in many contributions.

Proposal (TBD):

- Views on how to handle mixed numerology cases are premature. Consider making progress in RAN1#92bis by assuming same numerology if needed.

- Company views:

- [Huawei, HiSilicon] For forward compatibility, simultaneous transmission or simultaneous reception over different CC/BWPs with different numerologies should be considered in the first place. To this end, we proposed the simplest solution as above. For simultaneous transmission or simultaneous reception over different CC/BWPs with different numerologies, UE applies the QCL or spatial-relation assumption of the channel/RS with smallest subcarrier spacing

- [Samsung] This issue has been discussed in RS multiplexing agenda and some outcomes such as different numerologies for FDMed SSB and DMRS have been agreed. The discussion on the RS multiplexing with different numerologies is not finalized yet. So, we prefer to concentrate on spatial-QCL related issues in this agenda regardless of numerologies.

## 2.3 Non-spatial-QCL issues (rate matching)

Summary: RAN4-AH1 made the following agreement with respect an SMTC window in CA case that is not covered by a measurement gap (see Section 3). The consequence of this agreement is the introduction of a interruption time due to SSB. UE behavior is unclear for PDSCH reception that is multiplexed with SSB. This issue is raised in a couple of contributions [Nokia, Qualcomm]

Proposal (TBD)

- For PDSCH, UE should assume rate-matching or puncturing on the SSB symbols

- (may be handled in PT-RS session?) For PTRS associated with PDSCH, UE should assume puncturing on the SSB symbols

Company views:

- [Samsung] We think that the first bullet was discussed already (no need for further specification supports).

## 2.4 Specification

Summary: Majority of contributions have proposed certain specification changes to allow scheduler flexibility and UE behavior predictability (not left completely to UE implementation). Also, if multiplexing is allowed and behavior is left to UE implementation, it adds significant complexity on the UE side to make reasonable decisions. (RAN4-AH1 also made the agreements along these lines).

Proposal (TBD):

- Strive to specify UE behavior for cases of simultaneous multiplexing of physical channels and RSs. This includes certain multiplexing use-cases that the UE expects or does not expect and UE behavior upon determining such a multiplexing use-case.

- Consider the cases of simultaneous multiplexing of physical channels and RSs as concluded in RAN1#92 (additionally – SSB+PDCCH, PUCCH+PUSCH, SRS+PUCCH/PUSCH)

Company views:

- [Samsung] We prefer to minimize the specification impact as much as possible. Even though some cases may require specification support, we see that most of cases does not require any spec impact at all or just require same QCL assumption which is ensured by NW. In that sense, it is hard to agree to specify UE behaviors for all possible cases. Furthermore, concluding the existing cases would be better than discussing new or past issues.

# Downlink multiplexing issues

## 3.1 SSB+PDSCH

Summary: It was observed that a relevant RAN4-AH1 agreement as specified in 38.133 V15.1.0 (section 9.2.5.3.3) is as follows:

Scheduling availability of UE performing measurements on FR2:

The following scheduling restriction applies due to SS-RSRP or SS-SINR measurement on an FR2 intra-frequency cell

-	The UE is not expected to transmit PUCCH/PUSCH or receive PDCCH/PDSCH on SSB symbols to be measured, [1] symbol before each consecutive SSB symbols and 1 data symbol after each consecutive SSB symbols within SMTC window duration (it is assumed that  useServingCellTimingForSync is always enabled for FR2)

The following scheduling restriction applies to SS-RSRQ measurement on an FR2 intra-frequency cell

-	UE is not expected to transmit PUCCH/PUSCH or receive PDCCH/PDSCH on SSB symbols to be measured , RSSI measurement symbols, 1 data symbol before each consecutive SSB/RSSI symbols and 1 data symbol after each consecutive SSB/RSSI symbols within SMTC window duration (it is assumed that that  useServingCellTimingForSync is always enabled for FR2)

When intra-band carrier aggregation is preformed, the scheduling restrictions apply to all serving cells on the band.

Editor’s Note : Scheduling restrictions for inter-band carrier aggregation are FFS

In contributions, several solutions are proposed for the SSB+PDSCH multiplexing case with some contributions not mentioning the RAN4-AH1 discussion. The proposed solutions include the following:

- Skipping PDSCH which is an extension of RAN4 agreement

- Ensure QCL meaning NW configuration should ensure the same QCL assumption

- Left to UE implementation.

Proposal (TBD):

- RAN4 agreement exists for SSB-RRM + PDSCH multiplexing for non-CA and CA cases

- At least For cases outside SMTC window (SSB-RLM+PDSCH, SSB-BM+PDSCH) including CA:

  - Alt-1: (multiplexing restriction) Extend the RAN4 agreement to the case of SSB+PDSCH multiplexing for both CA and non-CA. This means that the UE is not expected to receive PDSCH on SSB symbols (for same numerology in FR2).

  - Alt-2: Alt-1 but without PDSCH associated with SI-RNTI, P-RNTI

  - Alt-3:  ensure-QCL,

  - Alt -4: b) No specification impact - left to UE implementation (Samsung)

  - Alt-5: The UE is not expected to receive PDSCH on SSB symbols (also other measurement signals like CSIRS-BM) within configured SMTC-like windows.

  - Alt-6: The UE is not expected to perform Rx beam refinement on SSB occasions (Rx beam refinement may be associated with BM, RLM, BFD)

Note: The above applies to the case with same numerology for SSB and PDSCH. The case with different numerologies for SSB and PDSCH is to be addressed.

Company views:

- [Samsung]: “Allow different Rx spatial QCL indication and resolve it based on UE implementation” seems to be enough in our understanding.

## 3.2 SSB+PDCCH

Summary: This case is raised in [E///]. The RAN4 agreement is applicable here as well. It was proposed to extend the RAN4 agreement. The following can be considered:

Proposal (TBD):

- RAN4 agreement is adopted/valid for SSB-RRM + CORESET multiplexing for non-CA cases

- For all other cases (SSB-RLM+CORESET, SSB-BM+CORESET) including CA:

  - Alt-1: (multiplexing restriction) Extend the RAN4 agreement to the case of SSB+CORESET multiplexing for both CA and non-CA. This means that the UE is not expected to receive PDCCH on SSB symbols (for same numerology in FR2).

Company views:

- [Samsung]: “Allow different Rx spatial QCL indication and resolve it based on UE implementation” seems to be enough in our understanding.

## 3.3 SSB+CSIRS

Summary: Several contributions have discussed this issue and observed that a prior RAN1 agreement is to allow FDM multiplexing if spatial QCL can be ensured by the gNB. A majority of contributions have proposed that NW configuration should ensure the same QCL assumption. It is also noted in some contributions [Intel, Vivo] that for multiplexing instances involving a beam-management RS or a RRM-RS, ensuring spatial QCL by the gNB may still limit Rx beam switching (refinement) by the UE. The proposal below is to consider dividing the cases into two groups for discussion.

Proposal (TBD):

- In cases where multiplexing is expected to degrade UE function/performance (even if QCL Type D can be ensured and the same Tx beam is used by the gNB) by limiting UE Rx beam switching opportunities: SSB + CSI-RS-CSI, SSB+CSI-RS-IM, SSB+CSI-RS-Tracking

  - Alt a: (multiplexing restriction) UE is not expected to receive such CSIRS on SSB symbols

  - Alt b: FFS

- For all other cases (SSB+CSIRS-RLM, SSB+CSIRS-RRM, SSB+CSIRS-BFD, SSB+CSIRS-BM)

  - NW configuration should ensure the same QCL assumption (if not ensured then multiplexing not allowed)

Company views:

- [Samsung]: Regarding spatial QCL, prefer to keep the current specification (Rx spatial QCL should be ensured by the NW configuration). In case of numerology issue, it is prefer to have unified behavior that for SSB and DMRS case (i.e. support different numerologies by UE capability).

## 3.4 CSIRS+PDSCH

Summary: Several contributions have discussed this case and the following solutions are proposed. It is, however, not clear if PDSCH is prioritized then whether the UE function would be compromised with respect to multiplexed CSI-RS-RRM or CSI-RS-BM.

- 1: Prioritization of CSIRS for certain combinations: QCL assumption is based on CSI-RS

- 2: NW configuration should ensure the same QCL assumption for certain cases

- 3: Prioritization of PDSCH: QCL assumption is based on PDSCH

- 4: Based on UE implementation for certain cases

- 5: Disallow multiplexing for certain cases

- 6. Prioritization based on channel type and threshold

Proposal (TBD):

- CSIRS-RRM/BM+PDSCH  multiplexing restriction similar to RAN4 interruption time for SSB+PDSCH

- CSIRS-RLM/BFD/CSI/IM/Tracking+PDSCH  NW should ensure same QCL or other solutions including QCL-overriding rule

- Prioritization of PDSCH: QCL assumption is based on PDSCH

Company views:

- [ZTE] Prioritization depends on whether the spatial QCL of signals/channels have been determined from TCI state during the reception e.g. depending on whether it is candidate PDSCH with scheduling offset < k

  - e.g. QCL-overriding rule is applied according to the following priority:

    - PDSCH > ap-CSI-RS > sp-CSI-RS/p-CSI-RS > candidate  PDSCH= candidate ap-CSI-RS

- [Samsung] Allow different Rx spatial QCL indication and resolve it based on UE implementation

- [vivo] We also think the above should be differentiated based on PDSCH scheduling offset and CSI-RS time domain behavior. If the spatial QCL could be guaranteed for the CSI-RS/PDSCH, they could always be multiplexed on the same symbol. If not, PDSCH beyond offset should be prioritized.

## 3.5 CSIRS+PDCCH

Summary: Several contributions have discussed this case and the following solutions are proposed. It has been observed in some contributions that in cases where the UE is expected to perform Rx-beam switching (e.g. CSI-RS-BM-rep-ON), multiplexing is not desired. It is also noted that a prior RAN1 agreement is to allow FDM multiplexing if spatial QCL can be ensured by the gNB.

From 38.214:

The CSI-RS defined in Subclause 7.4.1.5 of [4, TS 38.211], may be used for time/frequency tracking, CSI computation, L1-RSRP computation and mobility.

If the UE is configured with a CSI-RS resource and a CORESET in the same OFDM symbol(s), the UE may assume that a PDCCH DM-RS transmitted in the CORESET are quasi co-located with ‘QCL-TypeD’, if ‘QCL-TypeD’ is applicable. Furthermore, the UE shall not expect to be configured with the CSI-RS in PRBs that overlap those of the CORESET.

The UE is not expected to receive CSI-RS and [SystemInformationBlockType1] message in the overlapping PRBs.

- 1: NW configuration should ensure the same QCL assumption

- 2: Disallow multiplexing for certain CSIRS types

Proposal:

- CSIRS-RRM/BM+PDCCH  multiplexing restriction similar to RAN4 interruption time for SSB+PDCCH

- Other cases  NW configuration should ensure the same QCL assumption (if not ensured then multiplexing not allowed)

Company views:

- [Samsung]: Keep the current specification (Rx spatial QCL should be ensured by the NW configuration)

## 3.6 PDCCH+PDSCH

Summary: Several contributions have discussed this case and the following solutions are proposed. It is noted, however, that not much discussions have been observed considering the different types of PDSCH (e.g. different RNTIs) in combination with the different types of CORESETs (e.g. associated to different search space types).

- 1: NW configuration should ensure the same QCL assumption

- 2: Prioritization of CORESET: QCL assumption is based on CORESET

- 3: Prioritization of PDSCH: QCL assumption is based on PDSCH

- 4: Prioritization based on channel type and threshold

Company views:

- [ZTE]: UE assumes the same Spatial Rx QCL between PDCCH/CORESET and candidate PDSCH for proposals 2 and 3 above

- [Samsung]: Rx spatial QCL should be ensured by the NW configuration

- [vivo]: Scheduled PDSCH beyond offset should be prioritized.

## 3.7 PDCCH+PDCCH

Summary: Several contributions have discussed this case and the following solutions are proposed.

- 1: NW configuration should ensure the same QCL assumption

- 2: Prioritization based on CC ind, Pcell: QCL assumption is based on Pcell, CC index

- 3: Prioritization of UESS: QCL assumption is based on UESS

- 4: Prioritization of CSS: QCL assumption is based on CSS

Company views:

- [Samsung]: Alt. 2.

## 3.8 CSIRS+CSIRS

Summary: Several contributions have discussed this case and there is a variety of solutions proposed based on the type of CSI-RS. There seems to be more discussions needed here.

Company views:

[Huawei, HiSilicon] To facilitate the discussions, we provided the two possible solutions as follows.

- 1: Prioritization based on CSI-RS function: CSI-RS for RLM/BFR > CSI-RS for BM > CSI-RS for CSI.

- 2: Prioritization based on most dynamic QCL indication, AP CSI-RS > SP CSI-RS> P CSI-RS.

- [Samsung]: Rx spatial QCL should be ensured by the NW configuration

- [vivo] Aperiodic CSI-RS could override periodic ones.

## 3.9 PDSCH+PDSCH

Summary: Several contributions have discussed this case and the following solutions are proposed.

For the case of different RNTIs

- 1: NW configuration should ensure the same QCL assumption

- 2: Prioritization based on RNTI: QCL assumption is based on RNTI

For the case of (C-RNTI) CA

- 1: NW configuration should ensure the same QCL assumption

- 2: Prioritization based on CC index: QCL assumption is based on CC index

Company views:

[Huawei] Proposal 2: Alt-1: P-RNTI/SI-RNTI/RA-RNTI > CS-RNTI (Grant free) > C-RNTI (Grant based)

[Samsung] Alt. 2

[vivo] SP-PDSCH scheduled could be overridden by aperiodically scheduled PDSCH.

## 3.10 SSB+SSB

Summary: Few contributions have discussed this issue and the following solutions are proposed. It is noted that there is not much discussion on all the functionalities of SSB: RLM, RRM, BM, BFD.

- 1: NW configuration should ensure the same QCL assumption

- 2: Prioritization based on Pcell/CC index: QCL assumption is based on SSB transmitted on Pcell/CC index

# Uplink multiplexing issues

## 4.1 PUCCH+PUCCH

Summary: Several contributions have discussed this issue and the following solutions are proposed.

- 1: NW configuration should ensure the same spatial-relation assumption

- 2: Prioritization based on Pcell/CC index: spatial-relation assumption is based on PUCCH transmitted on Pcell/CC index

- 3: Prioritization based on content: spatial-relation assumption is based on PUCCH content

## 4.2 PUSCH+PUSCH

Summary: Several contributions have discussed this issue and the following solutions are proposed.

- 1: NW configuration should ensure the same spatial-relation assumption

- 2: Prioritization based on Pcell/CC index: spatial-relation assumption is based on PUSCH transmitted on Pcell/CC index

- 3: Prioritization based on RNTI

Company views:

[Huawei]: For proposal 3: CS-RNTI (Grant free) > C-RNTI (Grant based) > SP-CSI-RNTI

[Samsung] Alt 2.

## 4.3 SRS+SRS

Summary: Several contributions have discussed this issue and the following solutions are proposed.

- 1: NW configuration should ensure the same spatial-relation assumption

- 2: Prioritization based on Pcell/CC index: spatial-relation assumption is based on PUCCH transmitted on Pcell/CC index

- 3: Prioritization based on SRS function: spatial-relation assumption is based on SRS function (SRS-BM, SRS-CB, SRS-non-CB, SRS-antenna-switch)

- 4: Prioritization based on most dynamic spatial-relation indication: spatial-relation assumption is based on AP/SP/P SRS

Company views:

[ZTE] Add following proposal: Depending on UE capability: The number of different RSs in higher layer parameter spatialRelationInfo for all SRS+SRS should be no more than the maximum number of  SRS resources transmitted simultaneously for UE capability .

- [Samsung]: Alt. 2

## 4.4 PUCCH+PUSCH

Summary: Few contributions have discussed this issue and the following solutions are proposed.

- 1: NW configuration should ensure the same spatial-relation assumption

## 4.5 SRS+PUCCH

Summary: Few contributions have discussed this issue and the following solutions are proposed.

- 1: NW configuration should ensure the same spatial-relation assumption

## 4.6 SRS+PUSCH

Summary: Few contributions have discussed this issue and the following solutions are proposed.

- 1: NW configuration should ensure the same spatial-relation assumption

- 2: Prioritization of PUSCH: spatial-relation assumption is based on PUSCH

Company views:

## 4.7 PRACH+SRS/PUSCH/PUCCH

Summary: Few contributions have discussed this issue and proposed to consider this as error-case. Note that it is not clear whether this multiplexing is allowed.

Company views:

- [Huawei]: Add proposal: Prioritization based on channel type: PRACH > PUCCH > PUSCH > SRS

# References

- Chairman Notes RAN1 #92 meeting