3GPP TSG RAN WG1 Meeting #93	R1-1805926

Busan, Korea, May 21 – 25, 2018

Agenda Item:	7.8.1

Source:	Huawei, HiSilicon

Title:	Consideration on evaluation features and parameters for IMT-2020 self evaluation

Document for:	Discussion and decision

Introduction

In RAN1#92 meeting and RAN1#92bis meeting, the evaluation features and assumptions for IMT-2020 submission was discussed. It was agreed in [1] that

- The technical features and system configurations that are supported by Rel-15 can be used in self evaluation.

In [1] and [2], the detailed technical parameters and configuration parameters for FR1 and FR2 are provided. In [3], the detailed evaluation methodology for mobility is provided.

In [1], other features that are supported by Rel-15 are given in Section 3. We think these features can contribute to the technical performance requirements related to eMBB as well as URLLC, and can be included in self evaluation.

In this contribution, the consideration on NR evaluation features and parameters for IMT-2020 self-evaluation is provided.

Furthermore, to facilitate the progress of self evaluation, the detailed evaluation method for connection density and reliability are also proposed, which could help the simulation setup for these evaluations.

General feature consideration

As agreed in [1], the technical features developed in Rel-15 can be used in self evaluation. Specifically, the technical features that can contribute to fulfilling a specific requirement, or further enhance the performance related to the specific technical requirement, can be used for evaluation of that specific requirement.

Based on the above consideration, the following features are proposed to be included in self evaluation for eMBB, mMTC and URLLC.

General technical features for eMBB requirements

As discussed in [4], the general features of eMBB evaluation are shown in Table 1.

Table 1 General technical features for eMBB

| General features | FR1 (below 6 GHz) | FR2 (above 24 GHz) |
| --- | --- | --- |
| NR on single-band | Un-paired spectrum: TDD Paired spectrum: FDD, flexible duplex, etc. | Un-paired spectrum: TDD |
| LTE on single-band | Un-paired spectrum: TDD Paired spectrum: FDD | - |
| NR on multi-band | NR with SUL+TDD band |  |
| NR/LTE inter-working on multi-band | NR/LTE dual connectivity with two frequency bands, with or without SUL |  |

A flexible duplex scheme is considered to enhance eMBB spectral efficiency for paired spectrum in addition to conventional FDD operation. In this case, the frame structure is configured to include a DL dominant slot on the DL part of the paired spectrum. By this means the DL channel reciprocity is enabled, and DL spectral efficiency is expected to be improved. Other duplexing schemes that could contribute to spectral efficiency could also be considered.

In addition, the NR multi-band with TDD and SUL band is considered to be able to improve UL user experienced data rate (“cell edge” user data rate). Compared to NR single band operation on 4 GHz, the SUL scheme is expected to utilize wide bandwidth on higher frequency band and hence reduce the required bandwidth on lower band. On the other hand, the SUL scheme could reach higher user experienced data rate compared to NR single band operation on 30 GHz using the same bandwidth. Furthermore, the multi-band operation with TDD and SUL has the potential to reduce the latency (for both DL and UL) compared to TDD band only. The evaluation results are

General technical features for mMTC requirements

The following features as shown in Table 2 can be considered as starting point for mMTC evaluation.

Table 2 General technical features for mMTC

| General features | FR1 (below 6 GHz) |
| --- | --- |
| NB-IoT | Paired spectrum: FDD |
| eMTC | Paired spectrum: FDD |

The above features are based on the 3GPP development towards mMTC usage scenario and the 3GPP agreement to employ NB-IoT/eMTC for mMTC evaluation for IMT-2020 submission.

General technical features for URLLC requirements

The following features as shown in Table 3 can be considered as starting point for URLLC evaluation.

Table 3 General technical features for URLLC

| General features | FR1 (below 6 GHz) |
| --- | --- |
| NR on single-band | Paired spectrum : FDD Un-paired spectrum: TDD |
| NR on multi-band | NR with TDD+SUL band |

The NR multi-band with TDD and SUL band is considered to be able to improve UL latency and DL latency as compared to single-band TDD operation.

Proposal 1: General technical features shown in Table 1, 2 and 3 are used as starting point for eMBB, mMTC and URLLC evaluation towards IMT-2020 submission.

Evaluation parameters for eMBB

To further progress the work such that one could set up the simulator for eMBB evaluation, the detailed technical and configuration parameters for NR TDD+SUL evaluation are proposed in this section.

Parameters for NR multi-band operation with SUL and TDD band

This feature can be used for UL user experienced data rate evaluation. The parameters for SUL + TDD evaluation are provided in Table 4.

Table 4 Features for UL SUL+TDD evaluation

| UL SUL+TDD | NR |
| --- | --- |
| Test environment | Dense Urban - eMBB |
| Evaluation configuration | Configuration C, Macro layer only, with 30 GHz (TDD band) and 4 GHz (SUL band) |
| Cell association and band selection | UEs select either 30 GHz or 4 GHz on the same TRxP for UL data transmission. UE selects the band and serving TRxP based on pre-defined criteria |
| Assumptions for UEs on 30 GHz | Use the assumption for NR UL TDD for FR2 as in Table 2 of [2] as starting point |
| Assumptions for UEs on 4 GHz | Use the assumptions for NR UL FDD as in Table 4 of [1] as starting point. |

Proposal 2: Use the parameters as shown in Table 4 as starting point for NR TDD+SUL evaluation for user experienced data rate.

Evaluation method for mMTC and URLLC

To progress the evaluation of connection density and reliability for mMTC and URLLC usage scenario, the detailed evaluation method is discussed and proposed in [5] and [6], respectively. It is proposed to adopt the evaluation method proposed in there documents as starting point for connection density and reliability evaluation.

Proposal 3: As the starting point, the evaluation method and procedure as proposed in R1-1805934 is used for connection density evaluation for NB-IoT; and the evaluation method as proposed in R1-1805935 is used for reliability evaluation.

Conclusion

In this contribution, the evaluation features and parameters for IMT-2020 self evaluation are proposed. It is proposed that,

Proposal 1: General technical features shown in Table 1, 2 and 3 are used as starting point for eMBB, mMTC and URLLC evaluation towards IMT-2020 submission.

Proposal 2: Use the parameters as shown in Table 4 as starting point for NR TDD+SUL evaluation for user experienced data rate.

Proposal 3: As the starting point, the evaluation method and procedure as proposed in R1-1805934 is used for connection density evaluation for NB-IoT; and the evaluation method as proposed in R1-1805935 is used for reliability evaluation.

- References

- R1-1803386, “Summary of offline discussion for IMT-2020 self evaluation”, Huawei, Feb 2018.

- R1-1805644, “Summary of discussion on eMBB evaluation method and parameters for FR2”, Huawei, April 2018.

- R1-1805643, “Summary of discussion on mobility evaluation method and parameters”, Huawei, April 2018.

- R1-1801789, “General consideration on IMT-2020 self evaluation”, Huawei, HiSilicon, Feb 2018.

- R1-1805934, “Considerations and evaluation results for IMT-2020 for mMTC connection density”, Huawei, HiSilicon, May 2018.

- R1-1805935, “Considerations and evaluation results for IMT-2020 for reliability”, Huawei, HiSilicon, May 2018.