3GPP TSG RAN WG1 Meeting #93								R1-1807767

Busan, South Korea, May 21 - 25, 2018

Title:	LS on useServingCellTimingForSync definition

Response to:

Release:	Rel-15

Work Item:	NR_newRAT-Core

Source:	RAN1

To:	RAN2, RAN4

Cc:

Contact Person:

#### Name:	Daewon Lee

E-mail Address:	daewon.lee (at) intel (dot) com

Attachments:

1. Overall Description:

RAN1 would like to kindly inform to RAN2 and RAN4 that RAN1 has discussed further on the definition of useServingCellTimingForSync indication and concluded on the following:

- For RRM, UE may assume that in TDD bands, the half radio frame boundaries of the cells in the same frequency layer are aligned.

- When useServingCellTimingForSync is set to TRUE for measurements, it means the following:

  - For intra-frequency measurements, UE may use the serving cell timing to derive the SSB index of neighbour cells in the same frequency layer.

  - For inter-frequency measurements, UE may use timing of any detected cell in the target frequency layer to derive the SSB index of neighbour cells of the target frequency layer

    - Note: cells in different frequency layer are not assumed to be half radio frame aligned.

- UE may assume that in TDD bands, useServingCellTimingForSync is assumed to be set to TRUE

- Note: half radio frame alignment is the minimal requirement in RAN1’s understanding that allows the UE to use timing of one cell to derive the SSB index of another cell. However, it is up to RAN4 to determine the exact requirements for the “useServingCellTimingForSync”. In RAN1 understanding, the agreements above should not impact RAN4 discussion on the requirements for “useServingCellTimingForSync”.

Additionally, RAN1 believes useServingCellTimingForSync information should be also applicable to UEs in IDLE/INACTIVE mode. It is up to RAN2 to determine to how to provide/apply useServingCellTimingForSync information in IDLE/INACTIVE mode.

2. Actions:

To RAN2: 	RAN1 would like kindly ask RAN2 and RAN4 to consider the above in their further work.

3. Date of Next TSG-RAN WG1 Meetings:

TSG-RAN WG1 Meeting #94 	20 – 24 Aug 2018			Gothenburg, Sweden

TSG-RAN WG1 Meeting #94bis 	08 – 12 Oct 2018			Chengdu, China