3GPP TSG RAN WG1 Meeting #93		R1-1807972

Busan, Korea, May 21st – 25th, 2018

Title:	LS on LTE V2X UE Categories and Reception Capabilities

Release:	Release 15

Work Item:	LTE_eV2X-Core

Source:	RAN1

To:	RAN2

Cc:	RAN4

Contact person:	Alexey Khoryaev

alexey.khoryaev@intel.com

Attachments:	N/A

# 1	Overall Description

RAN1 discussed the introduction of new LTE V2X UE sidelink categories and reception capabilities as an outcome of the Rel15 work item on LTE V2X. The following agreements were made by RAN1 WG during two email discussions:

[93-LTE-12] - Email discussion to finalize

- Transmission of physical parameter values set by ue-CategorySL-C-TX

- Reception of physical parameter values set by ue-CategorySL-C-RX

The following tables (Table 1 and Table 2) were agreed as an outcome of e-mail discussion [93-LTE-12] with respect to UE sidelink categories for transmission and reception. In particular, three new sidelink categories (3, 4, 5) were introduced for sidelink transmission (see physical parameter values in Table 1). For sidelink reception, two new sidelink categories (3 and 4) were agreed with corresponding physical parameter values captured in Table 2.

Table 1: Sidelink physical layer parameters for transmission

| UE SL-C-TX Category | Maximum number of SL-SCH transport block bits transmitted within a TTI | Maximum number of bits of a SL-SCH transport block transmitted within a TTI | Maximum number of supported layers for spatial multiplexing in SL-C | Comments |
| --- | --- | --- | --- | --- |
| SL-C-TX Category 1 | 25456 | 25456 | 1 |  |
| SL-C-TX Category 2 | 31704 | 31704 | 1 | 20MHz aggregated BW 16QAM |
| SL-C-TX Category 3 | 49272 | 32856 | 1 | 30MHz aggregated BW 16QAM (Applicable for 10MHz+10MHz+10MHz and 10MHz+20MHz) |
| SL-C-TX Category 4 | 48936 | 48936 | 1 | 20MHz aggregated BW 64QAM (Applicable for 10MHz+10MHz and 20MHz) |
| SL-C-TX Category 5 | 73488 | 48936 | 1 | 30MHz aggregated BW 64QAM (Applicable for 10MHz+10MHz+10MHz and 10MHz+20MHz) |

Table 2: Sidelink physical layer parameters for reception

| UE SL-C-RX Category | Maximum number of SL-SCH transport block bits received within a TTI | Maximum number of bits of a SL-SCH transport block received within a TTI | Total number of soft channel bits | Comments |
| --- | --- | --- | --- | --- |
| SL-C-RX Category 1 | 25456 | 25456 |  |  |
| SL-C-RX Category 2 | 31704 | 31704 | 737280 | 20MHz aggregated BW 16QAM |
| SL-C-RX Category 3 | 48936 | 48936 | 995328 | 20MHz aggregated BW 64QAM (Applicable for 10MHz+10MHz and 20MHz) |
| SL-C-RX Category 4 | 73488 | 48936 | 1492992 | 30MHz aggregated BW 64QAM (Applicable for 10MHz+10MHz+10MHz and 10MHz+20MHz) |

[93-LTE-13] Email discussion on V2X Sidelink Processing Capabilities – Indication per CC

The following agreements were made as an outcome of e-mail discussion [93-LTE-13] with respect to UE reception capabilities:

- Define minimum UE reception capability vs amount of aggregated BW per sidelink RX band combination as provided in table below:

| Aggregated RX BW by UE, MHz | V2X High Reception Capability | V2X Default Reception Capability |
| --- | --- | --- |
| 10, 20 (Rel.14, Rel.15) | 20 PSCCHs and 136 RBs | 10 PSCCHs and 100 RBs |
| 30 (Rel.15) | 30 PSCCHs and 204 RBs | 15 PSCCHs and 150 RBs |

- UE reception capability (V2X High/ V2X Default) is indicated per sidelink RX band combination

# 2. Actions:

To RAN2

ACTION:  	RAN1 respectfully asks RAN2 to take the above information into account for further work.

# 3. Dates of Next RAN1 Meetings:

RAN1#94	20 Aug – 24 Aug 2018	Gothenburg (SE)

RAN1#94bis	8 Oct  – 12 Oct 2018	Chengdu (CN)