3GPP TSG RAN WG1 Meeting #94																	R1-1808229

Gothenburg, Sweden, August 20 –24th, 2018

Source: 		vivo

Title:	Rel-15 UE features for eMBB and URLLC

Agenda Item:    7.1.7

Document for:   Discussion and Decision

- Introduction

In this contribution we provide our views on mandatory/optional support of RAN1 UE features based on the endorsed UE feature list by RAN#80.

- References

[1] RP-181484	LS on NR UE feature list

2.1 On remaining issues for Rel-15 eMBB UE features

The recommendations are shown in the tables (RAN WG recommendation column) with track change. The features which have been concluded by RAN1 or RAN were not shown for simplicity.

| Features | # | Feature group |  | Components |  | Prerequisite feature groups (listed in this sheet only) |  | Need for gNB to know whether the feature is supported by the UE (what happens if gNB does not know?) |  | Consequences if the feature is not supported by the UE |  | Type (see R2-1712078) |  | Need of FDD/TDD differentiation |  | Need of FR1/FR2 differentiation |  | RAN5 implication |  | Note |  | Responsible WG |  | RAN WG recommendation |  | TSG-RAN decision |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2. MIMO | 2 -1 | Basic PDSCH reception |  | 1. Data RE mapping 2. Single layer transmission |  |  |  | Yes |  |  |  | N.A. |  | N.A. |  | N.A. |  |  |  |  |  | RAN1 |  | Mandatory without capability signaling |  | Mandatory without capability signaling |
|  | 2-2 | PDSCH beam switching |  | 1. Time duration (definition follows section 5.1.5 in TS 38.214), Xi, to determine and apply spatial QCL information for corresponding PDSCH reception Note: candidate value will be decided after feature is completed. (note: this may not needed) Time duration is defined counting from end of last symbol of PDCCH to beginning of the first symbol of PDSCH Xi is the number of OFDM symbols, i is the index of SCS, l=1,2, corresponding to 60,120 kHz SCS. |  | 2-1 |  | Yes |  |  |  | Type-3 |  | No need |  | Applicable only for FR2 |  |  |  |  |  | RAN1 |  | Mandatory with capability signaling Candidate value set for X1 is {7, 14, 28}, Candidate value set for X2, {14, 28} |  |  |
|  | 2-3 | PDSCH MIMO layers |  | 1. Supported maximal number of MIMO layers |  | 2-1 |  | Yes |  | Only one layer is supported |  | Type 3 |  | N.A. |  | N.A. |  |  |  |  |  | RAN1 |  | Candidate values: {1,2,4,8} FFS on the minimal layers for different band or band combination. |  | For single CC standalone NR, it is mandatory with capability signaling to support at least 4 MIMO layers in the bands where 4Rx is specified as mandatory for the given UE and at least 2 MIMO layers in FR2. Some relaxations to this requirement may be applicable in the future (including in Rel-15). Mandatory in all cases means mandatory with capability signaling. It is not expected that there is a signaling change (i.e. signaling remains to be defined as {1, 2, 4, 8} in every band and every band combination, including FR1 and FR2 in all cases. |
|  | 2-4 | TCI states for PDSCH |  | 1. Support number of active TCI states per CC 2. maximum number of configured TCI states, |  | 2-1 |  | Yes |  | Only one TCI state can be supported |  | Type 1 |  | N.A. |  | N.A. |  |  |  |  |  |  |  | Component-1: Candidate value set: {1, 2, 4, 8 } Component-2: candidate value set: {4, 8, 16, 32, 64} FFS: mandatory value For component 1: Values above 2 is optional; For component 2: Values above 16 are optional |  |  |
|  | 2-13 | PUSCH codebook coherency subset |  | Proposal: 1. Supported codebook coherency subset type: Candidate value set: {non-coherent, partial/non-coherent, full/partial/non-coherent} |  | 2-12 |  | Yes |  | Only non-coherent codebook subset is supported |  | Type 1 |  | N.A. |  | N.A. |  |  |  |  |  |  |  | Candidate values: {non-coherent, partial-non-coherent, full-coherent} FR1:Non-coherent mandatory. |  |  |
|  | 2-14 | Codebook based PUSCH MIMO transmission |  | 1. Supported codebook based PUSCH MIMO with maximal number of supported layers |  | 2-13 |  | Yes |  | Uplink codebook based MIMO (with >1 Tx port) transmission is not supported |  | Type 3 |  | N.A. |  | N.A. |  |  |  |  |  |  |  | Candidate value: {no-codebook based MIMO, 1, 2, 4} 1 layer mandatory |  |  |
|  | 2-15 | non-codebook based PUSCH transmission |  | 1. Maximal number of supported layers (non-codebook transmission scheme): |  | 2-12 |  | Yes |  | No support of non-codebook based PUSCH transmission |  | Type 3 |  | N.A. |  | N.A. |  |  |  |  |  |  |  | Component-1 candidate values: {“No non-codebook based MIMO”, 1, 2, 4} Optional with capability signaling |  |  |
|  | 2-20 | Beam correspondence |  | 1. Support Beam correspondence |  |  |  | Yes |  | Beam correspondence is not supported |  | Type 1 |  | No need |  | N.A. |  |  |  | Note: Beam correspondence means each Tx port can be beamformed in a desirable direction but does not imply setting phase across ports |  |  |  | [Mandatory at least for FR2] |  |  |
|  | 2-21 | Periodic beam report |  | 1. Support report on PUCCH formats over 1 – 2 OFDM symbols once per slot 2. Support report on PUCCH formats over 4 – 14 OFDM symbols once per slot |  |  |  | Yes |  | No support of periodic L1-RSRP report |  | Type 1 |  | No need |  | N.A. |  |  |  |  |  |  |  | Mandatory with UE capability at least for FR2 FFS: for FR1 For FR1: Optional with capability signaling |  | Mandatory with UE capability at least for FR2 |
|  | 2-22 | Aperiodic beam report |  | 1. Support report on PUSCH |  |  |  | Yes |  | No support of aperiodic L1-RSRP report |  | Type 1 |  | No need |  | Yes |  |  |  |  |  |  |  | Mandatory with UE capability at least for FR2 Optional with capability signaling for FR1 |  | Mandatory with UE capability at least for FR2 |
|  | 2-25 | Beam reporting timing |  | 1. The number of symbols, Xi, between the last symbol of SSB/CSI-RS and the first symbol of the transmission channel containing beam report is at least RBi, where i is the index of SCS, i=1,2,3,4 corresponding to 15,30,60,120 kHz SCS. |  | 2-24 |  | Yes |  | Beam reporting time capability is not known by gNB |  | Type 1 |  | N.A. |  | N.A. |  |  |  |  |  | RAN1/4 |  | Candidate value set for X1 is {2, 4, [8]} Candidate value set for X2 is {4,8, [14]} Candidate value set for X3 is {7 or 8,14 or 15, 28} RAN1 will further decide between 7 and 8 and between 14 and 15. Candidate value set for X4, {14, 28, [42]} For FR1: the whole feature is optional. For FR2: the largest timing value for each SCS is mandatory, other values optional |  |  |
|  | 2-26 | Receiving beam selection using CSI-RS resource repetition "ON" |  | 1. Support Rx beam switching procedure using CSI-RS resource repetition "ON" 2. Recommended CSI-RS resource repetition number per resource set, |  |  |  | Yes |  | Rx beam switching is not supported |  | Type 1 |  | N.A. |  | N.A. |  |  |  |  |  |  |  | Support Rx beam switching is mandatory for bands at least > 6GHz Componet-2: candidate value set {2, 3, 4, 5, 6, 7, 8} Mandatory number of Rx beams 2; Optional for beam switching within one CSI-RS set; Optional with capability signaling for FR1 |  | Mandatory with UE capability at least for FR2 |
|  | 2-27 | Beam switching |  | 1. Maximum number of Tx + Rx beam changes a UE can conduct during a slot across the whole band CC BB_Total,. This number is defined as per SCS Note: it is assumed that spec enable the possibility to restrict the same beam across intra-band CCs |  | 2-24 |  | Yes. |  | No resetriction on the maximum number of Tx+Rx beam change for a slot |  | Type 1 |  | N.A. |  | N.A |  |  |  |  |  |  |  | Candidate value set: {4, 7, 14} More discussion needed on other candidate values, e.g. 2. Only for FR2 Support value of 2 |  |  |
|  | 2-28 | A-CSI-RS beam switching timing |  | 1. Minimum time between the DCI triggering of AP-CSI-RS and aperiodic CSI-RS transmission shall be at least KBi symbols. (Symbols measured from last symbol containing the indication to first symbol of CSI-RS), where i is the index of SCS, l=1,2,3,4 corresponding to 15,30,60,120 kHz SCS. |  | 2-27 |  | Yes |  | No recommendation on the desired beam switching timing |  | Type1 |  | N.A. |  | N.A. |  |  |  | [Note: any value larger than 56 is not supported in RRC configuration now. ] |  |  |  | Only applicable to FR2 Candidate values: Alt.1: {14, 26, 28, [42], [336]} Alt.2: {“14-14”, “26-26”, “28-28”, “26-280”, “28-336” } the first threshold corresponding to the case when UE doesn’t activate antenna panel, and the second threshold corresponding to the case when UE actives antenna panel value 28 mandatory with capability signaling |  |  |
|  | 2-29 | Non-group based beam reporting |  | 1. Support of non-group based RSRP reporting with N_max RSRP values reported |  | 2-24 |  | Yes |  | Non-group based beam reporting is not supported |  | Type1 |  | N.A |  | N.A. |  |  |  |  |  |  |  | candidate value set is {1, 2,4} For FR2: 4 mandatory with capability signaling For FR1, optional. |  |  |
|  | 2-30 | Uplink beam management |  | 1 Support of SRS based beam management 2. Supported max number of SRS resource per set (SRS set use is configured as for beam management). 3. Supported max number of SRS resource sets (SRS set use is configured as for beam management). |  |  |  | Yes |  | Uplink beam management is not supported |  | Type1 |  | N.A. |  | N.A. |  |  |  |  |  |  |  | Component-2, candidate value set is {8, 16, 32} Component-3, candidate value set is {from 1 to 8} For FR2: Value 8 mandatory with capability signaling FR1 optional |  |  |
|  | 2-31 | Beam failure recovery |  | 1. Maximal number of CSI-RS resources across all CCs for UE to monitor PDCCH quality 2. Maximal number of different SSBs across all CCs for UE to monitor PDCCH quality 3. Maximal number of different CSI-RS and/or SSB resources across all CCs for new beam identifications. 4. COREST recoveryControlResourceSetId if configured |  |  |  | Yes |  | Beam failure recovery is not supported |  | Type 1 |  | N.A. |  | N.A. |  |  |  | Component 4) is in addition to component 1) of 3-1 |  |  |  | Optional with capability signaling Component-1 candidate value set: {from 1 to 64} Component-2 candidate: {from 1 to 64} Component-3: Candidate value set is: {from 1 to 256} [UE is mandate to support at least 64.] |  |  |
|  | 2-33 | CSI-RS and CSI-IM reception for CSI feedback |  | 1. Supported max # of configured NZP-CSI-RS resources per CC, 2. Supported max # of ports across all configured NZP-CSI-RS resources per CC 3. Supported max # of configured CSI-IM resources per CC 4. Supported max # simultaneous CSI-RS resources in active BWPs across all CCs 5. Supported max total # of CSI-RS ports in simultaneous CSI-RS resources in active BWPs across all CCs |  | 2-32 |  | Yes |  | CSI acquisition is not supported |  | Type 3 |  | N.A. |  | N.A. |  |  |  | Note: all the candidate values are the range of capability signaling which doesn’t determine whether UE is mandatory to support all the signaling values. |  |  |  | Component-1 candidate values: {from 1 to 32} Component-2 candidate values: , further down-select between: Alt.1: {from 4 to 64}, Alt.2: {from 4 to 256}, Alt.3: {from 4 to 256} Component-3: candidate values: {1,2,4,8,16,32} Component2. Supported max # of ports {from 4 to 64}, Component 4: {from 1 to 64} Component5: (including both channel and NZP-CSI-RS based interference measurement) is: {from 2 to 256} |  |  |
|  | 2-34 | NZP-CSI-RS based interference measurement |  | 1. Support NZP-CSI-RS based interference measurement FFS whether to add the number of NZP-CSI-RS port for interference measurement |  | 2-33 |  | Yes |  | NZP-CSI-RS based interference measurement is not supported |  | Type 4 |  | No need |  | No need |  |  |  |  |  |  |  | [Optional] |  |  |
|  | 2-35 | CSI report framework |  | 1. Maximum number of periodic CSI report setting per BWP 2. Maximum number of aperiodic CSI report setting per BWP 3. Maximum number of semi-persistent CSI report setting per BWP 4. Support of advanced CSI process table (the Minimum duration Z, Z’ for processing a CSI) [Conditioned to the completion of advanced CSI process] 5. UE can process X CSI report(s) simultaneously across all CCs. CSI reports can be P/SP/A CSI and any latency class and codebook type. 6. Supported CSI computation type FFS: whether X should also count the SRS precoder derivation in case of reciprocity based SRS Tx |  | 2-32 |  | Yes |  | CSI report is not supported |  | Type 3 |  | N.A. |  | N.A. |  |  |  | NOTE: Other MIMO capability other than component 5 may further restrict (reduce) the number of simultaneously CSI report that UE is required to update |  |  |  | Component-1 candidate values: {1, 2, 3, 4} Component-2 candidate values {1, 2, 3, 4} Component-3 candidate values: {0, 1, 2, 3, 4} Component-4: signaling is a bitmap of size 8 Component-5: candidate values: {from 5 to 32} Componet-6: Candidate values: {Type-A, Type-B} The component 6 should be deleted: X should also count the SRS precoding |  |  |
|  | 2-36 | Type I single panel codebook |  | 1. A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously. Note: the above list doesn’t differentiate the latency class and feedback type. 2. Supported Codebook Mode(s) |  | 2-35 |  | Yes |  | No additional Type I codebook configurations other than the basic CSI feedback (2-32) is supported |  | Type 3 |  | No need |  |  |  |  |  | Note: simultaneously doesn’t mean in the same slot |  |  |  | Component-1: the candidate values for the max # of Tx port in one resource is {2, 4, 8, 12, 16, 24, 32} The candidate value set of the max # of resources is: {from 1 to 64} The candidate value set of total # of ports (including both channel and NZP-CSI-RS based interference measurement) is: {from 2 to 256} Component-2 candidate values: {Mode-1, Mode-2} Down-select: Alt.1 Mode-1 as mandatory Alt.2: Both Mode-1 and Mode-2 are mandatory (in this case, this component is not needed) For component 1: support the smallest value as mandatory For component 2: Alt.1 Mode-1 as mandatory |  |  |
|  | 2-38 | CSI report without PMI |  | Support CSI report without PMI |  | 2-35 |  | Yes |  | CSI report without PMI is not supported |  | Type 4 |  | No need |  | Yes |  |  |  |  |  |  |  | FFS Optional with capability signaling |  |  |
|  | 2-41 | Type II codebook |  | 1. A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously. Note: the above list doesn’t differentiate the latency class and feedback type. 2. Parameter “Lx” (number of beams) in codebook generation, where x is index of Tx ports, corresponding to 4,8,12,16,24 and 32 ports. 3. Support amplitude scaling type 4. Support amplitude subset restriction level |  | 2-35 |  | Yes |  | Type II codebook is not supported |  | Type 3 |  | N.A. |  | N.A. |  |  |  | Note: simultaneously doesn’t mean in the same slot |  |  |  | Optional with capability signaling Component-1: the candidate values for the max # of Tx port in one resource is {4, 8, 12, 16, 24, 32} The candidate value set of the max # of resources is: {from 1 to 64} The candidate value set of total # of ports (including both channel and NZP-CSI-RS based interference measurement) is: {from 2 to 256} Component-2, candidate values {2,3,4} Component-3, candidate values set: {wideband, wideband/subband} Component-4, candidate value set: {no restriction, subset restriction}, “no restriction” is mandatory and RAN1 hasn’t decide whether “subset restriction” is mandatory or not, if it’s mandatory then this component is not needed. |  |  |
|  | 2-43 | Type II codebook with port selection |  | 1. A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously. Note: the above list doesn’t differentiate the latency class and feedback type. 2. Parameter “Lx” (number of selected ports) in codebook generation, where x is index of Tx ports, corresponding to 4,8,12,16,24 and 32 ports. 3. Support amplitude scaling type |  |  |  | Yes |  | Type II codebook with port selection is not supported |  | Type 3 |  | N.A. |  | N.A. |  |  |  | Note: simultaneously doesn’t mean in the same slot |  |  |  | Component-1: the candidate values for the max # of Tx port in one resource is {4, 8, 12, 16, 24, 32} The candidate value set of the max # of resources is: {from 1 to 64} The candidate value set of total # of ports (including both channel and NZP-CSI-RS based interference measurement) is: {from 2 to 256} Component-2, candidate values set for “Lx” is {2,3,4} Component-3, candidate values set: {wideband, wideband/subband} Optional with capability signaling |  |  |
|  | 2-51 | TRS (CSI-RS for tracking) |  | 1. Support TRS BW 2. TRS burst length (X), 3. Max # of TRS resource sets (per CC) UE is able to track simultaneously 4. Max # of TRS resource sets configured to UE per CC 5. Max # of TRS resource sets configured to UE across CCs |  | 2-50 |  | Yes |  | TRS is not supported |  | Type 1 |  | N.A. |  | N.A. |  |  |  |  |  |  |  | Component-1: candidate values set: {BWP, min(52,BWP), both} Component-2: candidate values {1,2} Component-3: Candidate value set: {1 to 8} Component-4: Candidate value set: {1 to 64} Component-5: Candidate value set: {1 to 128} For component 2: all values are mandatory supported For component 3: value 1 mandatory for FR1, value [2] mandatory for FR2 For component 4: value 1 mandatory for FR1, a larger value mandatory for FR2 For component 5: 1 TRS per CC mandatory for FR1, a larger value mandatory for FR2 |  |  |
|  | 2-54a | Simultaneous SRS Tx |  | 1. Maximum number of simultaneous transmitted SRS resources per CC at one symbol, FFS whether to break this FG into different SRS purposes and potentially adding total number of Tx ports |  |  |  | Yes |  | Only one SRS resource can be transmitted at a given time |  | Type 1 |  | No need |  | N.A. |  |  |  |  |  |  |  | Candidate Value Set: {1, 2, 3, 4} Optional with capability signaling |  |  |
|  | 2-55 | SRS Tx switch |  | 1. Support SRS Tx port switch, 2. Report whether the uplink TX switching impact to downlink receiving in a band, |  | 2-53 |  | Yes |  | SRS Tx Switch is not supported |  | Type 3 |  | N.A. |  | N.A. |  |  |  | Component-2 is agreed with conditioned to RAN4’s decision.— |  | RAN1/4 |  | Component-1 is a list of TRx pairs, candidates are {1T2R, 1T4R, 2T4R, 1T4R/2T4R} Component-2: Candidate value set:, {yes, no}, For FR1: 1T4R mandatory with capability signaling (for bands with 4Rx mandated) For FR2: optional with capability signaling |  |  |
|  | 2-56 | SRS carrier switch |  | 1. Report inter-cell switching time capability |  | 2-53 |  | Yes |  | SRS carrier switch is not supported |  | Type 1 |  | No need |  | N.A. |  |  |  | RAN4 reply LS, R1-1805817, includes candidate value sets |  | RAN1/4 |  | candidate values set is up to RAN4 optional with capability signaling |  |  |
| 4.UL control channel and procedure | 4-1 | Basic UL control channel |  | 1) PUCCH format 0 over 1 OFDM symbols once per slot 2) PUCCH format 0 over 2 OFDM symbols once per slot with frequency hopping as “enabled” 3) PUCCH format 1 over 4 – 14 OFDM symbols once per slot with intra-slot frequency hopping as “enabled” 5) One SR configuration per PUCCH group 6) HARQ-ACK transmission once per slot with its resource/timing determined by using the DCI 7) Multiplexing of SR and HARQ-ACK on a PUCCH 8) HARQ-ACK piggyback on PUSCH 9) Semi-static beta-offset configuration for HARQ-ACK |  |  |  | Yes |  |  |  | N.A. |  | N.A. |  | N.A. |  |  |  | RAN4 to check feasibility of frequency hopping for PUCCH formats for FR2 |  |  |  | Mandatory without capability signaling |  | Mandatory without capability signaling |
|  | 4-24 | PUCCH-spatialrelationinfo indication by a MAC CE per PUCCH resource |  |  |  |  |  | Yes |  |  |  | Type 4 |  | No need |  | No need |  |  |  |  |  | RAN1 |  | Optional with capability signaling |  | Optional or mandatory? |
| 5. Scheduling/HARQ operation | 5-1 | Basic scheduling/HARQ operation | 1) Frequency-domain resource allocation - RA Type 0 only and Type 1 only for PDSCH without interleaving - RA Type 1 for PUSCH without interleaving 2) Time-domain resource allocation - 1-14 OFDM symbols for PUSCH once per slot - Starting symbol, and duration are determined by using the DCI - PDSCH mapping type A with 7-14 OFDM symbols - PUSCH mapping type A and type B - For type 1 without dedicated RRC configuration and for type 0, 0A, and 2, PDSCH mapping type A with {4-14} OFDM symbols and type B with {2, 4, 7} OFDM symbols 3) TBS determination 4) Nominal UE processing time for N1 and N2 (Capability #1) 5) HARQ process operation with configurable number of DL HARQ processes of up to 16 6) Cell specific RRC configured UL/DL assignment 7) Dynamic UL/DL determination based on L1 scheduling DCI with/without cell specific RRC configured UL/DL assignment 8) Intra-slot frequency-hopping for PUSCH scheduled by Type 1 before RRC connection |  |  |  | Yes |  |  |  | N.A. |  | N.A. |  | N.A. |  |  |  | Note: If UE is configured with more than 8 HARQ processes, RAN4 continue to discuss the impact of 16 HARQ processes |  | RAN1 |  | Mandatory without capability signaling |  | Mandatory without capability signaling |  |
|  | 5-4 | Dynamic switching between RA Type 0 and RA Type 1 for PUSCH |  |  |  |  |  | Yes |  |  |  | Type 4 |  | No need |  | No need |  |  |  |  |  |  |  | Optional with capability signaling |  |  |
|  | 5-7 | Interleaving for VRB-to-PRB mapping for PDSCH |  |  |  |  |  | Yes |  |  |  | Type 4 |  | No need |  | No need |  |  |  | RAN1 will discuss this feature |  |  |  | Mandatory with capability signaling |  |  |
| 6. CA/DC, BWP, SUL | 6-1 | Basic BWP operation |  | 1) 1 UE-specific RRC configured DL BWP per carrier 2) 1 UE-specific RRC configured UL BWP per carrier 3) RRC reconfiguration of any parameters related to BWP 4) BW of a UE-specific RRC configured BWP includes BW of the initial DL BWP and SSB for Pcell[/PScell] and BW of the UE-specific RRC configured BWP includes SSB for Scell if there is SSB on Scell |  |  |  | Yes |  |  |  | N.A. |  | N.A. |  | N.A. |  |  |  | This feature should be mandatory without capability signaling for at least BWPs which is the same as the set of specified channel BW RAN4 may discuss other BW requirements. UE-specific RRC configured DL/UL BWP can have the same or different numerology from the initial active DL/UL BWP |  | RAN4 |  | [Mandatory without capability signaling] |  |  |
|  | 6-16 | Supplemental uplink |  | Initial access and RRC connected operation on SUL carrier (incl 7.5kHz configurable shift) 1) RACH, PUSCH, PUCCH, SRS operations in a band combination including SUL 2) Supplemental uplink with same numerology between SUL and non SUL carriers |  | 6-15 |  | Yes |  | The UE will not be able to access or operate on a SUL carrier UE will not be able to perform the RACH/PUSCH/PUCCH/SRS operation in a band combination including SUL |  | N.A. |  | N.A. |  | N.A. |  |  |  | This is conditioned on the support of SUL band combination(s). The band combination definition is up to RAN4. |  |  |  | Mandatory if UE support SUL band combination(s) |  |  |
|  | 6-18 | Supplemental uplink with dynamic switch |  | DCI based selection of PUSCH carrier |  | 6-16 |  | Yes |  |  |  | Type 3 |  | N.A. |  | N.A. |  |  |  | This is conditioned on the support of SUL band combination(s). The band combination definition is up to RAN4. |  |  |  | Optional with capability signaling |  |  |

2.1 On Rel-15 URLLC UE features

The structure of URLLC UE features are shown in the table below. The recommendations are shown in the tables (RAN WG recommendation column) with track change.

| Features | # | Feature group | Components | Prerequisite feature groups (listed in this sheet only) | Need for gNB to know whether the feature is supported by the UE (what happens if gNB does not know?) | Consequences if the feature is not supported by the UE | Type (see R2-1712078) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | RAN5 implication | Note | Responsible WG | RAN WG recommendation | TSG-RAN decision |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 9-1 | New CQI table | Support of new CQI table associated with BLER target 10^-5 |  | Yes | The new CQI table is not supported by the UE | Type 4 | No need | No need |  |  |  | Optional with capability signaling |  |
|  | 9-2 | New MCS table for dynamic scheduled PDSCH without using MCS-C-RNTI | Support of new MCS table (qam64LowSE) for dynamic scheduled PDSCH by DL grant in USS |  | Yes | The new MCS table for dynamic scheduled PDSCH is not supported by the UE | Type 4 | No need | No need |  |  |  | Optional with capability signaling |  |
|  | 9-3 | New MCS table for dynamic scheduled PDSCH using MCS-C-RNTI | Support of new MCS table (qam64LowSE) for dynamic scheduled PDSCH by DL grant with MCS-C-RNTI | 9-2 | Yes | The new MCS table cannot be indicated by DL grant with MCS-C-RNTI | Type 4 | No need | No need |  |  |  | Optional with capability signaling |  |
|  | 9-4 | New MCS table for DL SPS | Support of new MCS table (qam64LowSE) for DL SPS |  | Yes | The new MCS table for DL SPS is not supported by the | Type 4 | No need | No need |  |  |  | Optional with capability signaling |  |
|  | 9-5 | New MCS table for dynamic scheduled PUSCH without using MCS-C-RNTI | Support of new MCS table (qam64LowSE) for dynamic scheduled PUSCH (with and without transform precoding) by UL grant in USS |  | Yes | The new MCS table for dynamic scheduled PUSCH is not supported by the UE | Type 4 | No need | No need |  |  |  | Optional with capability signaling |  |
|  | 9-6 | New MCS table for dynamic scheduled PUSCH using MCS-C-RNTI | Support of new MCS table (qam64LowSE) for dynamic scheduled PUSCH (with and without transform precoding) by UL grant with MCS-C-RNTI | 9-5, 9-3 | Yes | The new MCS table cannot be indicated by UL grant with MCS-C-RNTI | Type 4 | No need | No need |  |  |  | Optional with capability signaling |  |
|  | 9-7 | New MCS table for PUSCH with configured grant | Support of new MCS table (qam64LowSE) for PUSCH with configured grant (with and without transform precoding) |  | Yes | The new MCS table for PUSCH with configured grant is not supported by the | Type 4 | No need | No need |  |  |  | Optional with capability signaling |  |