3GPP TSG RAN WG1 Meeting #94	R1-1808573

Gothenburg, Sweden, August 20th – 24th, 2018

Agenda item:		7.2.6.4

Source:	Nokia, Nokia Shanghai Bell

Title:	On the PDCCH limits on the number of CCEs for channel estimation and BDs for URLLC

Document for:		Discussion and Decision

# Introduction

In RAN#80, the URLLC L1 study item was approved [1]. The following PDCCH enhancements was included as one of the objectives:

URLLC L1 improvements (RAN1) for further improved reliability/latency and for other requirements related to the use cases identified,

- PDCCH enhancements. Study focus on Compact DCI, PDCCH repetition, increased PDCCH monitoring capability

- UCI enhancements. Study focus on Enhanced HARQ feedback methods (increased number of HARQ transmission possibilities within a slot), CSI feedback enhancements

In this contribution, we focus on the issue of increased PDCCH monitoring capability, more specifically, on the maximum number of CCEs and BDs that a UE can support. We will discuss how the Rel-15 UE capability significantly limits the performance of URLLC and propose options for new definition of capability to improve it in Rel-16.

# Issues

For URLLC, one feature to achieve low latency in NR is the support of multiple PDCCH monitoring occasions within a slot. This minimizes the scheduling delay when the data comes. It corresponds to Case 2 that has been specified for PDCCH in NR. Regarding the maximum number of BDs and CCEs for channel estimation in Case 2, the following agreements have been reached:

Agreements:

- Confirm the value for Case 1-2. X=0 and Y=0 for Case 2. No consensus on additional Case 2’.

| Max no. of PDCCH BDs per slot | SCS |  |  |  |
| --- | --- | --- | --- | --- |
|  | 15kHz | 30kHz | 60kHz | 120kHz |
| Case 1-1 | 44 | 36 | 22 | 20 |
| Case 1-2 | [44] |  |  | - |
| Case 2 | [44+X] | [36+Y] | [22+Y] | [20] |

Agreements:

- For Rel.15 December 2017 version of Case 2, number of CCEs for channel estimation per slot is {56, 56, 48, 32} CCEs for SCS {15kHz, 30kHz, 60kHz, 120kHz}

Basically the BD and CCE limits for Case 2 are exactly the same as for Case 1-1, despite of the fact that more monitoring occasions are configured within a slot and the candidates will be spread out in time in Case 2.

For URLLC, the more relevant SCSs for case 2 are 15kHz and 30kHz. As the slot duration for 15 and 30kHz SCS is 1ms and 0.5ms, respectively, it is important to allow multiple PDCCH monitoring occasions within a slot to shorten the scheduling delay in order to satisfy the overall delay target. It may even be useful for 60KHz SCS but that is considered as less critical.

From latency perspective, a reasonable assumption would be to allow at least 4 monitoring occasions within a slot for 15kHz SCS, and at least 2 monitoring occasions within a slot for 30kHz SCS.

Let us do some simple calculation to see what the current BD and CCE limits mean for URLLC operation.

Assume the UE monitors CSS only at the beginning of a slot. If we assume the UE monitors {4, 2, 1} candidates for AL={4, 8, 16} respectively (which is the default Type0 CSS configuration), that is 7 BDs, and 16 CCEs assuming CORESET#0 can fit either one AL16 candidate or two AL8 candidates. This would leave 37 BDs and 40 CCEs for USS, which are distributed among all the monitoring occasions within a slot. Note that the assumptions for CSS here is the most conservative, as there are other types of CSS. Moreover, the CSS configured via UE-specific RRC signalling can have more candidates and/or larger CORESET (which translates into more CCEs) leaving even less BDs and CCEs available for USS monitoring.

For URLLC, considering the high reliability requirement, AL8 and AL16 need to be supported properly. Table 1 summarizes the number of CCEs required in different cases. With 15kHz SCS and 4 monitoring occasions, a single AL16 candidate per monitoring occasion would require 64 CCEs in a slot, which is not possible to be supported with the current UE capability. Also note that we would want to support at least two candidates per monitoring occasion to accommodate one DL assignment and one UL grant. From Table 1, we can see that there are quite a few important cases which we cannot support, even when we use the very conservative assumptions on CSS. This clearly shows that the number of CCEs is a limiting factor and there is a strong need to increase the number if we want to support URLLC properly.

Observation 1: The current UE capability on the maximum number of CCEs for channel estimation per slot cannot support URLLC properly, so the number should be increased.

Table 1 Number of CCEs for channel estimation needed for different cases

|  | One candidate of AL8 | One candidate of AL16 | Two candidates of AL8 | Two candidates of AL16 |
| --- | --- | --- | --- | --- |
| 15kHz SCS, 4 monitoring occasions per slot | 32 | 64 | 64 | 128 |
| 30kHz SCS, 2 monitoring occasions per slot | 16 | 32 | 32 | 64 |

It is clear, that the number of CCEs for channel estimation is the most restrictive factor. In terms of the number of BDs, the issue is not as severe, but it still has impact on the blocking probability. This is especially a concern if a UE requires both eMBB and URLLC services, meaning that it will need to monitor other (larger) DCI formats for eMBB services. Without increasing the number of BDs, it means that the total number of BDs is to be split between eMBB and URLLC, which will certainly affect the blocking probability at least for eMBB (if we assume URLLC always takes priority).

As a simple comparison, LTE sTTI has added additional BD candidates when sTTI is introduced, instead of splitting the existing number. For a UE supporting sTTI, the UE supports an additional 6 BDs per subslot TTI (36 BDs per subframe), and 12 BDs per slot TTI (24 BDs per subframe). In short, LTE sTTI supports 12 (CSS) + 48 (USS, 1ms TTI, for UEs supporting UL MIMO) + 36 (USS, sTTI) = 96 BDs on a carrier, while NR case 2 supports 44 BDs, which is certainly a big gap.

A similar comparison can be made in terms of number of CCEs for channel estimation per subframe for LTE. In addition to LTE PDCCH monitoring (16 CCEs for CSS, up to 42 CCEs for USS), an sTTI UE will need to receive up to 16 SCCEs per occasion (5 SPDCCH occasions per subframe) for subslot TTI and one SPDCCH with up to 32 SCCEs for slot TTI. This would mean that e.g. a subslot TTI UE will need to perform all together up to 16 (CSS) + 42 (USS, PDCCH) + 5x 16 (SPDCCH) = 138 (S)CCEs within a 1ms subframe, compared to 56 CCEs per slot in case of NR. This surely will result in worse NR performance compared to LTE.

Observation 2: The current UE capability on the maximum number of BDs and the maximum number of CCEs for channel estimation for Case 2 in NR is much lower than for Rel-15 LTE sTTI.

# Potential Solutions

The current UE capability on the number of BDs and the number of CCEs for channel estimation is defined on a per-slot basis. If we simply increase this number, there is no restriction on how these numbers could be distributed in the slot. Theoretically that means all the BDs and CCEs can occur at the beginning of a slot, which would increase the UE complexity significantly because it does not allow the UE to take advantage of the pipelining. On the other hand, if we take into account the fact that the BDs will be distributed over time in practical scenarios, the processors for earlier BDs may already be available for later BDs, and the total number could be significantly increased without stressing UE implementation much. Thus, in order to alleviate the problem in UE implementation, it appears that we should adopt a different definition than the per-slot basis counting.

The current UE capability for Case 2 is defined as the following:

| SCS | 15 kHz | 30 kHz | 60 kHz | 120 kHz |
| --- | --- | --- | --- | --- |
| Number of BDs per slot | 44 | 36 | 22 | 20 |
| Number of CCEs per slot | 56 | 56 | 48 | 32 |

As we can see, e.g. a UE can process 36 BDs and 56 CCEs for channel estimation with 30 kHz SCS within 0.5 ms, while for 15kHz SCS it can process 44 BDs and still 56 CCEs within 1 ms. If we e.g. adopt a per-half-slot counting, practically speaking the numbers for 15kHz SCS can be potentially increased without affecting UE implementation much. Once the data samples are obtained, the amount of processing needed for a BD would be the same regardless of the SCS. So, for 15kHz SCS, the UE can probably process similar number of BDs and CCEs within 0.5 ms as 30kHz SCS. This also motivates the change of definition.

The simplest and most straightforward definition would be to define the number of BDs and CCEs at a finer time granularity, e.g. per half-slot or every 3-symbols. If we go for a simpler approach and use the half-slot definition, we could consider using the following the numbers as the starting point for discussion, where the number per half-slot is taken from the number per slot for next higher SCS:

Table 2 An example for the number of BDs and CCEs for Case 2

| SCS | 15 kHz | 30 kHz |
| --- | --- | --- |
| Number of BDs per half-slot | 36 | 22 |
| Number of CCEs per half-slot | 56 | 48 |

There may be certain monitoring occasions spanning across the half-slot boundary depending on the configuration. In this case, the counting can be done based on the starting symbol of the monitoring occasions, or if necessary, they can be counted in both half-slots. For the overbooking rules, the mapping can be done in each half-slot independently for simplicity.

If we go for a smaller granularity than half-slot, it would be difficult to define a fixed duration as 14 symbols can only be divided by 2 or 7. One possibility is to define a time window in terms of the number of symbols, and the number of BDs and CCEs within any sliding window should not exceed a certain limit. Any BD that partially falls into the window can be always counted, which should sufficiently address the issue of overlapping monitoring occasions. The time window e.g. can be defined as 2 or 3 symbols. However, this approach would significantly deviate from the current framework, and it can complicate the overbooking rules considering the sliding window.

Another option is to take an approach similar to LTE, where the numbers for slot-level monitoring and sub-slot-level monitoring are defined separately. Then for sub-slot-level monitoring, the numbers would still need to be defined at a finer granularity, as the per-slot definition would put unnecessarily high burden on the UE.

Proposal: Introduce a new definition for the number of BDs and CCEs for channel estimation that a UE can support for Case 2 with a sub-slot granularity at least for 15kHz and 30kHz SCS. Two options can be considered: (1) the simple extension of the current capability per slot to per-sub-slot (e.g. with half-slot granularity using Table 2); (2) capability per-sub-slot is defined in addition to the per-slot capability.

# Conclusions

In this contribution, we have discussed the limitation of the current UE capability of the number of BDs and CCEs for channel estimation in the context of URLLC with the following observations:

Observation 1: The current UE capability on the maximum number of CCEs for channel estimation per slot cannot support URLLC properly, so the number should be increased.

Observation 2: The current UE capability on the maximum number of BDs and the maximum number of CCEs for channel estimation for Case 2 in NR is much lower than for Rel-15 LTE sTTI.

We have proposed the following to address the issue:

Proposal: Introduce a new definition for the number of BDs and CCEs for channel estimation that a UE can support for Case 2 with a sub-slot granularity at least for 15kHz and 30kHz SCS. Two options can be considered: (1) the simple extension of the current capability per slot to per-sub-slot (e.g. with half-slot granularity using Table 2); (2) capability per-sub-slot is defined in addition to the per-slot capability.

| SCS | 15 kHz | 30 kHz |
| --- | --- | --- |
| Number of BDs per half-slot | 36 | 22 |
| Number of CCEs per half-slot | 56 | 48 |

# References

[1] RP-181477, “New SID on Physical Layer Enhancements for NR URLLC”, Huawei, HiSilicon, Nokia, Nokia Shanghai Bell, RAN#80, June 2018.