3GPP TSG RAN WG1 Meeting #94	R1-1809359

Gothenburg, Sweden, August 20th- 24th, 2018

Agenda Item:	6.1.6.3

Source:	Huawei, HiSilicon

Title:	Clarification on specification impact of no more than one DCI validation for UL SPS

Document for:	Discussion and decision

# Introduction

This contribution provides a text proposal to avoid misleading caused by a statement in section 8.0 of TS 36.213 [1] even though it was an RAN1 agreement of LTE HRLLC WI. However, it should be understood that the agreement does not necessarily imply RAN1 specification impact.

# Discussion

The following agreement was achieved:

Agreement:

For UL SPS repetition with subframe/slot/subslot operation with multiple UL SPS configurations on a serving cell for a given TTI length:

- Each UL SPS configuration is activated by individual SPS activation DCI.

- The UE is not expected to receive more than one activation DCI in a TTI.

The feature that a UE can be configured more than one UL SPS configuration for a serving cell was adopted for Rel-14 V2X and now for Rel-15 HRLLC as well. Rel-14 V2X UE is not expected to receive more than one uplink semi-persistent scheduling assignment in a given TTI that however was not reflected in RAN1 specifications and by default follows the normal procedure.

In fact, irrespective of it is UL grant or UL SPS validation, more than one DCI for UL grant or UL SPS validation is not expected by UE is the legacy normal procedure that has no RAN1 specification impact and will not be changed by the introduced feature of UL SPS with repetition in LTE HRLLC WI.

On the other hand, if keeping the statements as is for LTE HRLLC, it may mislead people to have the concern that if the normal procedure is changed as well to Rel-14 V2X, which is unexpected and should be avoided.

Therefore, it is proposed to delete the statement in section 8.0 of TS 36.213 [1] as the following text proposal.

Proposal 1: Endorse the following text proposal:

------------------------------------------------Start of Text Proposal to 36.213-----------------------------------------

## 8.0	UE procedure for transmitting the physical uplink shared channel

<Unchanged parts are omitted>

For a LAA SCell, a UE is not expected to receive more than one uplink scheduling grant for a subframe.

For a LAA SCell, the HARQ process ID shall be delivered to higher layers.

If a UE is configured with higher layer parameter shortTTI or shortProcessingTime, the HARQ process ID shall be delivered to higher layers.

<Unchanged parts are omitted>

------------------------------------------------End of Text Proposal to 36.213-----------------------------------------

# Reference

- 3GPP TS 36.213, (E-UTRA); Physical layer procedure (Release 15); V15.2.0 (2018-06).