3GPP TSG RAN WG1 Meeting #94	R1-1809366

Gothenburg, Sweden, August 20th- 24th, 2018

Agenda Item:	6.1.2.2.5

Source:	Huawei, HiSilicon, LG Electronics, Nokia, Nokia Shanghai Bell, ZTE, Ericsson

Title:	Corrections on RV determination for sPUSCH

Document for:	Discussion and decision

Introduction

In this contribution, we proposed the correction on the RV determination and scaling factor usage for sPUSCH.

Discussion

For sPUSCH, the RV is determined by RV field in 7-0A/B instead of MCS field. Therefore, we have made following proposal:

Proposal 1: Endorse the following text proposal:

------------------------------------------------Start of Text Proposal to 36.213-----------------------------------------

<Unchanged parts are omitted>

- 8.6	Modulation order, redundancy version and transport block size determination

To determine the modulation order, redundancy version and transport block size for the physical uplink shared channel, the UE shall first

-	for a cell that is

- a LAA SCell or

- configured with higher layer parameter shortProcessingTime and the PDCCH with CRC scrambled by C-RNTI corresponding to the PUSCH is in the UE-specific search space, or

- configured with higher layer parameter shortTTI and the associated DCI is of format 7-0A/7-0B,

read the “modulation and coding scheme” field () and “redundancy version” field (), otherwise read the "modulation and coding scheme and redundancy version" field () if the UE is a non-BL/CE UEs and read the "modulation and coding scheme" field () if the UE is a BL/CE UE, and

-	check the "CSI request" bit field, and

-	compute the total number of allocated PRBs () based on the procedure defined in Subclause 8.1, and

-	compute the number of coded symbols for control information.

- 8.6.1	Modulation order and redundancy version determination

<Unchanged parts in this subclause are omitted>

For a cell that is not a LAA SCell, and a non-BL/CE UE,

-	for subframe-PUSCH, if the UE is configured with higher layer parameter enable256QAM-r14, and if the PDCCH corresponding to the PUSCH transmission is located in UE specific search space with CRC scrambled by the C-RNTI, the UE shall useand Table 8.6.1-3 to determine the redundancy version (rvidx) to use in the physical uplink shared channel.

-	if higher layer parameter tpc-SubframeSet is configured, higher layer parameter subframeSet1-DCI-Format0=TRUE, the associated DCI is of format 0/0A/0B, and the subframe of the PUSCH belongs to uplink power control subframe set 1, or,

-	if higher layer parameter tpc-SubframeSet is configured, higher layer parameter subframeSet1-DCI-Format4=TRUE, the associated DCI is of format 4/4A/4B, and the subframe of the PUSCH belongs to uplink power control subframe set 1, or,

-	if higher layer parameter tpc-SubframeSet is configured, higher layer parameter subframeSet2-DCI-Format0=TRUE, the associated DCI is of format 0/0A/0B, and the subframe of the PUSCH belongs to uplink power control subframe set 2, or,

-	if higher layer parameter tpc-SubframeSet is configured, higher layer parameter subframeSet2-DCI-Format4=TRUE, the associated DCI is of format 4/4A/4B, and the subframe of the PUSCH belongs to uplink power control subframe set 2, or,

-	if higher layer parameter tpc-SubframeSet is not configured, higher layer parameter dci-Format0=TRUE, and the associated DCI is of format 0/0A/0B, or,

-	if higher layer parameter tpc-SubframeSet is not configured, higher layer parameter dci-Format4=TRUE, and the associated DCI is of format 4/4A/4B;

-	otherwise, the UE shall useand Table 8.6.1-1 to determine the redundancy version (rvidx) to use in the physical uplink shared channel.

For a LAA SCell and DCI format 0A/4A, the redundancy version (rvidx) to use in the physical uplink shared channel is given by .

For a LAA SCell and DCI format 0B/4B, the redundancy version (rvidx) to use in the physical uplink shared channel is given by .

For a serving cell, if the UE is configured with higher layer parameter

- shortProcessingTime if the PDCCH with CRC scrambled by C-RNTI corresponding to the PUSCH transmission is located in UE specific search space or

- shortTTI and if the associated DCI is of format 7-0A/7-0B,

the redundancy version (rvidx) to use in the physical uplink shared channel is given by .

------------------------------------------------End of Text Proposal to 36.213-----------------------------------------