3GPP TSG RAN WG1 Meeting #94	R1-1809386

Gothenburg, Sweden, 20th - 24th August, 2018

Agenda item:		6.1.2.2.4

Source:		Nokia, Nokia Shanghai Bell, LG Electronics, Ericsson, Huawei, HiSilicon

Title:	Correction to PUSCH/PDSCH processing skipping in case of DCI Format 7-X reception in 36.213 – TP#4

Document for:		Discussion and Decision

# 1	Introduction

The related envisioned needed changes can be summarized as follows:

| Reason for change: |  | The UE subframe TTI processing skipping description in 36.213 assumes the WUL/WDL window to have a value larger than 0, whereas in 36.331 the UE can indicate also the capability to not need to skip any PUSCH/PDSCH processing by indicating WUL/WDL =0. The related descriptions for PDSCH reception in 36.213 Sec. 7.1 and PUSCH transmission in 36.213 Sec. 8.0 need to be corrected. |
| --- | --- | --- |
| Summary of change: |  | Clarify, that the PDSCH processing skipping only applies in case of WDL>0 and the PUSCH processing skipping only applies in case of WUL>0. |
| Consequences if not approved: |  | Unclear/incorrect UE behavior, in case the UE indicates no need for PDSCH processing skipping by indicating WDL=0 or PUSCH processing skipping by indicating WUL=0. |
| Clauses affected: |  | 36.213 Sections 7.1 & 8.0 |

# 2	TP to 36.213 V15.2.0, Section 7.1

< Unchanged parts are omitted >

## 7.1	UE procedure for receiving the physical downlink shared channel

< Unchanged parts are omitted >

For a UE configured with higher layer parameter shortTTI, the UE may skip decoding any transport block(s) received in PDSCH transmissions signalled via PDCCH/EPDCCH with CRC scrambled by the C-RNTI/SPS C-RNTI and DCI Formats other than DCI Format 7-1 A/7-1B/7-1C/7-1D/7-1E/7-1F/7-1G in the last WDL subframes if the UE has detected PDCCH/SPDCCH associated with DCI Format 7-1 A/7-1B/7-1C/7-1D/7-1E/7-1F/7-1G, and if WDL>0 is indicated by skipSubframeProcessing capability [12]. If the UE skips decoding, the physical layer indicates to higher layer that the transport block(s) are not successfully decoded.

< Unchanged parts are omitted >

# 3	TP to 36.213 V15.2.0, Section 8.0

< Unchanged parts are omitted >

## 8.0	UE procedure for transmitting the physical uplink shared channel

The term "UL/DL configuration" in this Subclause refers to the higher layer parameter subframeAssignment unless specified otherwise.

Throughout this section, if the UE is configured with higher layer parameter shortTTI and the corresponding PDCCH/SPDCCH with DCI format 7-0A/7-0B is detected in a subslot, if the UE is configured for subslot uplink transmissions, is determined based on higher layer configuration from , otherwise. If subslot number n is in subframe N, subslot refers to subslot number in subframe .

For a given serving cell, if a UE is configured with higher layer parameter shortProcessingTime, the UE is not expected to receive

-	more than one uplink scheduling grants for an uplink subframe.

-	PDCCH in common search space with DCI format 0 in subframe n and PDCCH in User-specific search space with DCI format 0/4 in the same subframe n.

For a serving cell, and a UE configured with higher layer parameter ul-TTI-Length, the UE is not expected to transmit subframe-PUSCH

-	in a given subframe corresponding to PDCCH with uplink DCI format other than 7-0A/7-0B or without a corresponding PDCCH if the UE detects PDCCH/SPDCCH with uplink DCI format 7-0A/7-0B corresponding to a PUSCH transmission in the same subframe or if the UE transmits a slot/subslot-PUSCH without a corresponding PDCCH/SPDCCH. The UE shall transmit the HARQ-ACK response corresponding to the subframe-PUSCH using the slot/subslot-PUSCH (as defined in Subclause 7.3). The UE shall apply spatial HARQ-ACK bundling on the HARQ-ACK response

-	in case subslot-PUSCH is used

-	in case slot-PUSCH is used if the bundling is configured for the cell.

-	in a given subframe corresponding to PDCCH/EPDCCH with uplink DCI format other than 7-0A/7-0B received in subframe n if the UE detects PDCCH/SPDCCH with uplink DCI format 7-0A/7-0B in any subframe from subframe n+1 to subframe n+WUL corresponding to a PUSCH transmission, and if WUL>0 is indicated by skipSubframeProcessing capability [12],

-	in case of a collision between the subframe-PUSCH and slot/subslot-PUCCH. The UE shall transmit the HARQ-ACK response corresponding to the subframe-PUSCH using the slot/subslot-PUCCH (as defined in Subclause 7.3). The UE shall apply spatial HARQ-ACK bundling on the HARQ-ACK response

-	in case subslot-PUCCH is used

-	in case slot-PUCCH is used if the bundling is configured for the cell.

-	in case of a collision between the subframe-PUSCH, subframe-PUCCH, and slot/subslot-PUSCH when simultaneous PUSCH and PUCCH transmission is configured for the UE. The UE is also not expected to transmit subframe-PUCCH. The UE shall transmit the HARQ-ACK response corresponding to the subframe-PUCCH using the slot/subslot-PUSCH.

< Unchanged parts are omitted >