3GPP TSG RAN WG1 Meeting #94		R1-1809902

Gothenburg, Sweden, August 20th – 24th, 2018

Agenda Item:	7.2.7.2

Source:	ZTE

Title:	Consideration on Evaluation of connection density of IMT-2020 self-evaluation

Document for:	Discussion

Introduction

There is an approved Study Item on Self Evaluation towards IMT-2020 submission in the RAN#75 meeting, which mainly focuses on evaluated RAN technologies based on Rel-15 and beyond to satisfy all ITU-R IMT-2020 requirements including eMBB scenario.

At RAN#77 meeting, the general work plan of self evaluation is approved [1]. Moreover, “[ITU-R AH 01] Calibration for self-evaluation”[2] has been set up for discussion on calibration and collection of calibration results. It is observed that the calibration results are well aligned according to the results collected so far for InH_x, UMa_x and RMa_x in all IMT-2020 defined test environments.

Moreover, At RAN#80, an approved Way forward shown Urban Macro – mMTC should be evaluated by LTE [3]. In this contribution, some considerations on evaluation details and results on Connection Density of both NR and LTE will be shown by full-buffer system-level simulation followed by link-level simulation.

Full-buffer system-level simulation followed by link-level simulation

In the system-level part, the main difference between NB-IoT and NR is uplink bandwidth, which 180kHz is applied for obtaining NR UL SINR distribution, while 15kHz for NB-IoT’s in LTE.

NR

NR SINR distributions are shown in Figure1 by Full buffer system level simulation. However, in the part of link level simulation, NR link adoption is supported. Table 1 presents that NR can meet the IMT-2020 connection density target by 180kHz bandwidth.

Figure1 the Uplink SINR distribution of NR

Figure2 NR link spectral efficiency

| Urban Macro-mMTC | Configuration B, UMa_a |
| --- | --- |
| NR | 1050952 |

Table 1 Connection density NR

Observation1: NR can meet Connection Density in Urban Marco-mMTC test environment by full-buffer system-level simulation followed by link-level simulation.

LTE NB-IoT

In LTE side, NB-IoT SINR distributions are shown in Figure 2 by Full buffer system level simulation. However, in the part of link level simulation, 1, 2, 4, 8, 16 repetitions were simulated to reach link spectral efficiency for different SINR values, which comes from system level simulation’s results. A fixed TBS of 256 bits was simulated, which exactly matches the agreed packet size of 32 bytes. Table2 presents that NR meets the IMT-2020 connection density target by 180 kHz system bandwidth.

Figure3 the Uplink SINR distribution of NB-IoT

Figure4 NB-IoT link spectral efficiency

| Urban Macro-mMTC | Configuration B, UMa_a |
| --- | --- |
| NB-IoT | 2059600 |

Table 1 Connection density NB-IoT

Observation2: NB-IoT can meet Connection Density in Urban Marco-mMTC test environment by full-buffer system-level simulation followed by link-level simulation.

Conclusion

In this document, we provide our considerations on evaluation results of Connection Density towards IMT-2020 submission. Furthermore, both NR and NB-IoT connection density performance are shown for the full buffer evaluation procedure. It has been clearly shown that both two technologies with margin meets the requirement of 10^6 devices/km2.

Observation1: NR can meet Connection Density in Urban Marco-mMTC test environment by full-buffer system-level simulation followed by link-level simulation.

Observation2: NB-IoT can meet Connection Density in Urban Marco-mMTC test environment by full-buffer system-level simulation followed by link-level simulation.

References

[1] RP-171451, “New Study Item on Self Evaluation towards IMT-2020 Submission”, RAN#75, Ericsson et al.

[2] Report ITU-R M.2410 - Minimum requirements related to technical performance for IMT-2020 radio interface(s).

[3] RP-181306, “Way forward on on LTE evaluation towards 3GPP Updated Submission”, RAN#80, Huawei, Ericsson, Telecom Italia

- Appendix

Table A-1 UL system level configuration parameters for Connection Density

| Parameter | NR | NB-IoT |
| --- | --- | --- |
| System bandwidth | 180kHz | 15 kHz |
| Number of BS antenna elements per TRxP | 16Rx cross-polarized antenna, (M,N,P,Mg,Ng) = (8,1,2,1,1), (dH,dV) = (N/A, 0.8)λ |  |
| Number of TXRU per TRxP | 2TXRU, (Mp,Np,P,Mg,Ng) = (1,1,2,1,1) |  |
| Number of UE antenna elements | 1Tx |  |
| Number of TXRU per UE | 1TXRU |  |
| UE antenna element pattern | Omni-directional |  |
| Channel model variant | Alt. 1: Channel model A |  |
| TRxP per site | 3 |  |
| Mechanic tilt | 90° in GCS (pointing to horizontal direction) |  |
| Electronic tilt | 99° for configuration A 93° for configuration B |  |
| TRxP boresight | 30 / 150 / 270 degrees |  |
| UT attachment | Based on RSRP (formula (8.1-1) in TR36.873) from port 0 |  |
| Wrapping around method | Radio distance based wrapping |  |
| Minimum distance of TRxP and UE | 10 m |  |
| Polarized antenna model | Model-2 in TR36.873 |  |
| System layout | 7 sites, 3 sectors per site |  |
| UL Power control SINR target | P0= -110，Alpha=1 |  |

Table A-2 Link level configuration parameters for Connection Density

| Parameter | NR | NB-IoT |
| --- | --- | --- |
| Physical channel | PUSCH | NPUSCH F1 |
| Channel bandwidth | 180 kHz | 15 kHz |
| Subcarrier spacing | 15 kHz | 15 kHz |
| TBS | 32, 42, 48, 64, 80, 104, 144, 168, 184 bits | 256 bits |
| Modulation | QPSK | QPSK/BPSK |
| # Resource units | - | 2, 3, 4, 5, 6, 8, 10 |
| # Repetitions | - | 1, 2, 4, 8, 16 |
| Channel estimation | Realistic | Realistic |
| SNR range | -20…15dB | -20…15 dB |
| Channel model | NLOS: TDL-C | NLOS: TDL-C |