TSG-RAN WG1 #94	R1-1809977

Gothenburg, Sweden, August 20 – 24, 2018

Source:	Ericsson

Title:	Outcome of offline session on data rates and TB size limitation

Agenda Item:	7.1.3.3

Document for:	Discussion and Decision

Offline proposal:

- Take the following as a working assumption:

A UE is not required to handle overlapped PDSCH/PUSCH transmissions not fulfilling the following condition at all points in time:

j=0N-1C'j.Kr,jDj≤DataRate

- Cj' is the number of scheduled code blocks of the transport block in the j-th PDSCH/PUSCH (from 212)

- Kr,j is the number of  bits in j-th code block (from 212)

- Dj is the duration of the i-th PDSCH/PUSCH ( = #symbols * symbol duration )

- where the maximum aggregate data rate (DataRate) is computed by the approximate date rate given by Subcluase 4.1.2 in [13, TS 38.306] based on

  - Alt 1 (Apply per CC):

    - Per CC, N = 1

  - Alt 2 (Apply per band per CG):

    - N is number of configured carriers in the band per CG

  - Alt 3 (Apply per CG):

    - N is the number of configured NR carriers in that  CG

  - Alt 4 (Apply per band combination)

    - N is the number of configured NR carriers

  - Alt 5 (Apply per frequency range per CG):

    - N is the number of configured NR carriers of the frequency range of the CG

- The PDSCHs/PUSCHs for  j = 0 to N-1 overlapping or partially overlapping PDSCH transmission

- In addition to limiting the number of information bits as above, constraints on the number of coded bits may be needed.

- Symbol duration = , where  is the numerology for the PDSCH/PUSCH