3GPP TSG RAN WG1 Meeting #94			R1-1810002

Gothenburg, Sweden, August 20th – August 24th, 2018

Title:	LS on bandwidth configuration for initial BWP

Release:	Rel-15

Work Item:	NR_newRAT-Core

Source:	RAN1

To:	RAN2

CC:

Contact Person:

#### Name:	Pei-Kai Liao

E-mail Address:	pk.liao@mediatek.com

1. Overall Description:

RAN1 thanks RAN2’s reply LS on bandwidth configuration for initial BWP. For the support of both Option #1 and Option #2 in R15 NR, RAN1 achieves the following agreements.

Agreements: - Modify the agreements in RAN1#92bis as follows. - When monitoring for DCI in a BWP, the size of DCI format 0-0/1-0 is given by - For format 0-0/1-0 (regardless of RNTI) in CSS, the size is given by the initial DL BWPCORESET#0 - For format 0-0/1-0 in USS, the size is given by the active BWP as long as the DCI size budget is fulfilled - Otherwise, for format 0-0/1-0, the size is given by the initial DL BWPCORESET#0 - For DCI format 1-0 in CSS with P-RNTI, SI-RNTI, RA-RNTI, C-RNTI, CS-RNTI, or TC-RNTI: - the RB numbering for the scheduled PDSCH starts from the lowest RB in the CORESET the DCI is received in - the maximum number of RBs possible to indicate in the DCI is given by the size of the initial DL BWPCORESET#0. - Payload sizes for 2-2 and 2-3 are padded (if needed) to match the size of formats 0-0/1-0 as defined by the initial BWP in CSS - Modify the agreements in RAN1#91 as follows if Option #2 is supported in PCell - CORESET configured by RMSI is confined within the initial DL BWPthe bandwidth of CORESET#0 - For PCell, the initial DL BWP can be configured in SIB1 to be the same as or different with the initial DL BWP as initially defined by CORESET#0 - The initial DL BWP configured in SIB1 includes the bandwidth of CORESET#0 - If the initial DL BWP configured by SIB1 is different with the initial DL BWP as initially defined by CORESET#0, the configuration of the initial DL BWP configured by SIB1 is applicable after the initial access - For PSCell and SCell, the initial DL BWP of a cell can be configured to be the same as or different with the initial DL BWP as defined by CORESET#0 of the cell - The initial DL BWP of a cell includes the bandwidth of CORESET#0 of the cell - Note: RAN1 assumes that the above agreements related to PCell, PSCell and SCell have no RAN1 specification impact and will be captured in RAN2 specs

2. Actions:

To RAN2 group

ACTION:

RAN1 respectfully asks RAN2 to capture the above agreements related to PCell, PSCell and SCell in RAN2 specs.

3. Date of Next RAN1 Meetings:

RAN1#94bis	8th – 12th October 2018	Chengdu, China

RAN1#95	12th – 16th November 2018	Spokane, USA