3GPP TSG RAN WG1 Meeting #94bis															R1-1810780

Chengdu, China, October 8th – 12th, 2018

Source:	Intel Corporation

Title:	On QoS Management for NR V2X Communication

Agenda item:	7.2.4.4

Document for:	Discussion and Decision

Introduction

At the RAN#80 meeting, the study item on NR V2X was approved [1]. Study of technical solutions for QoS management over Uu and PC5 radio-interfaces is one of the study item objective.

- NR V2X SI Objective # 5 QoS management [RAN1, RAN2]: - Study technical solutions for QoS management of the radio interface (including both Uu and sidelink) used for V2X operations based on input from SA2

At the RAN1#94 WG meeting, the following agreements were made by RAN1 on QoS aspects for NR-V2X work:

- RAN1#94 Agreements - From RAN1 perspective, at least the following QoS-related parameters relevant to physical layer studies are considered: - Priority, Latency, Reliability

This contribution is a revision of our previous submission in [5]. Here we remove content that was already agreed by RAN1 and continue analysis of the impact of QoS management on physical layer, while our views on other NR-V2X design aspects are summarized in our companion contributions [6]-[14]. It should be noted that proper support of QoS for NR V2X services requires consideration at each layer and thus requires coordinated work across multiple 3GPP RAN and SA working groups.

General Description of QoS Management Problem

The eV2X services are characterized by diverse set of communication requirements in terms of payload, transmission rate, data rate, latency, reliability, minimum communication range. Depending on eV2X application different technical characteristics may need to be satisfied. For instance, traffic properties and service characteristics may significantly depend on automation level in advanced driving applications: 0 - no automation, 1 - driver assistance, 2 - partial automation, 3 - conditional automation, 4 - high automation, 5 - full automation. Different automation levels require different QoS levels in terms of eV2X communication performance.

QoS Handling for LTE-V2X Uu Interface

When LTE-Uu is used for V2X, messages can be delivered via Non-GBR bearer as well as GBR bearer. There are several standardized QCI values defined in [2] for unicast delivery (QCI 3 and QCI 79) and over broadcast (MBMS) bearers (QCI 75). The QCI values describe the following QoS parameters to treat traffic forwarding:

Resource Type (GBR or Non-GBR);

Priority;

Packet Delay Budget;

Packet Error Loss Rate;

Maximum Burst Size (for some GBR QCIs);

Data rate Averaging Window (for some GBR QCIs).

QoS Handling for LTE-V2X PC5 Interface

When LTE-PC5 is used for the transmission of V2X messages, the following traffic characteristics are taken into account in case of network scheduled and UE autonomous resources allocation modes [2]:

ProSe Per-Packet Priority (PPPP)

  - PPPP has a value range of 1 to 8 where the higher value represents the lower priority. The application layer is expected to set PPPP for each V2X message and pass it to lower layers. The mapping of application layer V2X message priority to PPPP is configured on the UE. The setting of the PPPP value may reflect the latency required.

ProSe Per-Packet Reliability (PPPR)

  - PPPR has a value range of 1 to 8, where the higher value represents the lower reliability requirement for that message. The UE may be configured with a list of V2X services, e.g. PSID or ITS-AIDs allowed to use the specific PPPR value.

The main difference between LTE Uu and LTE PC5 QoS models is that Uu QCI based model is “connection oriented” and requires establishment of radio-bearer whereas the main motivation beyond PC5 QoS model is to enable “connectionless” operation for broadcast PC5 transmissions where QoS handling is expected on a per-packet basis.

NOTE: RAN2 have kept the notion of “radio bearers” for modelling of PC5 communications, however, in contrast to Uu radio bearers, there is no signalling used for establishment or release of “PC5 radio bearers”.

QoS Handling for NR Uu Interface

In NR, similar QoS characteristics are defined to describe the packet forwarding treatment between the UE and the UPF [3]:

Resource Type (RT including GBR, Delay critical GBR or Non-GBR);

Priority Level (PL);

Packet Delay Budget (PDB);

Packet Error Rate (PER);

Averaging Window (AW);

Maximum Data Burst Volume (MDBV - for Delay-critical GBR resource type only).

Open Aspects

From radio layer perspective, the 5G QoS characteristics should be understood as guidelines for 3GPP radio access. Therefore 5QIs may not be directly visible at physical layer, rather can affect UE radio-layer behavior aiming to support given QoS attribute. Based on analysis, we see that at least the following open aspects need to be resolved with respect to QoS support for eV2X use cases considering NR Uu and PC5 interfaces:

Whether existing NR QoS characteristics are sufficient for eV2X services delivered over NR Uu interface?

Whether existing NR QoS characteristics can be re-used for eV2X services delivered over NR PC5 interface?

Whether existing LTE PC5 QoS framework is applicable for eV2X services delivered over NR PC5?

QoS impact on NR Uu and NR PC5 radio layers and physical layer aspects in particular?

QoS support on NR PC5 in case of gNB controlled and UE autonomous resource allocation modes?

The first two questions require joint discussion and inputs from RAN2 and SA2 WGs and can be addressed at a later stage. With respect to QoS model we think it is out of RAN1 scope and should be decided by RAN2/SA2.

  - The NR V2X QoS model, i.e. per-packet QoS (PPPP and PPPR) or concept of QoS flows is to be decided by RAN2/SA2

Therefore in the next sections of this contribution, we mainly focus on the last two bullets and analyse potential QoS implications on NR physical layer design. In particular, we do see the following V2X specific challenges with respect to QoS support of eV2X services:

Node mobility and dynamic radio-propagation environment that complicates support of the fixed QoS level and may eventually require some multi-layer mechanism for QoS adaptation.

Intra and inter-UE handling of multiple eV2X services with different QoS in distributed architecture over NR PC5 interface.

Intra and inter-UE handling of multiple eV2X services with different QoS in centralized architecture over NR Uu interface.

eV2X QoS Support for NR Uu Interface

Applicability of NR Uu QoS for eV2X on Uu and PC5 Links

It needs to be discussed whether NR 5QI characteristics are sufficient for eV2X services delivered over NR Uu. From physical layer perspective, the existing set of NR Uu QoS parameters such as: RT, PL, PDB, PER, AW, MDBV are sufficient for physical layer to properly cover and treat majority of eV2X requirements, except minimum required communication range and transmission rate [4]. In Table 1, we provide linkage between eV2X requirements and related QoS parameters.

Table 1: eV2X requirement vs 5QI QoS parameters

| eV2X Requirement | Relevant QoS parameter(s) from 5QI |
| --- | --- |
| Payload | Maximum Data Burst Volume (MDBV) |
| TX Rate | Resource Type (RT) 2) |
| Max. end-to-end latency | Packet Delay Budget (PDB) |
| Reliability | Packet Delay Budget (PDB) + Packet Error Rate (PER) |
| Data rate | Maximum Data Burst Volume (MDBV) + Packet Delay Budget or Resource Type (GBR) + Guaranteed Flow Bit Rate (GFBR)2) |
| Min. required communication range | NA |

1): Resource Type – assume that GBR can be characterized by message rate

2): GFBR – is a part of 5G QoS profile

Based on above analysis, we can see that “TX Rate” is not explicitly represented by 5QI. At the same time if traffic is periodic, this knowledge can be utilized at the radio-layer to improve radio-resource management. The periodicity of traffic generation may not be viewed as a QoS attribute rather than be a parameter characterizing traffic property.

In addition, there is no relevant QoS attribute in 5QI characterizing communication range. From physical layer perspective, communication range cannot be directly considered as QoS attribute for eV2X traffic. As an alternative, the link budget or radiated energy per information bit can be potentially considered to ensure reliable performance at certain radio-distance in interference-free conditions. The latter can be controlled through multiple physical layer mechanisms including transmit power as well as resource allocation and does not seem to be a traffic QoS attribute rather than radio-layer performance target / indicator.

  - The existing set of 5QI parameters does not explicitly cover eV2X requirements on transmission rate and minimum required communication range

  - From physical layer perspective, the existing set of 5QI parameters can be used to handle eV2X services on Uu and PC5 links.

  - It needs to be further studied if 5QI set of QoS parameters needs to be extended to cover transmission rate and minimum required communication range

  - Additional 5QI values can be introduced to reflect eV2X QoS traffic attributes if needed (up to RAN2 and SA2)

- In general information on communication range may be useful from overall system perspective. For instance if target communication range is small, UE may reduce TX power or optimize MCS to facilitate more spectrum efficient transmission from system perspective. At the same time, additional more complete information on traffic is beneficial for more efficient reservation of resources. Therefore, we think that RAN1 need to consult with SA2, whether this information can be derived from upper layers.

  - Ask SA2 whether upper layers can provide information to represent minimum required communication range and provide additional data on transmission rate (e.g. period / packet size or statistical information) for NR–V2X communication

NR Uu QoS Support for eV2X

The mechanism how to support QoS over NR Uu link is a network implementation issue and can be handled by different resource allocation, admission control, scheduling strategies, etc. The NR Uu radio-layer specification defines sufficient set of tools for proper handling of eV2X services including delay critical traffic. Therefore, from physical layer perspective, we do not see additional work needed to manage QoS over NR Uu for eV2X use cases. On the other hand it needs to be discussed whether any additional assistance information on eV2X traffic characteristics need to be provided by UE to gNB in order to assist in UL scheduling.

  - Do not introduce additional physical layer support to handle QoS for eV2X services on NR Uu link

  - FFS if additional UE assistance information is needed to characterize eV2X UL traffic (e.g. unicast/groupcast or broadcast)

eV2X QoS Support for NR PC5 Interface

NR PC5 QoS Support for eV2X – Physical Layer Aspects

Resource Type

Resource type indicates whether guaranteed bit rate (GBR), delay critical GBR or non-GBR traffic can be supported. The eV2X traffic is typically either delay-critical GBR or GBR. The guarantee of certain bit rate can be feasible from TX perspective if there is no resource constraints or medium congestion. From physical layer perspective the resource type considerations may affect UE behavior in terms of resource reservation, resource selection and channel access procedures. Another possible consideration here is potential switching between resources or operating mode, e.g. gNB controlled and UE autonomous resource selection mode in case of medium congestion.

Priority Level

At the previous RAN1 WG meeting it was agreed that priority is relevant to physical layer studies. Considering that various eV2X applications with different traffic QoS attributes can share the same spectrum the priority indication can be used for the purpose of resource pre-emption, transmission pre-emption (packet drop), and admission control mechanism at PC5 radio-layers. The priority level can also affect UE behaviour in terms of channel selection and channel access procedures where for example different procedures or configurations can be used to determine UE behaviour for resource selection or sensing. For instance, sidelink transmissions with different priority levels may have different channel access granularity (opportunities) providing more favourable radio-access conditions to higher priority transmissions. Another priority-specific examples are radio-layer procedures for pre-emption of low priority transmissions or resource reservation options (e.g. possibility to reserve/occupy more resources). In summary, the priority indication can be used to handle intra-UE and inter-UE traffic conflicts by providing more favoured radio-layer conditions to higher priority traffic.

  - Priority information is used to handle intra and inter UE transmission conflicts

  - Priority information signaled over the air and utilized in channel access and resource selection procedures

Packet Delay Budget

At the previous RAN1 WG meeting it was agreed that latency is relevant to physical layer studies. The PDB can be used to support the dynamic configuration of scheduling and link layer functions including settings of resource selection windows, channel access parameters or transmission configurations (e.g. number of retransmissions or resource specific configurations). The PDB clearly has significant impact on radio-layer UE behavior given that radio-layers need to make sure that packet is sent reliably within a given time budget. The PDB in combination with PER metrics can affect UE decisions in terms of amount and structure of resources to be used for transmission in order to control the level of reliability as well as e.g. settings of radio-layer feedback mechanism if latter are defined/utilized.

  - Latency budget information is used in channel access and resource selection procedures at the transmitter side (e.g. to determine window for resource selection)

Packet Error Rate (Reliability)

From physical layer perspective, the PER mainly affects radio-link adaptation procedures and resource selection mechanisms. For eV2X use cases sophisticated link level adaption based on CSI may be quite problematic due to mobility and fast channel variation. On the other hand, the intelligent channel access and resource selection procedures are important to avoid collisions and thus improve link and system PER performance in non-heavily congested scenarios.

  - Reliability information is used to derive number of retransmissions, MCS level / transmission format for sidelink shared and control channel as well as to guide resource selection decisions and transmit power decisions (e.g. to ensure minimum energy per information bit)

    - For instance, minimum bounds on values for certain transmission parameters can be set

Averaging Window

Averaging window represents the duration over which the guaranteed bit rate is evaluated. It is applicable only to guaranteed bit rate flows.

Maximum Data Burst Volume

The MDBV denotes the largest amount of data to serve within PDB. The MDBV normalized by PDB gives an estimate of the data rate that has to be achieved at PDB time interval and thus together these two metrics can reflect the maximum average data rate over PDB interval. The knowledge of data rate is important for selection of the specific transmission scheme and transmission resources.

In radio-layers, for proper reservation of resources it is desirable to know the variation of packet size and packet arrival rate/statistics. This information may be in general obtained by UE implementation or provided by upper layers.

  - Ask SA2 if traffic characteristics such as packet size and packet arrival statistics can be provided for eV2X services

## Considerations on gNB-Controlled and UE-Autonomous Mode

In case of UE-autonomous mode, when UE selects sidelink resources autonomously QoS attributes will guide UE transmission behaviour in terms of channel access, sensing and resource selection, congestion control procedures, etc. In general, specific details of many procedures may be left up to UE implementation.

In case when sidelink resources are controlled by gNB, UE may need to inform gNB on sidelink traffic QoS indicators and provide traffic assistance information, so that gNB can properly schedule sidelink transmissions and allocate sidelink resources for transmission. For instance, similar to LTE-V2X UE may be able to provide assistance information characterizing eV2X traffic attributes or UE recommendations for scheduling sidelink resources.

  - UE reports QoS traffic information to gNB in order to facilitate proper scheduling of gNB controlled sidelink transmissions

## Congestion Control Considerations

It is important to note that depending on medium congestion and radio-layer propagation characteristics, the given levels of QoS may not be satisfied. For eV2X use cases, it is important to support priority handling mechanisms and proper congestion control schemes to make sure that high priority traffic is reliably delivered to intended recipients. Therefore cross-layer mechanisms with fast QoS adaptation are important for eV2X services.

From radio-layer perspective, the QoS framework can also affect congestion control mechanisms if congestion control functions are implemented at radio-layers. In distributed architecture, the goal of congestion control would be to enable fair usage of resources among UEs by taking into account traffic attributes and especially priority of transmissions.

  - Introduce sidelink radio-layer measurements to facilitate QoS adaptation and congestion control

Conclusions

In this contribution, we have analyzed handling of QoS for NR Uu and PC5 links for eV2X services from physical layer perspective.  In our view, final decisions on the specific QoS model to be used for eV2X services on NR Uu and PC5 links are beyond RAN1 scope and should be further discussed by RAN2 and SA2 WGs. Our analysis shows that for delivery of eV2X services over NR Uu link the existing set of QoS attributes represented by 5QI values can be sufficient and no additional L1 enhancements are needed for NR Uu interface related to QoS framework. However if for Uu link we do not see additional impact on physical layer design, the impact on PC5 physical layer is rather significant starting from proper definition of the sidelink physical structure, resource allocation, channel access and congestion control procedures. Based on analysis we have following proposals:

  - The NR V2X QoS model, i.e. per-packet QoS (PPPP and PPPR) or concept of QoS flows is to be decided by RAN2/SA2

  - Ask SA2 whether upper layers can provide information to represent minimum required communication range and provide additional data on transmission rate (e.g. period / packet size or statistical information) for NR–V2X communication

  - Do not introduce additional physical layer support to handle QoS for eV2X services on NR Uu link

  - FFS if additional UE assistance information is needed to characterize eV2X UL traffic (e.g. unicast/groupcast or broadcast)

  - Priority information is used to handle intra and inter UE transmission conflicts

  - Priority information signaled over the air and utilized in channel access and resource selection procedures

  - Latency budget information is used in channel access and resource selection procedures at the transmitter side (e.g. to determine window for resource selection)

  - Reliability information is used to derive number of retransmissions, MCS level / transmission format for sidelink shared and control channel as well as to guide resource selection decisions and transmit power decisions (e.g. to ensure minimum energy per information bit)

    - For instance, minimum bounds on values for certain transmission parameters can be set

  - Ask SA2 if traffic characteristics such as packet size and packet arrival statistics can be provided for eV2X services

  - UE reports QoS traffic information to gNB in order to facilitate proper scheduling of gNB controlled sidelink transmissions

  - Introduce sidelink radio-layer measurements to facilitate QoS adaptation and congestion control

References

- RP-181480, “New SID: Study on NR V2X”, Vodafone, La Jolla, USA, June 11th -14th, 2018

- TS 23.285 “Architecture enhancements for V2X services”, V15.1.0

- TS 23.501 “System Architecture for the 5G System; Stage 2”, V15.2.0

- 3GPP TS 22.186, “Service requirements for enhanced V2X scenarios”, 15.3.0

- R1-1808701, “QoS management for NR V2X communication”, Intel Corporation, Gothenburg, Sweden, August, 2018.

- R1-1810772, “Support of sidelink unicast, groupcast and broadcast modes for NR V2X communication”, Intel Corporation, Chengdu, China, October, 2018.

- R1-1810773, “Sidelink physical layer structure and procedures for NR V2X communication”, Intel Corporation, Chengdu, China, October, 2018.

- R1-1810774, “Sidelink synchronization for NR V2X communication”, Intel Corporation, Chengdu, China, October, 2018.

- R1-1810775, “Sidelink resource allocation mechanisms for NR V2X communication”, Intel Corporation, Chengdu, China, October, 2018.

- R1-1810776, “Remaining aspects of eV2X evaluation methodology and assumptions”, Intel Corporation, Chengdu, China, October, 2018.

- R1-1810777, “Analysis of NR Uu support for advanced V2X use cases”, Intel Corporation, Chengdu, China, October, 2018.

- R1-1810778, “Enhancements of NR and LTE Uu link to control NR sidelink”, Intel Corporation, Chengdu, China, October, 2018.

- R1-1810779, “Enhancements of NR Uu link to control LTE sidelink”, Intel Corporation, Chengdu, China, October, 2018.

- R1-1810781, “Coexistence mechanisms for eV2X services”, Intel Corporation, Chengdu, China, October, 2018.