3GPP TSG RAN WG1 Meeting #94bis		R1-1811045

Chengdu, China, October 8th – 12th, 2018

Source:	CMCC

Title:	Updated evaluation results for downlink eMBB spectrum efficiency

Agenda Item:	7.2.7.2

Document for: 	Discussion

- Introduction

At RAN1#94 meeting, we submitted a preliminary evaluation results on DL spectral efficiency for Indoor-eMBB, Dense Urban-eMBB and Rural-eMBB test environments. In this contribution, we update the configurations of SRS  transmission period for eMBB test environments and provide the updated evaluation results of DL spectral efficiency. The evaluation assumptions are aligned with the section 2 of  offline discussion in RAN1#92 (see [1]).

- Test environments for eMBB DL spectral efficiency

According to Report ITU-R M.2410 (see [2]), the key minimum technical performance requirements for spectral efficiency are provided in Table 1.

Table 1. Minimum requirements for average spectral efficiency and 5th percentile user spectral efficiency

| Test environment | Downlink (bit/s/Hz/TRxP) |  |
| --- | --- | --- |
|  | Average spectral efficiency | 5th percentile user spectral efficiency |
| Indoor Hotspot – eMBB | 9 | 0.3 |
| Dense Urban – eMBB | 7.8 | 0.225 |
| Rural – eMBB | 3.3 | 0.12 |

NOTE 1 – This requirement will be evaluated under Macro TRxP layer of Dense Urban.

Report ITU-R M.2412 (see [3])  provides the evaluation configurations for eMBB test environments. In this contribution, the following evaluation configurations, as given in Table 2, are considered for downlink spectral efficiency evaluation.

TABLE 2. Evaluation configurations for downlink spectral efficiency evaluation

| Test env. | Evaluation configuration | Carrier frequency | ISD | Duplex scheme | Bandwidth | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| Indoor Hotspot – eMBB | Config. A | 4 GHz | 20 m | TDD | 100MHz |  |
| Dense Urban – eMBB | Config. A | 4 GHz | 200 m | TDD | 100MHz | Macro layer only |
| Rural - eMBB | Config. A | 700MHz | 1732 m | TDD | 20MHz |  |
|  | Config. B | 4 GHz | 1732 m | TDD | 100MHz |  |
|  | Config. C | 700MHz | 6000 m | TDD | 20MHz |  |

- Updated evaluation results

Based on the above discussion, initial evaluation results for average spectral efficiency and 5th percentile spectral efficiency are given in Table 3. The detailed evaluation assumptions can be found in Appendix.

TABLE 3. Updated DL spectral efficiency evaluation for TDD

| Test environment | Indoor Hotspot – eMBB | Dense Urban – eMBB | Rural - eMBB |  |  |
| --- | --- | --- | --- | --- | --- |
| Evaluation configuration | Configuration A | Configuration A | Configuration A | Configuration B | Configuration C |
| Carrier Frequency | 4 GHz | 4 GHz | 700MHz | 4GHz | 700MHz |
| System bandwidth | TDD: 100MHz | TDD: 100MHz | TDD: 20MHz | TDD: 100MHz | TDD: 20MHz |
| Subcarrier spacing | 30 kHz | 30 kHz | 15kHz | 30 kHz | 15kHz |
| Guard band ratio (see [4]) | TDD: 1.72% (for 100 MHz) | TDD: 1.72% (for 100 MHz) | TDD: 4.6% (for 20 MHz) | TDD: 1.72% (for 100 MHz) | TDD: 4.6% (for 20 MHz) |
| Antenna config. | 32T4R | 64T4R | 8T2R | 64T4R | 8T4R |
| MIMO mode | MU-MIMO with rank 4 adaptation per user; Maximum MU layer = 12 | MU-MIMO with rank 4 adaptation per user; Maximum MU layer = 12 | MU-MIMO with rank 2 adaptation per user; Maximum MU layer = 8 | MU-MIMO with rank 4 adaptation per user; Maximum MU layer = 12 | MU-MIMO with rank 2 adaptation per user; Maximum MU layer = 8 |
| SRS period | 10 slot (5ms) | 10 slot (5ms) | 5 slot (5ms) | 10 slot (5ms) | 5 slot (5ms) |
| DMRS overhead | 11.08% | 11.42% | 6.6% | 10.62% | 6.67% |
| Total overhead | 17.19% | 17.53% | 28.54% | 16.73% | 28.61% |
| Average spectral efficiency (bit/s/Hz/TRxP) | 10.48 | 16.73 | 4.14 | 13.04 | 4.50 |
| Req. of average spectral efficiency (bit/s/Hz/TRxP) | 9 | 7.8 | 3.3 | 3.3 | 3.3 |
| 5th percentile spectral efficiency (bit/s/Hz) | 0.31 | 0.53 | 0.12 | 0.33 | 0.12 |
| Req. of 5th percentile spectral efficiency (bit/s/Hz) | 0.3 | 0.225 | 0.12 | 0.12 | - |

Observation 1: NR with sub 6 GHz can fulfill DL spectral efficiency requirements in all eMBB test environments.

- Conclusions

In this contribution, we provide our initial evaluation results for spectral efficiency in eMBB scenario. We have the following observation.

Observation 1: NR with sub 6 GHz can fulfill DL spectral efficiency requirements in all test environments.

References

- R1-1803386, “Summary of offline discussion for IMT-2020 self evaluation”, RAN1 #92, Huawei, Athens

- Report ITU-R M.2410, “Minimum requirements related to technical performance for IMT-2020 radio interface(s)”, Nov. 2017.

- Report ITU-R M.2412, “Guidelines for evaluation of radio interface technologies for IMT-2020”, Nov. 2017.

- R1-1805927, “Considerations and evaluation results for IMT-2020 for eMBB spectral efficiency”, RAN1 #93, Huawei, Busan

Appendix: Evaluation assumptions

Table A-1. Evaluation assumptions for DL TDD

| Parameter |  |  | Value |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test environment | Indoor Hotspot – eMBB | Dense Urban – eMBB |  | Rural - eMBB |  |  |
| Evaluation configuration | Configuration A | Configuration A |  | Configuration A | Configuration B | Configuration C |
| Channel model | InH_A | UMa_A |  | RMa_A | RMa_A | RMa_A |
| ISD | 20 m (12TRxP) | 200 m |  | 1 732 m | 1 732 m | 6000 m |
| TDD frame structure | DDDSU; S(10DL:2GP:2UL) | DDDSU; S(10DL:2GP:2UL) |  | DDDSU; S(10DL:2GP:2UL) | DDDSU; S(10DL:2GP:2UL) | DDDSU; S(10DL:2GP:2UL) |
| Carrier Frequency | 4 GHz | 4 GHz |  | 700 MHz | 4GHz | 700 MHz |
| System bandwidth | TDD: 100MHz | TDD: 100MHz |  | TDD: 20MHz | TDD: 100MHz | TDD: 20MHz |
| Subcarrier spacing | 30 kHz | 30 kHz |  | 30 kHz | 30 kHz | 30 kHz |
| Symbols number per slot | 14 | 14 |  | 14 | 14 | 14 |
| Number of antenna elements per TRxP | 32Tx cross-polarized antenna | 192Tx cross-polarized antenna |  | 64Tx cross-polarized antenna | 192Tx cross-polarized antenna | 64Tx cross-polarized antenna |
|  | (M,N,P,Mg,Ng) = (4,4,2,1,1) | (M,N,P,Mg,Ng) = (12,8,2,1,1) |  | (M,N,P,Mg,Ng) = (8,4,2,1,1) | (M,N,P,Mg,Ng) = (12,8,2,1,1) | (M,N,P,Mg,Ng) = (8,4,2,1,1) |
| Number of TXRU per TRxP | 32TXRU, (Mp,Np,P,Mg,Ng) = (4,4,2,1,1) | 64TXRU, (Mp,Np,P,Mg,Ng) = (4,8,2,1,1) |  | 8TXRU, (Mp,Np,P,Mg,Ng) = (1,8,2,1,1) | 64TXRU, (Mp,Np,P,Mg,Ng) = (4,8,2,1,1) | 8TXRU, (Mp,Np,P,Mg,Ng) = (1,8,2,1,1) |
| Number of antenna elements per UE | 4Rx with 0°,90° polarization | 4Rx with 0°,90° polarization |  | 2Rx with 0°,90° polarization | 4Rx with 0°,90° polarization | 4Rx with 0°,90° polarization |
| Device deployment | 100% indoor | 80% indoor, 20% outdoor (in car) |  | 50% indoor, 50% outdoor (in car) | 50% indoor, 50% outdoor (in car) | 50% indoor, 50% outdoor (in car) |
|  | Randomly and uniformly distributed over the area | Randomly and uniformly distributed over the area under Macro layer |  | Randomly and uniformly distributed over the area | Randomly and uniformly distributed over the area | Randomly and uniformly distributed over the area |
| UE mobility model | Fixed and identical speed ¦v¦ of all UEs, randomly and uniformly distributed direction | Fixed and identical speed ¦v¦ of all UEs of the same mobility class, randomly and uniformly distributed direction |  | Fixed and identical speed ¦v¦ of all UEs of the same mobility class, randomly and uniformly distributed direction | Fixed and identical speed ¦v¦ of all UEs of the same mobility class, randomly and uniformly distributed direction | Fixed and identical speed ¦v¦ of all UEs of the same mobility class, randomly and uniformly distributed direction |
| UE speeds of interest | 3 km/h | Indoor users: 3 km/h Outdoor users (in-car): 30 km/h |  | Indoor users: 3 km/h; Outdoor users (in-car): 120 km/h | Indoor users: 3 km/h; Outdoor users (in-car): 120 km/h | Indoor users: 3 km/h; Outdoor users (in-car): 120 km/h |
| Transmit power per TRxP | TDD: 24 dBm | TDD: 44 dBm |  | TDD: 49 dBm | TDD: 49 dBm | TDD: 49 dBm |
| TRxP number per site | 1 | 3 |  | 3 | 3 | 3 |
| Mechanic tilt | 180deg in GCS (pointing to the ground) | 90deg in GCS (pointing to the horizontal direction) |  | 90deg in GCS (pointing to the horizontal direction) | 90deg in GCS (pointing to the horizontal direction) | 90deg in GCS (pointing to the horizontal direction) |
| Electronic tilt | 90deg in LCS | 105deg in LCS |  | 100deg in LCS | 100deg in LCS | 92deg in LCS |
| UT attachment | Based on RSRP (Eq. (8.1-1) in TR 36.873) from port 0 | Based on RSRP (Eq. (8.1-1) in TR 36.873) from port 0 |  | Based on RSRP (Eq. (8.1-1) in TR 36.873) from port 0 | Based on RSRP (Eq. (8.1-1) in TR 36.873) from port 0 | Based on RSRP (Eq. (8.1-1) in TR 36.873) from port 0 |
| Scheduling | MU-PF | MU-PF |  | MU-PF | MU-PF | MU-PF |
| MIMO mode | MU-MIMO with rank 4 adaptation per user; | MU-MIMO with rank 4 adaptation per user |  | MU-MIMO with rank 1 adaptation per user | MU-MIMO with rank 4 adaptation per user | MU-MIMO with rank 1 adaptation per user |
|  | Maximum MU layer = 12 | Maximum MU layer = 12 |  | Maximum MU layer = 8 | Maximum MU layer = 12 | Maximum MU layer = 8 |
| Guard band ratio (see [4]) | TDD: 1.72% (for 100 MHz) | TDD: 1.72% (for 100 MHz) |  | TDD: 8.2% (for 20 MHz) | TDD: 1.72% (for 100 MHz) | TDD: 8.2% (for 20 MHz) |
| BS receiver type | MMSE-IRC | MMSE-IRC |  | MMSE-IRC | MMSE-IRC | MMSE-IRC |
| CSI feedback | 5 slots period based on CSI-RS with delay | 5 slots period based on CSI-RS with delay |  | 5 slots period based on CSI-RS with delay | 5 slots period based on CSI-RS with delay | 5 slots period based on CSI-RS with delay |
| Precoder derivation | TDD: SRS based | TDD: SRS based |  | TDD: SRS based | TDD: SRS based | TDD: SRS based |
| Channel estimation | Non-ideal | Non-ideal |  | Non-ideal | Non-ideal | Non-ideal |
| Waveform | OFDM | OFDM |  | OFDM | OFDM | OFDM |
| PDCCH | 2 complete symbols within 20MHz | 2 complete symbols within 20MHz |  | 2 complete symbols within 20MHz | 2 complete symbols within 20MHz | 2 complete symbols within 20MHz |
| DMRS | Type II (dynamic) | Type II (dynamic) |  | Type II (dynamic) | Type II (dynamic) | Type II (dynamic) |
| SSB | 8 SSB / 20ms | 8 SSB / 20ms |  | 8 SSB / 20ms | 8 SSB / 20ms | 8 SSB / 20ms |
| CSI-RS (for CSI) | 8*4 ports with 10 slots period | 8*4 ports with 10 slots period |  | 8*2/4 ports with 10 slots period | 8*4 ports with 10 slots period | 8*2/4 ports with 10 slots period |
| CSI-RS (for IM) | N/A | N/A |  | N/A | N/A | N/A |
| SRS period | 10 slot (5ms) | 10 slot (5ms) |  | 5 slot (5ms) | 10 slot (5ms) | 5 slot (5ms) |
| TRS | 1 ports, bandwidth 50RB,burst length 2 slots 6 RE/PRB/20ms | 1 ports, bandwidth 50RB,burst length 2 slots 6 RE/PRB/20ms |  | 1 ports, bandwidth 50RB,burst length 2 slots 6 RE/PRB/20ms | 1 ports, bandwidth 50RB,burst length 2 slots 6 RE/PRB/20ms | 1 ports, bandwidth 50RB,burst length 2 slots 6 RE/PRB/20ms |