3GPP TSG-RAN WG1#94bis	R1-1811131

Chengdu, China, October 8th – 12th, 2018

Agenda item:		7.2.1.5

Source:	Nokia, Nokia Shanghai Bell

Title:	Complexity Analysis of MMSE-based Hard IC Receiver

Document for:		Discussion and Decision

# 1	Introduction

In this contribution, we analyze the complexity of the MMSE-based hard IC receiver. In RAN1#94 [1], the template for receiver complexity was agreed.

Agreements:

- The following table for computation complexity analysis of the receiver as the starting point, entries can be updated till RAN1#94bis.

Table I	Template of Receiver Computation Complexity breakup

| Receiver component | Detailed component | Computation in parametric number of usages, O(.) analysis, [impact factor] |  |  |
| --- | --- | --- | --- | --- |
|  |  | Receiver type 1 | Receiver type 2 | … |
| Detector | UE detection |  |  |  |
|  | Channel estimation |  |  |  |
|  | Rx combining, if any |  |  |  |
|  | Covariance matrix calculation, if any |  |  |  |
|  | Demodulation weight computation, if any |  |  |  |
|  | UE ordering, if any |  |  |  |
|  | Demodulation, if any |  |  |  |
|  | Soft information generation, if any |  |  |  |
|  | Soft symbol reconstruction, if any |  |  |  |
|  | Message passing, if any |  |  |  |
|  | Others |  |  |  |
| Decoder | LDPC decoding |  |  |  |
| Interference cancellation | Symbol reconstruction(Including FFT operations for DFT-S-OFDM waveform), if any |  |  |  |
|  | LLR to probability conversion, if any |  |  |  |
|  | Interference cancellation |  |  |  |
|  | LDPC encoding, if any |  |  |  |
|  | Others |  |  |  |

- The impact factor is to be estimated based on the analysis of computation, memory size, hardware and software implementation, etc.

  - If/How and which entries are to be combined/compared in order to get the total complexity estimate is FFS.

- Companies may provide the impact factor

  - The impact factor is for each cell

- The rows in the above table are subject to potential re-finement, e.g., adding new row(s), merge some rows, etc.

- Note: the numbers may or may not be a function of UL waveform

- FFS whether or not to add row(s) for memory blocks

In our companion contribution [4], we describe how the impact factor can be determined.

# 2	Overview of MMSE-based Hard IC Receiver

Figure 1 shows a block diagram of an MMSE-based Hard IC receiver. Let the received signal be expressed by:

yrx=Ax+n

Where, A=H1S1…HkSK. Hk is the channel estimate of user k, and Sk is the spreading code of user k. During the first iteration, the recovered signal yr is 0, and the residual signal yres=yrx. The output of the joint MMSE equalizer is given by:

x=AHA+σN2I-1AHyres

The output of the equalizer is demodulated and decoded, and the CRC of the decoded data is checked for each user. Users with a passing CRC, will have their signal reconstructed. Signal reconstructions involves:

- Re-encoding, re-modulation and re-spreading the decoded data.

- Applying the channel estimate to the re-encoded data to get an estimate of that user’s signal at the input to the receiver.

The recovered signal of users decoded correctly is subtracted from the received signal. This residual signal is used to receive the remaining users.

Figure 1: MMES-based Hard IC Receiver.

# 3	Complexity of MMSE-based Hard IC Receiver

In this section we analyze the complexity of an MMSE-based hard IC receiver.

## 3.1	Complexity of Detector

The detector is made up of the following components: UE detection, channel estimation, noise estimation, MMSE weight calculation, and equalization (application of the MMSE weights).

### 3.1.1	UE detection

UE detection is performed once per UE per received-slot. UE detection is performed on the DMRS symbols or on the preamble. The order of complexity of UE detection per UE is:

UE Detection Complexity ~ Nsc∙NsymbDMRS∙NRx

Where,

- Nsc is the number of DMRS subcarriers in one symbol.

- NsymbDMRS is the number of DMRS symbols.

- NRx is the number of receive antennas.

A simple UE detector can be a correlator with a reference signal, which requires Nsc∙NsymbDMRS∙NRx complex multiplications and Nsc∙NsymbDMRS∙NRx complex additions. The square of the amplitude of each antenna’s correlation output is summed, this is followed by a comparison against a threshold.

The number of UE detections per received slot equals the number of potential UEs that can transmit in that slot.

### 3.1.2	Channel estimation

Channel estimation is performed once per UE per received-slot. Channel estimation is performed on the DMRS symbols, and filtered (interpolated) across the data symbols. The order of complexity of channel estimation per UE is:

Channel Estimation Complexity ~ Nsc∙NsymbDMRS+Nsymbdata∙NRx

Where,

- Nsc is the number of DMRS/data subcarriers in one symbol.

- NsymbDMRS is the number of DMRS symbols.

- Nsymbdata is the number of DMRS symbols

- NRx is the number of receive antennas.

A simple channel estimation can be a multiplication of the received DMRS signal with a reference signal, reused from the UE detection, hence not included in the channel estimation row of Table 3, and then filtering in time domain to get the channel estimate at each data RE. Assuming two DMRS symbols are used for filtering, this requires 2∙Nsc∙Nsymbdata∙NRx complex multiplications and Nsc∙Nsymbdata∙NRx complex additions per UE.

The number of channel estimations per received slot equals the number of UEs received in a slot (NUE). This assumes that channel estimation is performed once for each UE and used in all the following iterations.

The channel estimates are buffered to be used in subsequent stages such as MMSE weight calculation and signal reconstruction. The memory for storing the channel estimates is: NUE∙Nsc∙Nsymbdata∙NRx Complex values.

### 3.1.3	Noise estimation

Noise estimation can be performed for all UEs in each iteration per iteration per received slot.

Noise Estimation Complexity ~ Nsc∙NsymbDMRS∙NRx∙NUE,avg

Where, NUE,avg is the average number of UEs per iteration.

The number of noise estimations per received slot equals the total number of iterations (Nitr).

### 3.1.4	MMSE weight calculation

The equation of the MMSE detector is:

x=AHA+σN2I-1AHyres

A is a matrix with size NRx.NSF×NUE,avg. Where, NRx is the number of receive antennas. NSF is the spreading factor and NUE,i is the number of users in iteration i. Nsymbdata is the number of data symbols and Nsc is the number of subcarriers allocated to the NOMA users. The MMSE weights are:

WMMSE=AHA+σN2I-1AH

An example for the calculation of the number of complex operations is given by Table 1.

Table 1: Computational complexity of MMSE detector.

| Function | Complex Operations |
| --- | --- |
| AHA+σN2I | Nsymbdata NscNSFNRx.NSF2NUE,i+NUE,i |
| AHA+σN2I-1 | Nsymbdata NscNSFNUE,i3/2 |
| AHA+σN2I-1AH | Nsymbdata NscNSFNRx.NSFNUE,i2 |
| Total | Nsymbdata NscNSFNRx.NSF2NUE,i+NRx.NSFNUE,i2+NUE,i32+NUE,i |

The order of computation complexity, with a large number of UEs is:

MMSE weight calculation Complexity ~ Nsymbdata NscNUE3NSF

The number of MMSE weight calculations per received slot equals the total number of iterations (Nitr).

The MMSE weights are buffered to be used in equalization. The memory for storing the MMSE weights is: NUE∙Nsc∙Nsymbdata∙NRx Complex values.

### 3.1.5	Equalization

Equalization applies the MMSE weights to the received signal:

x=WMMSEyres

MMSE weight calculation Complexity ~Nsymbdata NscNRxNUE,avg

The number of complex multiplication is: Nsymbdata NscNRxNUE,avg

The number of complex additions is: Nsymbdata NscNRxNUE,avg

The number of equalizations per received slot equals the total number of iterations (Nitr).

The output of the equalizer is buffered to be used in the decoder. The memory for storing the equalizer output is: NUE∙Nsc∙Nsymbdata∙NRxNSF Complex values.

## 3.2	Complexity of decoder

The computation complexity of the LDPC decoder is given by Table 2 [2]. Where, N is the code length, M is the number of parity bits, dv is the average variable degree of LDPC parity check matrix (PCM), dc is the average check degree of LDPC PCM.

Table 2: LDPC decoder computation complexity.

|  | LDPC (Sum-Product) [3] | LDPC (min-sum) [3] |
| --- | --- | --- |
| Additions | Imax*(2*N*dv + M*(2*dc-1)) | Imax*(2*N*dv + 2M) |
| MAX process/ Comparison | NA | Imax* (2*dc-1)*M |
| Look-up-table operations | Imax*M*dc | NA |

The number of LDPC decodes per received slot equals the average number of UEs decoded per iteration in a slot multiplied by the number of iterations (NUE.avg∙Nitr). LDPC is a well optimized block, that would be used for the different receiver architectures. We don’t include the impact factor of LPDC in Table 3, but use the number of usage when comparing the complexity of different receiver structures. It should be noted, however, that the impact factor could be different if the number of iterations of the LDPC decoder is different.

The output of the decoder is hard bits, which require a relatively small buffer. For simplification, we ignore this buffer.

## 3.3	Complexity of Interference Cancellation

Interference cancellation involves the following steps, as shown in Figure 2:

- Re-encoding the decoded signal, rate matching and scrambling.

- Modulation. We assume a look up table for the modulator.

- Spreading. We assume a complex multiply for the spreading

- Symbol reconstruction. We assume a multiplication by the channel estimate.

- Interference subtraction.

Figure 2: Interference Cancellation

### 3.3.1	Re-encoding

Re-encoding the decoded signal, rate matching and scrambling. This is done at the bit level, in a typical hardware implementation, the processing complexity of this stage is quite small and we will ignore the computational complexity of this part.

The maximum number of re-encodes is the number of UEs transmitted in a slot minus 1 (NUE-1).

### 3.3.2	Modulator

We assume a look up table for the modulator

Modulator Complexity ~Nsymbdata NscNSF

The maximum number of modulations is the number of UEs transmitted in a slot minus 1 (NUE-1).

### 3.3.3	Spreader

Spreading is a complex multiply of each modulated symbol by a sequence.

Modulator Complexity ~Nsymbdata Nsc

Number of complex multiplies: Nsymbdata Nsc

The maximum number of spreadings is the number of UEs transmitted in a slot minus 1 (NUE-1).

### 3.3.4	Symbol Reconstruction

Symbol reconstruction is a complex multiply by the channel estimates for each antenna.

Symbol Reconstruction Complexity ~Nsymbdata NscNRx

Number of complex multiplies: Nsymbdata NscNRx

The maximum number of symbol reconstructions is the number of UEs transmitted in a slot minus 1 (NUE-1).

We assume that modulation, spreading and signal reconstruction is performed on the fly for each user, the reconstructed symbol of each user is then stored in memory, and that data is subtracted from the receive single buffer before moving on to the next user. Hence, we store data for a single user, at the output of symbol reconstruction. The size of this memory is: Nsc∙Nsymbdata∙NRx Complex values.

### 3.3.5	Interference Subtraction

This involves memory read of the received signal and subtraction of the reconstructed signal and writing back the signal to memory.

Interference Complexity ~Nsymbdata NscNRx

Number of complex additions: Nsymbdata NscNRx

The maximum number of symbol reconstructions is the number of UEs transmitted in a slot minus 1 (NUE-1).

## 3.4	Memory Aspects

While a detailed memory analysis of the MMSE-based hard IC receiver is beyond the scope of this document. We can identify key buffers that are needed for proper operation of the receiver. In addition to the buffers identified in the previous subsections for the output of channel estimation, MMSE weight estimation, equalization and symbol reconstruction, the following memory block is also needed:

- The received signal needs to be buffered to be used later for interference cancellation and subsequent decodes. The amount of data that is buffered depends on the number of resource elements per transmission and number of receive antennas (NsymbdataNscNRx), and the number of transmissions that need to be buffered NTx. Where NTx depends on the latency and pipelining within the architecture. For this analysis, we assume NTx=2, i.e. double buffering.

## 3.5	Number of iterations and number of user per iteration

The number of iterations and number of users per iteration determine the overall performance of the receier. As the number of iterations in the receiver increases performance improves until we get to the point of deminsing return as shown in Figure 3 and Figure 4. Most of the gain is captured when going from 1 to 2 iterations, beyond 3 iterations there is little gain. For complexity analysis it is reasonable to assume a maximum 3 iterations for the MMSE-based hard IC receiver.

Figure 3: NOCA BLER-SNR performance with different number of iterations for an MMSE-based Hard IC receiver with 6 users.

Figure 4: NOCA BLER-SNR performance with different number of iterations for an MMSE-based Hard IC receiver with 12 users.

During the first iteration all users are processed, even though it is also possible to have a scheme where only a subset of users are processed during the first iteration, let’s say the K1 users with the strongest power. During subsequent iterations the number of users processed is reduced as users that are successfully decoded in pervious iterations are not processed any more. Figure 5 shows the average number of users per iteration, this depends on the signal-to-noise ratio. If we assume the SNR is -10 dB, which is around the 10% BLER point from Figure 4. During the first iteration all 12 users are processed. During the second iteration the number of users processed drops to about 2.2 users on average and about 1 user on average during the third iteration. Using the number of iterations and the average number of users per iteration, we can compute the computation complexity of the receiver processing.

Figure 5: Average number of users per iteration.

## 3.6	Receiver Complexity Template for MMSE-based receiver

Based on the discussions of the previous sub-sections Table 3 gives the complexity template for MMSE-based hard IC receiver.

Table 3: Complexity Template of MMSE-based hard IC receiver.

| Receiver Component | Detailed Component | Number of Usage | Order of Complexity | Impact Factor |
| --- | --- | --- | --- | --- |
| Detector | UE Detection | NUE,Dect | Nsc∙NsymbDMRS∙NRx | CMult: Nsc∙NsymbDMRS∙NRx CAdd: Nsc∙NsymbDMRS∙NRx |
|  | Channel Estimation | NUE | Nsc∙NsymbDMRS+Nsymbdata∙NRx | CMult: 2∙Nsc∙Nsymbdata∙NRx CAdd: Nsc∙Nsymbdata∙NRx Mem: NUE∙Nsc∙Nsymbdata∙NRx Complex values |
|  | Noise Estimation | Nitr | Nsc∙NsymbDMRS∙NRx∙NUE,avg |  |
|  | MMSE Weight Calculation | Nitr | Nsymbdata NscNUE,i3NSF “i” is the iteration number. | Nsymbdata NscNSFNRx.NSF2NUE,i+NRx.NSFNUE,i2+NUE,i32+NUE,i Mem: NUE∙Nsc∙Nsymbdata∙NRx Complex values |
|  | Equalization | Nitr | Nsymbdata NscNRxNUE,avg | Cmult: Nsymbdata NscNRxNUE,avg CAdd: Nsymbdata NscNRxNUE,avg Mem: NUE∙Nsc∙Nsymbdata∙NRxNSF Complex values |
| Decoder | LDPC Decoder | NUE,avg∙Nitr |  |  |
| Interference Cancellation | Re-encoding | NUE-1 |  |  |
|  | Modulation | NUE-1 | Nsymbdata NscNSF | Memory Lookup: Nsymbdata NscNSF |
|  | Spreading | NUE-1 | Nsymbdata Nsc | CMult: Nsymbdata Nsc |
|  | Symbol reconstruction | NUE-1 | Nsymbdata NscNRx | CMult: Nsymbdata NscNRx Mem: Nsc∙Nsymbdata∙NRx Complex values |
|  | Interference subtraction | NUE-1 | Nsymbdata NscNRx | CAdd: Nsymbdata NscNRx |
| Memory | Receive Buffer |  |  | NsymbdataNscNRxNTx Complex values NTx is the number of buffering instances, e.g. 2 for double buffering. |

Observation 1: Table 3 gives the complexity template of the MMSE-based hard IC receiver.

Numerical example:

Table 4 gives the assumptions used for the numerical evaluation of the complexity of the MMSE-based hard IC receiver. The complexity is given by Table 5.

Table 4: Assumptions for the evaluation of the MMSE-base hard IC receiver.

| Parameter | Value | Parameter | Value |
| --- | --- | --- | --- |
| NUE,Dect= NUE | 12 | NRx | 2 |
| NsymbDMRS | 2 | NsymbData | 12 |
| Nsc | 12 x 12 = 144 | Average Iterations | 1.94 |
| Max Iterations | 3 | NSF | 12 |

Table 5: Complexity of MMSE-based hard IC receiver.

| Receiver Component | Detailed Component | Number of Usage | Order of Complexity | Impact factor |
| --- | --- | --- | --- | --- |
| Detector | UE Detection | 12 | Nsc∙NsymbDMRS∙NRx | CMult: 576 CAdd: 576 |
|  | Channel Estimation | 23.28 | Nsc∙NsymbDMRS+Nsymbdata∙NRx | CMult: 6912 CAdd: 3456 Mem: 41472 CV (complex values) |
|  | Noise Estimation | 3 | Nsc∙NsymbDMRS∙NRx∙NUE,avg |  |
|  | MMSE Weight Calculation | 3 | Nsymbdata NscNUE,i3NSF “i” is the iteration number. | CMult and CAdd: 1912230 Mem: 41472 CV |
|  | Equalization | 3 | Nsymbdata NscNRxNUE,avg | CMult: 10057 CAdd: 10057 Mem: 3456 CV |
| Decoder | LDPC Decoder | 19.6 |  |  |
| Interference Cancellation | Re-encoding | 11 |  |  |
|  | Modulation | 11 | Nsymbdata NscNSF | Memory Lookup: 144 |
|  | Spreading | 11 | Nsymbdata Nsc | CMult: 1728 |
|  | Symbol reconstruction | 11 | Nsymbdata NscNRx | CMult: 3456 Mem: 3456 CV |
|  | Interference subtraction | 11 | Nsymbdata NscNRx | CAdd: 3456 |
| Memory | Receive Buffer |  |  | Mem: 8064 CV |
|  | Total Memory |  |  | 97920 CV |

# 4	Conclusion

In this document we presented a detailed analysis of the of the complexity of the MMSE-based hard IC receiver. The following observation was made:

Observation 1: Table 3 gives the complexity template of the MMSE-based hard IC receiver.

# References

- RAN1 Chairman’s Notes 3GPP TSG RAN WG1 Meeting#94, Gothenburg, Sweden, 3GPP.

- R1-165360, “Selection of the eMBB channel coding scheme,” Nokia, Alcatel-Lucent Shanghai Bell, Nanjing, China, May 2016.

- W. E. Ryan, S. Lin, “Channel Codes: Classical and Modern”, New York, NY: Cambridge University Press, 2009.

- R1-1811102, “Receiver considerations for UL NOMA”, Nokia, NSB, Chengdu, China, October 2018.