3GPP TSG RAN WG1 Meeting #94bis			R1-1811377

Chengdu, China, October 8th – 12th, 2018

Source:	NTT DOCOMO, INC.

Title:	Remaining details on evaluation methodology

Agenda Item:	7.2.6.1

Document for: 	Discussion and Decision

- Introduction

In RAN1#94 meeting, the simulation assumptions on Rel.16 NR URLLC were discussed and the following agreements were made [1]:

Agreements: - Select one or more representative use case(s) for the prioritized use cases in the SID and/or the Rel-15 enabled use case for evaluation, which use case(s) to evaluate is up to companies. - Further discussion how/whether to capture them in the TR - Further discussion other detailed simulation assumptions The following table of representative use cases for selection for evaluation is an example as the starting point for further discussion: Use case (Clause #); Reliability (%); Latency (ms); # of UEs (per cell); Data packet size and traffic model; Description. Transport Industry (22.186: 5.5); [99.999]; [5] (end to end latency); [30]; DL: [TBD] byte; ftp model 3 with arrival interval [TBD] s UL: [TBD] byte; Periodic with arrival interval [TBD] s; Remote driving. Power distribution (22.804:5.6.4 &5.6.6); 99.9999; 5(end to end latency); 8; [80] byte ftp model 3 with arrival interval 100ms; Power distribution grid fault and outage management. [99.999]; 15(end to end latency); 8; 250 byte Periodic and deterministic with arrival interval 0.833 ms; Differential protection. Factory automation (22.804: 5.3.2); 99.9999; [2](end to end latency); [4, 40]; 20 byte, 50 byte Periodic and deterministic traffic model; Motion control. Rel-15 enabled use case (e.g. AR/VR); 99.999; [1ms] (air interface delay); 1, 5, 10, 20; [32, 256] bytes FTP model 2/3 or periodic with different arrival rates; Companies report the combination of the requirement. - All the entries in the above table are subject to further discussion which can be revisited in the next meeting - Note: The details on above the requirements can refer to R1-1809337. - Note: 3ms ~ 10ms CN delay for differential protection (i.e. power distribution case 2) could be considered. - Note: Rel-15 higher layer mechanisms for reliability may be applicable for achieving the reliability requirement - Note: The reliability and latency are as defined in 22.186. - Note: For AR/VR, the requirement can refer to section 7.2.3 in TS 22.261. - Note: FFS whether the packet size is based on application layer or L2/L3. The packet size listed in the table needs to further discussed, especially depending on the outcome of whether the packet size is based on application layer or L2/L3 - Further discussion on how to map the requirements (e.g., reliability, latency, etc.) to RAN-level requirements Agreements: - Further discussion till next meeting regarding whether/how to evaluate the number of users, the % of users, etc., satisfying reliability and latency requirements. Agreements: - Companies are encouraged to report the CDF of UE geometry - Further discussion whether/how to re-use the deployment and channel models in the existing TRs (e.g. 38.802, 37.885 and 38.901)

In the email discussion [94-NR-06] after RAN1#94 meeting, additional simulation assumptions on Rel.16 NR URLLC were discussed and the following proposals were approved by email [2]:

Proposals approved by the email discussion: - Additional assumptions for evaluation: - Companies describe overhead modeling (e.g. PDCCH overhead) used by companies in the simulation - Companies describe modification to channel models if any - Companies describe power control mechanisms - 1 ms air interface latency is assumed for evaluation for factory automation, with the assumption of 1 ms CN delay in 2 ms end-to-end latency. - Other values for evaluation are not precluded - In evaluations, it is assumed that the packet size is based on L2/L3 SDU in the evaluation - FFS header overhead - FFS whether to describe the following assumptions: - Duplex mode: FDD or TDD (DL/UL configuration) - Re-dropping or discarding UEs which do not satisfy certain channel quality if any - Blockage due to moving metal parts for channel model for factory automation - Other assumptions like TTI size, gNB/UE processing time, CSI measurement and reporting - Companies are encouraged to provide evaluation on baseline performance achievable with Rel-15 NR URLLC, for the prioritized URLLC use cases identified in the Rel-16 URLLC SID. - Take the simulation settings in the following table as the starting point for Rel-16 NR URLLC system level evaluation for urban macro for power distribution: Parameters; Value. Layout; Single layer - Macro layer: Hex. Grid. Inter-BS distance; 500m Note: Other value (e.g. 150 m) is not precluded. Carrier frequency; 4 GHz. Channel model; UMa in TR 38.901. UE Tx power; 23dBm. BS antenna configurations; FFS antenna ports (e.g., 4 Tx/4 Rx antenna, etc.) (M, N, P, Mg, Ng) = (8, 8, 2, 1, 1); dH = 0.5λ, dV = 0.8λ; 102 degree for 500m ISD. BS antenna height; 25m. BS antenna element gain + connector loss; 8 dBi. BS receiver noise figure; 5dB. UE antenna configuration; FFS antenna ports (e.g., 2 Tx/4 Rx antenna ports, etc) Panel model 1: Mg=1, Ng=1, P=2, dH=0.5. UE antenna height; Follow the modelling of TR 38.901 (e.g. 1.5m). UE antenna gain; 0dBi as starting point. UE receiver noise figure; 9 dB. Total transmit power per TRxP; 49 dBm. BS receiver; MMSE-IRC as the baseline receiver Note: Advanced receiver is not precluded.. Parameters with the value not defined in 38.802. Number of UEs per cell; Up to 10. Simulation bandwidth; 40 MHz. SCS; 30 kHz Note: Other values for evaluation are not precluded.. UE distribution; 100% of users are outdoors. UE power control; Companies report the PC mechanisms used for URLLC.. HARQ/repetition; Companies report (including HARQ mechanisms).. Channel estimation; Realistic. - Take the simulation settings in the following table as the starting point for Rel-16 NR URLLC system level evaluation for indoor hot-spot for factory automation: Parameters; Value. Inter-BS distance; 20m. Carrier frequency; 4 GHz. UE Tx power; 23dBm. BS antenna element gain + connector loss; 5 dBi. BS receiver noise figure; 5dB. BS antenna configurations; FFS antenna ports (e.g., 4 Tx/4 Rx antenna ports, etc) (M, N, P, Mg, Ng) = (4, 4, 2, 1, 1), dH = dV = 0.5 λ for 4GHz. BS antenna height; [3] m Note: Companies report the modification of the layout. UE antenna configuration; FFS antenna ports (e.g., 2 Tx/4 Rx antenna ports, etc.) Panel model 1: Mg = 1, Ng = 1, P = 2, dH = 0.5. UE antenna height; Follow the modelling of TR 38.901 (e.g. 1.5m) Note: Companies report the modification of the layout. UE antenna gain; 0dBi as starting point. BS Tx power; 24 dBm per 20 MHz. BS receiver; MMSE-IRC as the baseline receiver Note: Advanced receiver is not precluded.. UE receiver noise figure; 9 dB. Parameters with the value not defined directly for factory automation in 38.802. SCS; 30 kHz Note: Other values for evaluation are not precluded.. Simulation bandwidth; 40 MHz. Layout; Single layer as defined in 38.802 Indoor floor: [(3, 6, 12) BSs per 120 m x 50 m] Note: Companies report the modification of the layout. Channel model; ITU InH for 4 GHz Companies report the modification of the channel model. Number of UEs per cell; Up to [40]. UE distribution; 100% of users are indoor: 3 km/h and/or 30 km/h UE-speed Note: which one to use is up to companies and other value(s) are not precluded. UE power control; Companies report the PC mechanisms used for URLLC.. HARQ/repetition; Companies report (including HARQ mechanisms).. Channel estimation; Realistic. - Evaluation of 30 GHz carrier frequency is not precluded.

In this contribution, we will discuss the simulation methodology and assumptions for Rel.16 NR URLLC.

- Simulation methodology and assumptions

  - System-level simulation

Performance metrics

In the email discussion [94-NR-06] on URLLC simulation assumption, two options on the performance metrics for URLLC were discussed:

- Option 1: Percentage of users satisfying reliability and latency requirements.

- Option 2: URLLC capacity as defined in TR 38.802.

URLLC capacity and URLLC / eMBB multiplexing capacity as defined in 38.802 Definition: URLLC system capacity is calculated as follows: - C(L, R) is the maximum offered cell load under which Y% of UEs in a cell operate with target link reliability R under L latency bound - X = (100 – Y) % is the percentage of UEs in outage - A UE in outage is defined as the UE cannot meet latency L and link reliability R bound Companies report their assumption on X

Option 1 is useful to evaluate whether a given number of URLLC UEs in the evaluated bandwidth can satisfy the URLLC requirements assuming that there are only the URLLC UEs in the evaluated bandwidth. However, this would not be realistic assumption in various cases since, in general, the system/carrier is not only for a particular URLLC service. For instance, in the case of factory automation, the system must accommodate terminals for automation devices, security/monitoring cameras, various sensors, handsets, etc. as shown in Fig. 1. It is possible to reserve a certain amount of bandwidth of the carrier for the URLLC service. However, as analyzed in [3,4], reserving a certain amount of bandwidth for a particular URLLC service is not resource-efficient. We believe that even for URLLC, spectral efficiency is not compromised. Option 2 is more appropriate to evaluate scenarios where eMBB and URLLC are in the same cell and share the resources.

Fig.1: Illustration of the cell serving both specific URLLC UEs and non-URLLC UEs

Proposal 1:

- For system-level simulation, the capacity as defined in TR38.802 can be used as a performance metric for Rel.16 NR URLLC.

FR2

NR natively supports FR1/FR2 and FDD/TDD in Rel.15. Since SID does not preclude any frequency range and duplex mode, and there will be a market demand, there is no reason to preclude some in RAN1 study. Without evaluation, it is premature to exclude FR2 from the study at this early stage. On the other hand, it is understandable that FR2 may not be suitable for wide area URLLC operation. If this is the common understanding, our suggestion is that the system-level evaluation of FR2 focuses on small area use-cases, i.e., factory automation.

Then, it is necessary to further discuss how to evaluate the case of 30GHz carrier frequency for indoor hot-spot factory automation. For further discussion, we provide a possible set of simulation assumptions/parameters for system-level simulation assumption for 30GHz carrier frequency for indoor hot-spot factory automation in Table 1. The parameters are from TR38.802 [5] if there are no specific descriptions.

Table 1: System-level evaluation assumptions (Indoor hot-spot for factory automation)

| Parameters | Values |
| --- | --- |
| Layout | Single layer as defined in 38.802 Indoor floor: (3, 6, 12) BSs per 120 m x 50 m Note: Companies report the modification of the layout |
| Inter-BS disctance | 20m |
| Carrier frequency | 30GHz |
| Channel model | 5GCM office |
| UE Tx power | 23dBm |
| BS antenna configurations | 8 Tx/Rx antenna ports as starting point. (M,N,P,Mg,Ng) = (4,8,2,1,1) ,(dH,dV) = (0.5, 0.5)λ |
| BS antenna height | 3m |
| BS antenna element gain + connector loss | 5dBi |
| BS receiver noise figure | 7dB |
| UE antenna configuration | 4 Tx/Rx antenna ports as starting point. Config 1+Config a: (M, N, P, Mg, Ng) = (2, 4, 2, 1, 2) Θmg,ng=90; Ω0,1=Ω0,0+180; (dgH, dgV)=(0,0) The polarization angles are 0 and 90. |
| UE antenna height | Follow the modelling of TR 38.901 (e.g. 1.5m) Note: Companies report the modification of the layout |
| UE antenna gain | 5dBi as starting point |
| UE receiver noise figure | 10dB (high performance) |
| Total transmit power per TRxP | 23dBm for 80MHz bandwidth |
| BS receiver | MMSE-IRC as the baseline receiver Note: Advanced receiver is not precluded. |
| Parameters with the value not defined directly for factory automation in 38.802 |  |
| Number of UEs per cell | Up to 10 |
| Simulation bandwidth | 160MHz |
| SCS | 120kHz Other values for evaluation are not precluded. |
| UE distribution | 100% of users are indoor: 3 km/h and/or 30 km/h UE-speed Note: which one to use is up to companies and other value(s) are not precluded |
| UE power control | Companies report the PC mechanisms used for URLLC. |
| HARQ/repetition | Companies report (including HARQ mechanisms). |
| Channel estimation | Realistic |

Proposal 2:

- Consider the simulation settings in table 1 as the starting point for FR2 for Rel-16 NR URLLC system-level evaluation for indoor hot-spot for factory automation.

  - Link-level simulation

In the email discussion [94-NR-06], the link-level simulation assumptions for 4GHz were discussed. Same as system-level simulation, 30GHz should also be evaluated. Therefore, in this part, some link simulation parameters for 30GHz are given in the following table for further discussion.

Table 2 Link-level simulation assumptions (Indoor hot-spot)

| Parameter | Value |
| --- | --- |
| Carrier frequency for evaluation | 30 GHz |
| Channel model | Baseline: TDL-D (delay spread: 30ns) as in 38.901 Note: Companies report the modification of the channel model |
| Deployment | Baseline: Indoor hot-spot as listed in 3GPP 38.802 Note: Companies report the modification of the channel model |
| UE speed | 3 km/h, 60 km/h |
| BS TX antenna configuration | 2 TX ports, 4 Tx ports Note: One or more can be simulated |
| BS RX antenna configuration | 4 Rx ports Note: One or more can be simulated |
| UE TX antenna configuration | 2TX ports |
| UE RX antenna configuration | 4 RX ports |
| System bandwidth | 160 MHz |
| Sub-carrier spacing | 120kHz Note: other value(s) for evaluation are not precluded |
| Channel estimation | Practical |
| Receiver type | MMSE |
| Q value (i.e. SINR range) | Companies could report the Q value |

Proposal 3:

- Consider the simulation settings in table 2 as the starting point for FR2 for Rel-16 NR URLLC link-level evaluation for indoor hot-spot for factory automation.

- Preliminary evaluation results of DL/UL SINR CDF

In this part, we evaluate the DL/UL SINR at 4GHz and 30GHz for factory automation scenario. For 4GHz, the agreed simulation assumption by email discussion [94-NR-06] is used and for 30GHz, the above simulation assumption in table 1 is used unless noted otherwise. For this evaluation, for 4GHz, 32 antenna ports are assumed at the gNB and 4 antenna ports are assumed at the UE. For both 4GHz and 30GHz, 10MHz simulation bandwidth and 15kHz SCS are assumed for the following evaluation.

Fig.1, Fig.2 show cumulative distribution functions (CDFs) of DL SINR and UL SINR for different carrier frequency, respectively. From the results, we can see that for both DL and UL, the SINR of 30GHz is significantly better than the SINR of 4GHz. This is mainly because of the beamforming gain achieved at 30GHz and reduced power-spectrum density. Considering better SINR performance can be achieved at 30GHz with an appropriate beam-management, we would like to encourage company to study and evaluate the URLLC at 30GHz for factory automation.

Fig.1 DL SINR for 4GHz and 30GHz

Fig.2 UL SINR for 4GHz and 30GHz

Observation 1:

- For indoor hot-spot scenario for factory automation, the DL/UL SINR of 30GHz is significantly better than that of 4GHz because of the beamforming gain and reduced power-spectrum density.

- Conclusion

In this contribution, we discussed the simulation methodology and assumptions for URLLC and proposed following:

Proposal 1:

- For system-level simulation, the capacity as defined in TR38.802 can be used as a performance metric for Rel.16 NR URLLC.

Proposal 2:

- Consider the simulation settings in table 1 as the starting point for FR2 for Rel-16 NR URLLC system-level evaluation for indoor hot-spot for factory automation.

Proposal 3:

- Consider the simulation settings in table 2 as the starting point for FR2 for Rel-16 NR URLLC link-level evaluation for indoor hot-spot for factory automation.

Observation 1:

- For indoor hot-spot scenario for factory automation, the DL/UL SINR of 30GHz is significantly better than that of 4GHz because of the beamforming gain and reduced power-spectrum density.

References

- 3GPP RAN1#94 Chairman’s notes.

- R1-1810224, Summary of email discussion [94-NR-06] on additional simulation assumptions for Rel-16 NR URLLC.

- R1-166395, URLLC system capacity and URLLC/eMBB multiplexing efficiency analysis.

- R1-1705624, UL URLLC capacity study and URLLC/eMBB dynamic multiplexing design principle.

- 3GPP, TR 38.802, Study on New Radio (NR) Access Technology; Physical Layer Aspects.