3GPP TSG RAN WG1 Meeting #94bis	R1-1811826

Chengdu, China, October 8th-12th, 2018

Source:	Ericsson

Title:	Performance of NC-JT with large number of antenna ports

Agenda Item:	7.2.8.6

Document for:	Discussion and Decision

# Introduction

NC-JT was studied and specified in LTE Rel-14 [1] and Rel-15, respectively.  Some NC-JT gains were observed with 2 antenna ports at each TRP and 4Rx at the UE. Most of the gains occur at lower loads where active UEs are sparse and with NC-JT, full rank 4 operation is possible from the UE’s perspective.

NC-JT has been proposed as a MIMO enhancement in Rel-16 NR [2].  While 2 antenna ports remain the mostly deployed base station antennas in LTE, it is expected that large number of antenna ports per TRP will be deployed in NR from the beginning.  Therefore, it would be interesting to understand how NC-JT performs in such NR deployment scenarios.

In this contribution, we present some preliminary simulation results on NC-JT with 2 or 32 antenna ports at each TRP.  This is a revision of R1-1811150.

# NC-JT with 2 Antenna Ports

We have first simulated NC-JT with 2 antenna ports at each TRP and 4Rx at all the UEs under the urban macro scenario as defined in TR 38.802. For comparison, single TRP scheduling and dynamic point selection (DPS) are also simulated as baselines as these are already supported techniques. For NC-JT and DPS, fixed clusters are formed, and each cluster consists of 3 sectors within one site, i.e., intra-site clustering.  Ideal channel feedback with SVD based precoders are used. Other simulation parameters can be found in the Appendix.

Table 1 shows the cell edge UE throughput gains by NC-JT over single TRP SU-MIMO scheduling at various resource utilization (RU) levels. The gains of DPS over single TRP are also shown. At 10% and 20% RU, NC-JT provides 14% and 13% gains, respectively, over single TRP scheduling. The gain diminishes at RUs above 40%. DPS provides better throughput than NC-JT above 20% RU. The large NC-JT gain at low load is due to the higher combined rank transmission from multi-TRPs, but this effect cannot be effectively utilized- at higher loads because of inter-cell interference, particularly for cell edge UEs, and the gain disappears.

Table 1:  Cell edge UE throughput gains with NC-JT and DPS (2 antenna ports and 4 Rx)

| RU (Single TRP) | Single TRP (Baseline 1) | DPS (Baseline 2) | NC-JT |
| --- | --- | --- | --- |
| 10% | 0% | 2% | 14% |
| 20% | 0% | 9% | 13% |
| 30% | 0% | 7% | 3% |
| 40% | 0% | 10% | 2% |
| 50% | 0% | 6% | -7% |
| 60% | 0% | 13% | -1% |

Table 2 shows the mean UE throughput gains by NC-JT and DPS at various resource utilization (RU) levels. Again, large NC-JT gains can be seen at low loads.  At higher loads, NC-JT still has some gains, but they are marginal compared to DPS.

Table 2: Mean UE throughput gains with NC-JT and DPS (2 antenna ports and 4 Rx)

| RU (Single TRP) | Single TRP (Baseline 1) | DPS (Baseline 2) | NC-JT |
| --- | --- | --- | --- |
| 10% | 0% | 1% | 14% |
| 20% | 0% | 2% | 11% |
| 30% | 0% | 3% | 9% |
| 40% | 0% | 3% | 8% |
| 50% | 0% | 4% | 8% |
| 60% | 0% | 4% | 9% |

Given that DPS is already supported in NR Rel-15, it makes sense to use DPS as the baseline when evaluating NC-JT performance in the cases where it has benefit over single TRP.  Thus, we make the following observation:

With 2 antenna ports and 4 Rx, NC-JT provides moderate to large gains at low load (i.e. <30% of RU) and small to negligible gains at high load.

# NC-JT with 32 Antenna ports

Next, we have simulated NC-JT with 32 antenna ports, i.e., (M, N, P) = (8,2,2), at each TRP and 4Rx at the UEs, again under the urban macro scenario as defined in TR 38.802. The results are shown in Table 3 and Table 4.

The results show that DPS provides some moderate to large cell edge gain and small mean throughput gain over single TRP scheduling.  The gains of NC-JT are about the same as DPS, no additional gain if observed or NC-JT when compared to DPS. The reason for the disappearance of NC-JT gain comparing to the 2 antenna ports case is that with large number of antenna ports, full rank (rank 4 in this case) transmission from a single TRP can also be achieved.

Table 3: Cell edge UE throughput gains with NC-JT and DPS (32 antenna ports and 4 Rx)

| RU (single TRP) | Single TRP (baseline 1) | DPS (baseline 2) | NC-JT |
| --- | --- | --- | --- |
| 10% | 0% | 7% | 8% |
| 20% | 0% | 10% | 8% |
| 30% | 0% | 12% | 12% |
| 40% | 0% | 19% | 12% |

Table 4: Mean UE throughput gains with NC-JT and DPS (32 antenna ports and 4 Rx)

| RU (single TRP) | Single TRP (baseline 1) | DPS (baseline 2) | NC-JT |
| --- | --- | --- | --- |
| 10% | 0% | 1% | 2% |
| 20% | 0% | 2% | 2% |
| 30% | 0% | 2% | 3% |
| 40% | 0% | 3% | 3% |

With 32 antenna ports at each TRP, NC-JT has similar performance as DPS, even at very low load.

# Conclusions

In this contribution, we made the following observations:

Observation 1	With 2 antenna ports and 4 Rx, NC-JT provides moderate to large gains at low load (i.e. <30% of RU) and small to negligible gains at high load.

Observation 2	With 32 antenna ports at each TRP, NC-JT has similar performance as DPS, even at very low load.

# References

TR36.741, “Study on further enhancements to Coordinated Multi-Point (CoMP) operation for LTE”

RP-181453 WID on NR MIMO.

# Appendix:  Simulation Parameters

| Carrier frequency | 4 GHz |
| --- | --- |
| Bandwidth | 20 MHz |
| Scenarios | Urban Macro as defined in TR 38.802, 500m ISD |
| TRP antenna configurations | 2 ports: (M,N,P) = (1,1,2) 32 ports: (M,N,P) = (8,2,2) |
| UE antenna | 4Rx: (M,N,P) = (1,1,2) |
| TRP antenna spacing | 0.8 lambda in vertical, 0.5 lambda in horizontal |
| Cell layout | 57 homogeneous cells |
| Wrapping | Radio distance based |
| UE receiver | MMSE-IRC |
| CSI periodicity | 1 ms |
| CSI delay | 1 ms |
| CSI mode | Explicit |
| Link Adaptation | Ideal |
| UE noise figure | 9 dB |
| gNB Tx power | 49 dBm |
| Traffic model | FTP Model 1, 100 kB packet size |
| UE speed, outdoor | 30km/h |
| Scheduling | Single TRP; DPS; NC-JT |
| Multi-TRP cluster | Intra-site 3 sectors |
| Handover margin | 0 dB |
| Transmission | SU-MIMO with up to rank 4 from UE perspective (Rank adaptation enabled) |