3GPP TSG-RAN1 Meeting #94bis			R1-1811866

Chengdu, China, October 8th – 12th, 2018

Title:	Draft Reply to LS on Preamble Power Ramping Counter

Response to:	R2-1813493

Release:	Release 15

Work Item:	NR_newRAT-Core

Source:	Fujitsu [to be RAN WG1]

To:	RAN WG2

Cc:	RAN WG4

Contact Person:

Name: Zhang Lei

E-mail Address:	zhanglei@cn.fujitsu.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	none

1. Overall Description:

RAN1 discussed how to handle the power ramping counter for the four Cases (C-F).

(C) The CSI-RS is selected and the CSI-RS is same as the last RA preamble transmission.

(D) The CSI-RS is selected but the CSI-RS is not same as the last RA preamble transmission.

(E) The SSB is selected but the CSI-RS was selected in the last RA preamble transmission.

(F) The CSI-RS is selected but the SSB was selected in the last RA preamble transmission.

The following was agreed:

Agreements:

- Regarding RAN2’s LS on Power Ramping: For case (C), UE is allowed to increment the power ramping counter; for case (D)(E)(F), the power ramping counter remains unchanged.

2. Actions:

To RAN2 group.

ACTION: 	RAN1 respectfully asks RAN2 to take the above information into consideration.

3. Date of Next TSG-RAN1 Meetings:

TSG-RAN1 Meeting #95			12th – 16th November 2018		Spokane, WA, USA

TSG-RAN1 Meeting AH#1901		21st – 25th January 2019			Taipei