3GPP TSG-RAN WG1 Meeting #95	R1-181xxxx

# Spokane, USA, November 12 - 16, 2018

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.212 | CR | Draft | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on hop splitting related bit number calculation for PUSCH with UCI only |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2018-10-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Two reasons for this correction to current 38.212-v15.3.0: - The current hop splitting rule for coded bits for UCI-only PUSCH can result in two bad consequences: (1) CSI-part1 will be punctured on hop2; and (2) some reserved REs of hop2 have no data to transmit. - When hop1 has no resource for HARQ-ACK, the current calculation for coded bits number of reserved REs for HARQ-ACK in the first hop is not correct (can be larger than 0, but no resources to convey). |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Exchange the rounding down and rounding up operation to calculate the number of coded bits in each hop for both HARQ-ACK and reserved RE for HARQ-ACK. - Set an upper bound to the encoded bits number calculation for reserved REs for HARQ-ACK in the first hop. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification is incorrect for hop splitting of encoded bits mapped on UCI-only PUSCH |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

--------------------------------------- Start of Text Proposal ----------------------------------------------

< Unchanged parts are omitted >

6.2.7	Data and control multiplexing

-	if HARQ-ACK is present for transmission on the PUSCH with UL-SCH, let

-	 and;

-	if CSI is present for transmission on the PUSCH with UL-SCH, let

-	;

-	;

-	; and

-	;

-	if only HARQ-ACK and CSI part 1 are present for transmission on the PUSCH without UL-SCH, let

-	;

-	;

-	; and

-	;

-	if HARQ-ACK, CSI part 1 and CSI part 2 are present for transmission on the PUSCH without UL-SCH, let

-	;

-	;

< Unchanged parts are omitted >

The multiplexed data and control coded bit sequence  is obtained according to the following:

Step 1:

Set  for ;

Set  for ;

Set  for ;

Set  for ;

if the number of HARQ-ACK information bits to be transmitted on PUSCH is 0, 1 or 2 bits

the number of reserved resource elements for potential HARQ-ACK transmission is calculated according to Subclause 6.3.2.4.1.1, by setting ;

denote  as the number of coded bits for potential HARQ-ACK transmission using the reserved resource elements;

if frequency hopping is configured for the PUSCH, let  and ;

< Unchanged parts are omitted >

--------------------------------------------- End of Text Proposal --------------------------------------------