3GPP TSG RAN WG1 Meeting #95   		                                                        R1-1812469

Spokane, USA, November 12th – 16th, 2018

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | DRAFT | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 38.214 for CSI-RS (TRS, CSI, IM) + PDSCH multiplexing |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2018-11-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | To allow FDM multiplexing of CSIRS and PDSCH in FR2 when UE is not capable of receiving transmission with more and one QCL Type-D assumption |  |  |  |  |  |  |  |  |
| Summary of change: |  | Prioritize CSI-RS reception over PDSCH |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Imcomplete specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.1.5.1, 5.1.5, |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

- Text proposal for section 5.2.1.5.1 (AP CSI-RS CSI/IM collision with PDSCH):

-	For each aperiodic CSI-RS resource in a CSI-RS resource set associated with each CSI triggering state, the UE is indicated the quasi co-location configuration of quasi co-location RS source(s) and quasi co-location type(s), as described in Subclause 5.1.5, through higher layer signaling of qcl-info which contains a list of references to TCI-State's for the aperiodic CSI-RS resources associated with the CSI triggering state. If a State referred to in the list is configured with a reference to an RS associated with 'QCL-TypeD', that RS may be an SS/PBCH block located in the same or different CC/DL BWP or a CSI-RS resource configured as periodic or semi-persistent located in the same or different CC/DL BWP.

-	If the scheduling offset between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of the aperiodic CSI-RS resources in a NZP-CSI-RS-ResourceSet configured without higher layer parameter trs-Info and without the higher layer parameter repetition is smaller than the UE reported threshold in FG 2-28 when the reported value is one of the values of {14, 28, 48}, the UE may assume that the CSI-RS resources are quasi co-located with the RS(s) in the TCI state with respect to the QCL parameter(s) used for PDCCH quasi co-location indication of the lowest CORESET-ID in the latest slot in which one or more CORESETs within the active BWP of the serving cell are configured for the UE.

-  if there is only PDSCH transmitted in the same symbols as the CSI-RS, and the scheduling offset for the PDSCH is larger than or equal to Threshold-Sched-Offset, the UE applies the QCL Type-D assumption of the DM-RS of the PDSCH as the QCL Type-D assumption for the aperiodic CSI-RS resources, if QCL Type D is applicable as the QCL assumption used for the DM-RS of the PDSCH.

- -	If the scheduling offset between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of the aperiodic CSI-RS resources is equal to or greater than the UE reported threshold in FG 2-28 when the reported value is one of the values of {14,28,48}, the UE is expected to apply the QCL assumptions in the indicated TCI states for the aperiodic CSI-RS resources in the CSI triggering state indicated by the CSI trigger field in DCI.

- -  if there is PDSCH transmitted in the same symbols as the CSI-RS, the UE applies the QCL Type D assumption for the aperiodic CSI-RS as the QCL Type D assumption used for the DM-RS of the PDSCH, if QCL Type D is applicable.

- Text proposal for 38.214 section 5.1.5: (P/SP CSIRS CSI/IM collision with PDSCH)

- If the ‘QCL-TypeD’ of the PDSCH DM-RS is different from that of a periodic or semi-persistent CSI-RS resource in a NZP-CSI-RS-ResourceSet configured without higher layer parameter trs-Info and without the higher layer parameter repetition, the CSI-RS resource with which it overlaps in at least one symbol, the UE is expected to prioritize the reception of the CSI-RS resource. This also applies to the intra-band CA case (when PDSCH and the CSI-RS are in different component carriers).

- Text proposal for 38.214 section 5.1.5: (TRS collision with PDSCH)

- If the ‘QCL-TypeD’ of the PDSCH DM-RS is different from that of a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info the CSI-RS resource with which it overlaps in at least one symbol, the UE is expected to prioritize the reception of the CSI-RS resource. This also applies to the intra-band CA case (when PDSCH and the CSI-RS are in different component carriers).