3GPP TSG RAN WG1 Meeting #95   		                                                        R1-1812469

Spokane, USA, November 12th – 16th, 2018

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | DRAFT | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 38.214 for SRS + SRS multiplexing |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2018-11-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | To allow FDM multiplexing of multiple SRS resources in FR2 when UE is not capable of transmitting with more and one spatialRelationInfo assumption |  |  |  |  |  |  |  |  |
| Summary of change: |  | Prioritization based on AP/SP/P SRS SRS |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Imcomplete specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

- Text proposal for 38.214 section 6.2.1:

For a UE configured with one or more SRS resource configuration(s), and when the higher layer parameter resourceType in SRS-Resource is set to 'periodic':

-	if the UE is configured with the higher layer parameter spatialRelationInfo containing the ID of a reference 'ssb-Index', the UE shall transmit the target SRS resource with the same spatial domain transmission filter used for the reception of the reference SS/PBCH block, if the higher layer parameter spatialRelationInfo contains the ID of a reference 'csi-RS-Index', the UE shall transmit the target SRS resource with the same spatial domain transmission filter used for the reception of the reference periodic CSI-RS or of the reference semi-persistent CSI-RS, if the higher layer parameter spatialRelationInfo containing the ID of a reference 'srs', the UE shall transmit the target SRS resource with the same spatial domain transmission filter used for the transmission of the reference periodic SRS. If the SRS resource overlaps in at least one symbol with another SRS resource in a different serving cell (in intra-band CA) with resourceType set to ‘periodic’ and configured with spatialRelationInfo that is not identical, the UE prioritizes the SRS transmission on the serving cell with the lowest index.

For a UE configured with one or more SRS resource configuration(s), and when the higher layer parameter resourceType in SRS-Resource is set to 'semi-persistent':

  - if an SRS resource in the activated resource set is configured with the higher layer parameter spatialRelationInfo and if the SRS resource overlaps in at least one symbol with another SRS resource in a different serving cell (in intra-band CA) with resourceType set to ‘periodic’ or ‘semi-persistent’ and configured with spatialRelationInfo that is not identical, the UE prioritizes the semi-persistent SRS transmission on the serving cell with the lowest index.

For a UE configured with one or more SRS resource configuration(s), and when the higher layer parameter resourceType in SRS-Resource is set to 'aperiodic':

  - if the UE is configured with the higher layer parameter spatialRelationInfo and if the SRS resource overlaps in at least one symbol with another SRS resource in a different serving cell (in intra-band CA) with resourceType set to ‘aperiodic’, ‘periodic’ or ‘semi-persistent’ and configured with spatialRelationInfo that is not identical, the UE prioritizes the aperiodic SRS transmission on the serving cell with the lowest index.