3GPP TSG RAN WG1 Meeting #95   		                                                          R1-1812475

Spokane, USA, November 12th – 16th, 2018

Source: 	Intel Corporation

Title:	NOMA transmission scheme

Agenda item:		7.2.1.1

Document for:	Discussion and Decision

# Introduction

At the RAN1#94bis meeting, the following agreements were made to capture the proposed NOMA transmission schemes in TR38.812 [1]:

Agreements:

- The TP in R1-1812076 is approved (except the Annex parts)

In this contribution, we discuss some details of the data processing aspects and provide the details of Low Code Rate Spreading (LCRS) which utilizes UE-specific bit-level scrambling.

# Transmitter side data processing

As shown in Figure 1, general structure of NOMA transmitter processing was agreed and captured in the TR38.812 [1]. In the figure, the blocks in black and white reuse the current NR design, while new blocks with specification impact are highlighted in green. It should be noted that, the initialization seed for the legacy bit-scrambling sequence generation can be updated which may involve certain specification impact.

Figure 1. General structure of NOMA transmitter processing

This section discusses the detailed aspects of each scheme and provides pros and cons for accurate comparison between different NOMA transmission schemes.

| Tx side processing | Genera properties |  |
| --- | --- | --- |
|  | Pros | Cons |
| UE-specific bit-level scrambling | Bit level scrambling is randomizing the interference from other users, which enables UE differentiation in the receiver side |  |
|  | - Negligible spec impact for supporting NOMA - sufficient number of MA signatures can be supported (almost infinite number can be supported) | N.A. |
| UE-specific bit-level interleaving | Bit level scrambling is randomizing the interference from other users, which enables UE differentiation in the receiver side |  |
|  | - Large number of MA signatures can be supported | - Additional spec impact is required for defining interleaver |
| UE-specific symbol-level spreading | Symbol-level spreading is randomizing the interference by well-defined spreading sequences |  |
|  | - It has been observed from some contributions that well-defined spreading sequence provides good correlation properties which may enable the receiver to differentiate signals from different UEs. | - Additional spec impact - Number of MA signatures are limited (depends on which sequences are used) - Full coding gain may not be obtained for high spectral efficiency transmission scenario since |
| UE-specific symbol-level scrambling | Not much difference from UE-specific bit-level scrambling from the performance perspective |  |
|  | - Potential PAPR decrease | - Additional spec impact is required compared to UE-specific bit-level scrambling - Bigger memory size is required compared to bit-level scrambling |
| UE-specific symbol-level interleaving, with symbol-level zero padding | Not much difference from UE-specific symbol-level interleaving from the performance perspective |  |
|  | N.A. | - Bigger memory size is required compared to bit-level interleaving |
| UE-specific power assignment | Different power is used for different users in order to facilitate interference cancellation in the receiver side |  |
|  | - IC performance can be improved in the receiver side | - Additional configuration is required for differentiating Tx power between users |
| UE-specific sparse RE mapping | Sparse mapping can be done by ether RE mapping or designing spreading sequences. It is preferable to have sparsity (if needed) by defining the sparse spreading sequence not by modifying sparse RE mapping from the spec impact perspective |  |
|  | - Sparse spreading can suppress the inter-user interference | - Significant spec impact is required |
| Cell-specific MA signature | Cell-specific MA signature is not for differentiating UEs sharing the same resource but potentially for other purpose, e.g., PAPR reduction. No big motivation is observed for cell-specific MA signature. |  |
|  | N.A. | N.A. |
| Multi-branch /MA signature transmission per UE | Multi-layer can be used for increasing per-UE spectral efficiency while keeping sufficient code rate as well as enough spreading factor |  |
|  | - High per-UE SE can be supported | - overloading factor is decreased: less number of users can be supported inside the same NOMA resource - significant spec impact is required |

Observation 1

- NOMA can be supported by applying UE-specific bit-level scrambling with negligible spec impact

# Low Code Rate Spreading (LCRS)

## Basic Transmission processing

It is well known that simultaneous transmissions from two users on a common resource along with the use of the SIC receiver is a channel capacity achieving transmission strategy [2]. Since non-orthogonal multi-user multiplexing can accommodate more number of UEs simultaneously, it potentially leads to user and system throughput gains. Furthermore, non-orthogonal multi-user multiplexing schemes efficiently allow grant-free operation, which may be beneficial for supporting large number of UEs requesting intermittent transmissions of small data packets. On the other hand, the receiver complexity for a certain non-orthogonal multi-user multiplexing scheme may increase significantly with the number of simultaneously transmitting UEs, and error propagation in the SIC receiver may limit the throughput gains.

Some non-orthogonal multi-user multiplexing schemes exploit power- and/or code-domain multiplexing. In addition, time- and/or frequency-domain spreading is essential to make effective SINRs of de-spread symbols high enough for successful demodulation and decoding. Thus, the simple spreading schemes for synchronous non-orthogonal multi-user multiplexing may be desirable, which can maintain the common waveform (e.g. OFDM) irrespective of multiple access schemes and can still take advantage of grant-free non-orthogonal transmission for a certain scenario such as small data transmission.

For having the spreading effects in order to support non-orthogonal multi-user multiplexing, there can be multiple ways. One simple alternative is to utilize actual spreading sequence for signal spreading on top of the encoded bits or modulated symbols and the other alternative is to reduce the code rate by utilizing rate matching block so as to effectively spread the encoded bits. Both schemes can be potential candidate methods for NOMA Tx scheme but we believe the second alternative, Low Code Rate Spreading (LCRS), is more promising from the following perspectives:

- Support of flexible code rate,

- Support of (comparatively) full coding gain

- Support of NOMA without introducing new transmission schemes,

Figure 2. Transmission process of LRCS

LCRS transmission scheme is basically the same as the current NR UL transmission. As shown in Figure 2, it utilizes same coding procedure as PUSCH including channel encoding, rate matching, bit-level scrambling and then modulator. As can be seen in the Figure, either OFDMA or SC-FDMA could be used in the last part of the transmission processing. The difference from the normal PUSCH is that the physical resources mapped to the generated signals can be shared by multiple users. The bit-level scrambling part performs as the user separation by randomizing the signals from other users and then making multi-user interference as random as possible. Bit level scrambling function defined in 38.211 [3] can be one example as it is defined in a UE-specific manner.  The only thing to be taken into account is to make sure the code rate is sufficiently low for achieving the maximum coding gain in order to overcome the inter-user interference.

Observation 2

- Low Code Rate Spreading (LCRS) is a promising candidate for NOMA from the perspectives of

  - Support of flexible code rate,

  - Support of (comparatively) full coding gain

  - Support of NOMA without introducing new transmission schemes,

## Potential specification impacts

In the previous section, it was mentioned that LCRS reuses the currently defined NR PUSCH coding chain. Therefore, if we consider the specification impact from the transmission signal processing schemes, there would be no additional block or function to be introduced or defined at least in 38.212 [4].

However, depending on the maximum possible number of users to be multiplexed in the same resources, DMRS design may need to be improved. To decode the packet from multiple UEs, gNB may utilize the DMRS for the detection of whether the uplink data packet is present in a common physical resource. Subsequently the gNB uses DMRS to perform the appropriate channel estimation and timing/frequency offset compensation. This indicates overall demodulation and decoding performance heavily relies on reliable channel estimation, especially in the presence of non-orthogonal transmissions from multiple UEs.

As discussed in our companion contribution [5], for either random selection of DMRS APs or random activation of potential UEs for NOMA transmission when the number of potential UEs is larger than MA signature pool size, it is inevitable to result in DMRS AP collision. When the DMRS collision probability is relatively high, NOMA decoding performance can be degraded substantially. In some cases, it can be observed that the decoding performance can be dominated by the DMRS collision probability. To overcome this issue and reduce potential DMRS collision, it may be more desirable to increase the number of orthogonal DMRS APs for UL NOMA transmission.

As defined in NR, the maximum number of orthogonal DMRS APs for PUSCH is 12 when 2-symbol front-loaded DMRS is employed. In this regards, the maximum number of orthogonal DMRS APs can be increased to 24, which may help in reducing DMRS collision between UEs. It should be noted that DMRS design improvement is needed regardless of which types of NOMA transmission scheme is introduced.

Observation 3

- No (or negligible) specification impact is needed for the LCRS transmission scheme

- DMRS enhancement is needed for uplink NOMA to reduce DMRS collision rate regardless of NOMA transmission schemes

# Conclusions

In this contribution, we introduced our proposed NOMA transmission scheme: Low Code Rate Spreading (LCRS). Based on the discussions presented, we summarize our views through the following observations and proposals:

Observation 1

- NOMA can be supported by applying UE-specific bit-level scrambling with negligible spec impact

Observation 2

- Low Code Rate Spreading (LCRS) is a promising candidate for NOMA from the perspectives of

  - Support of flexible code rate,

  - Support of (comparatively) full coding gain

  - Support of NOMA without introducing new transmission schemes

Observation 3

- No (or negligible) specification impact is needed for the LCRS transmission scheme

- DMRS enhancement is needed for uplink NOMA to reduce DMRS collision rate regardless of NOMA transmission schemes

Proposal 1

- LCRS is the baseline for the NOMA transmission scheme.

- DMRS enhancement needs to be considered for uplink NOMA to reduce DMRS collision rate regardless of NOMA transmission schemes.

# References

- Chairman’s notes, RAN1 94bis, Chengdu, China, October 2018

- T. Cover and J. Thomas, “Elements of Information Theory,” Wiley, 2006.

- 3GPP TS 38.211 V15.0.0, “Physical channels and modulation (Release 15)”

- 3GPP TS 38.212 V15.0.0, “Multiplexing and channel coding (Release 15)”

- R1-1812477, “NOMA related procedure”, Intel Corporation, Spokane, USA, November 2018