3GPP TSG RAN WG1 Meeting #95	R1-1812666

Spokane, USA, November 12th – 16th, 2018

Agenda Item:	7.2.1.5

Source:	Huawei, HiSilicon

Title:	LLS evaluations and observations in uRLLC

Document for:	Discussion and Decision

Introduction

In RAN1#92, the link-level evaluation parameters for NOMA have been agreed as in the Appendix. In RAN1#94bis, we presented some link-level evaluation results for URLLC scenario based on the agreed parameters [2]. In this contribution, we further present link-level evaluation results for URLLC scenario.

Link-level Evaluation

Evaluation method

In the following simulation, two type of numerologies are assumed. One is with SCS (Subcarrier Spacing) = 60 kHz, 7 OS (OFDM Symbol) with normal CP (cyclic prefix), and the bandwidth is 12 PRBs. The other is with SCS = 30 kHz, 4 OS with normal CP, and the bandwidth is 24 PRBs. Each UE is allocated with the whole bandwidth. One-symbol DMRS with NR Type-2 configuration is applied, which supports 6 orthogonal DMRS ports. To support more UEs than the number of orthogonal DMRS ports, more frequency comb patterns or cyclic shifts can be introduced. Both ICE (ideal channel estimation) and RCE (realistic channel estimation) are considered, and MMSE algorithm is applied for RCE. The number of transmit antennas is 1, and the number of receive antennas is 2 or 4.

Among the NOMA schemes proposed up to RAN1#94, we evaluate SCMA [3], MUSA [4], RSMA [5], and spreading-based NCMA [6], and present their performance for URLLC scenario. The overall evaluation methodology for URLLC is similar to that for mMTC as described in [7]. The methodology for URLLC establishes the target BLER as 0.1% and adopts PIC with 2 outer-loop iterations at the receiver. For each case, the optimal MCS, number of branches, and mapping length/spreading factor (SF) for each scheme could be different from that of the mMTC case.

Evaluation Results and Analysis

As expected, in all the figures, the required per UE SNR increases for higher # of multiplexed UEs for all compared transmission schemes. In the following, we will discuss for each case how the required SNR changes in terms of TBS, # of Rx antennas, and how it varies between different simulated schemes. More simulation results can be found in the Appendix.

SCS = 30KHz

Figure 1 presents the required per UE SNR versus # of multiplexed UEs when SCS is 30 KHz and the combination of MCS, SF, and # of branches is optimized for each simulation case.

In term of TBS (transport block size)

The curves of the smaller TBSs are flatter than those of the larger TBSs, which means the smaller the TBS is, the larger # of multiplexed UEs can be supported. For 1T2R,

- Less than 0.2 dB required-SNR increases for # of UE from 2 to 6 when TBS=10 bytes;

- More than 2.0 dB required-SNR increases for # of UE from 2 to 6 when TBS=75 bytes.

In term of # Rx Antennas

The curves of 4 Rx antennas are flatter than those of 2 Rx antennas, which means that more Rx antennas can support a larger # of multiplexed UEs.

- In the case of 1T2R, more than 2.0 dB required-SNR increases for # of UE from 2 to 6 when TBS=75 bytes.

- In the case of 1T4R, less than 0.2 dB required-SNR increases for # of UE from 2 to 6 when TBS=75 bytes.

In term of NOMA schemes

The performance of sequence spreading based NOMA schemes (i.e., MUSA, RSMA, NCMA) primarily relies on the SF and the number of branches. The performance of different sequence spreading based NOMA schemes depends on the SF and the branch configuration.

With a similar adaption of the SF and the number of branches, SCMA can provide larger SNR gains compared to the sequence spreading based NOMA schemes. The gap between the required SNR of SCMA and the best performing sequence spreading based NOMA scheme increases for higher TBS values. Specifically, our simulations results show that:

- In the case of 1T2R, the SNR gain of SCMA is up to 0.8 dB when TBS=40 bytes, 1.8 dB when TBS=60 bytes, and 2.3 dB when TBS=75 bytes.

- In the case of 1T4R, the SNR gain of SCMA is up to 0.6 dB when TBS=40 bytes, 1.3 dB when TBS=60 bytes, and 1.9 dB when TBS=75 bytes.

These results show that SCMA requires a lower SNR to serve the same # of UEs with the same TBS compared with sequence spreading based NOMA schemes.

In term of realistic channel estimation

The required SNR in terms of # of simultaneous UEs increases faster for RCE than that for ICE. This is because the channel estimation error will propagate during MUD and interference cancellation, and the performance degradation increases with the number of UEs.

However, the relative performance difference between the NOMA schemes does not change considerably for RCE as compared to the ICE and SCMA achieves superior performance compared to sequence spreading based NOMA schemes.

| (a). TDLA-30ns, 1T2R, ICE | (b). TDLA-30ns, 1T2R, RCE |
| --- | --- |
| (c). TDLA-30ns, 1T4R, ICE | (d). TDLA-30ns, 1T4R, RCE |
| (e). TDLC-300ns, 1T2R, ICE | (f). TDLC-300ns, 1T2R, RCE |
| (g). TDLC-300ns, 1T4R, ICE | (h). TDLC-300ns, 1T4R, RCE |

Figure 1. Required SNR versus # of UEs (30kHz SCS)

SCS = 60KHz

Figure 2 presents the required per UE SNR versus different # of multiplexed UEs when SCS is 60 KHz. The MCS, SF, and # of branch configuration is optimized for each simulation case.

In term of NOMA schemes

Similar to SCS = 30 KHz, SCMA can provide large SNR gains over sequence spreading based NOMA schemes. The achieved SNR gains of SCMA is up to 1.2 dB when TBS=60 bytes, and 2.0 dB when TBS=75 bytes. These gains shows the superior performance of SCMA design as compared to the sequence spreading based NOMA schemes.

In term of realistic channel estimation

Similar to the observations made for SCS=30 KHz with realistic channel estimation, SCMA achieves superior performance compared to the sequence spreading based NOMA schemes in SCS=60 KHz as well.

Observation 1: SCMA provide large SNR gain over sequence spreading based NOMA schemes in URLLC scenario for both SCS=30 KHz and SCS=60 KHz.

| (a). TDLA-30ns, 1T4R, ICE | (b). TDLA-30ns, 1T4R, RCE |
| --- | --- |
| (c). TDLC-300ns, 1T4R, ICE | (d). TDLC-300ns, 1T4R, RCE |

Figure 2. Required SNR versus # of UEs (60kHz SCS)

Receiver Impact

Chip-MMSE can be applied for all NOMA schemes. As discussed in [8], chip-MMSE can be regarded as a special case of EPA without inner loop iterations, and the complexity of chip-MMSE is much lower than that of block-MMSE, as the latter increases fast with the SF. The performance of SCMA, RSMA, and MUSA with chip-MMSE is shown in Figure 3 and 4. The results show that SCMA with chip-MMSE can perform better than sequence spreading based NOMA schemes with chip-MMSE receiver. The performance of SCMA with EPA and chip-MMSE, RSMA, and MUSA with block-MMSE is shown in Figure 5 and 6. The results show that SCMA with chip-MMSE can perform similar to linear spreading schemes with block-MMSE.

Observation 2: SCMA with chip-MMSE receiver can have better performance than sequence spreading based NOMA schemes with chip-MMSE receiver.

Observation 3: SCMA with chip-MMSE receiver can perform similar to sequence spreading based NOMA schemes with block-MMSE receiver.

| (a). TDLA-30ns, 1T2R, ICE | (b). TDLA-30ns, 1T2R, RCE |
| --- | --- |
| (c). TDLA-30ns, 1T4R, ICE | (d). TDLA-30ns, 1T4R, RCE |

Figure 3. Required SNR versus # of UEs (30kHz SCS)

| (a). TDLA-30ns, 1T4R, ICE | (b). TDLA-30ns, 1T4R, RCE |
| --- | --- |

Figure 4. Required SNR versus # of UEs (60kHz SCS)

| (a). TDLA-30ns, 1T2R, ICE | (b). TDLA-30ns, 1T2R, RCE |
| --- | --- |
| (c). TDLA-30ns, 1T4R, ICE | (d). TDLA-30ns, 1T4R, RCE |

Figure 5. Required SNR versus # of UEs (30kHz SCS)

| (a). TDLA-30ns, 1T4R, ICE | (b). TDLA-30ns, 1T4R, RCE |
| --- | --- |

Figure 6. Required SNR versus # of UEs (60kHz SCS)

When MPA is adopted as MUD, the receiver can achieve near optimal performance, even without outer-loop iterations, this advantage helps NOMA schemes with MPA receiver to meet both reliability and latency requirement in URLLC transmission. The computational complexity of MPA is to the order of O(Mdf), where M is the constellation size and df is the number of colliding UEs over each RE. The sparsity pattern and constellation design of SCMA can help to greatly reduce the complexity. As shown in Figure 7, SCMA with MPA receiver and no outer-loop iteration outperforms sequence spreading based NOMA schemes with Block-MMSE and outer-loop iteration.

Observation 4: SCMA with MPA receiver and no outer-loop iteration can outperform sequence spreading based NOMA schemes with Block-MMSE and outer-loop iteration.

| (a). TDLC-300ns, 6UE | (b). TDLA-30ns, 8UE |
| --- | --- |

Figure 7. BLER Performance of NOMA schemes (1T2R, ICE, 75Bytes, 30 kHz SCS)

Conclusions

In this contribution, we present the link-level evaluation results for URLLC scenario. Based on the discussion, we have following observation and proposal:

Observation 1: SCMA provide large SNR gain over sequence spreading based NOMA schemes in URLLC scenario for both SCS=30 KHz and SCS=60 KHz.

Observation 2: SCMA with chip-MMSE receiver can have better performance than sequence spreading based NOMA schemes with chip-MMSE receiver.

Observation 3: SCMA with chip-MMSE receiver can perform similar to sequence spreading based NOMA schemes with block-MMSE receiver.

Observation 4: SCMA with MPA receiver and no outer-loop iteration can outperform sequence spreading based NOMA schemes with Block-MMSE and outer-loop iteration.

- References

R1-1810121, “Evaluation results for the URLLC scenario”, Huawei, HiSilicon, RAN1#94bis, Chengdu, China, October 8th – 12th, 2018.

RAN1 Chairman’s notes, RAN1#94, Gothenburg, Sweden, August 20th – 24th, 2018.

R1-1808049, “Discussion on the design of SCMA”, Huawei, HiSilicon, RAN1#94, Gothenburg, Sweden, August 20th – 24th, 2018.

R1-1808155, “Link-level simulation results for mMTC scenario”, ZTE, RAN1#94, Gothenburg, Sweden, August 20th – 24th, 2018.

R1-1809434, “Transmitter side signal processing schemes for NOMA”, Qualcomm, RAN1#94, Gothenburg, Sweden, August 20th – 24th, 2018.

R1-1808499, “Transmitter side signal processing schemes for NCMA”, LGE, RAN1#94, Gothenburg, Sweden, August 20th – 24th, 2018.

R1-1812665, “Evaluation results for the mMTC scenario”, Huawei, HiSilicon, RAN1#95, Spokane, USA, November 12th – 16th, 2018.

R1-1812188, “Discussion on the design of NOMA receiver”, Huawei, HiSilicon, RAN1#95, Spokane, USA, November 12th – 16th, 2018.

- Appendix

- A.1 Link-level evaluation parameters

Table 1 Link-level evaluation assumptions

| Parameters | mMTC | URLLC |  | eMBB |  | Further specified values |
| --- | --- | --- | --- | --- | --- | --- |
| Carrier Frequency | 700 MHz | 700 MHz or 4 GHz |  | 4 GHz, 700 MHz as optional |  |  |
| Waveform (data part) | CP-OFDM and DFT-s-OFDM | CP-OFDM as starting point |  | CP-OFDM as starting point |  |  |
| Channel coding | URLLC: NR LDPC eMBB: NR LDPC mMTC: NR LDPC |  |  |  |  | The choice of channel coding here is only for the performance evaluation purpose for NOMA study |
| Numerology (data part) | SCS = 15 kHz, #OS = 14 | Case 1: SCS = 60 kHz, #OS = 7 (normal CP), optionally 6 (ECP) Case 2: SCS = 30 kHz, #OS = 4 |  | SCS = 15 kHz #OS = 14 |  |  |
| Allocated bandwidth | 6 as the starting point | 12 as the starting point |  | 12 as the starting point |  | For high payload such as 75 bytes, larger number of RBs can be considered. |
| TBS per UE | At least five TBS that are [10, 20, 40, 60, 75] bytes. Other values higher than 10 bytes are not precluded. Lower than 0.1 bits/RE is optional | At least five TBS that are [10, 20, 40, 60, 75] bytes. Other values higher than 10 bytes are not precluded. |  | At least five TBS that are [20, 40, 80, 120, 150] bytes. Other values higher than 20 bytes are not precluded. |  | #bits per RE calculation does not include DMRS overhead (e.g., REs of one every 7 symbols for DMRS would not be used to carry the data) |
| Target BLER for one transmission | 10% | 0.1% |  | 10% |  |  |
| Number of UEs multiplexed in the same allocated bandwidth | To be reported by companies. |  |  |  |  | Companies are encouraged to perform evaluations with various number of UEs Note: refined set of numbers of UEs should be further discussed in the next meeting. |
| BS antenna configuration | 2 Rx or 4 Rx for 700MHz, 4Rx or 8 Rx for 4 GHz 8Rx as optional |  |  |  |  | CDL model in 38.901 should be considered for 8Rx |
| UE antenna configuration | 1Tx |  |  |  |  |  |
| Propagation channel & UE velocity | TDL-A 30ns and TDL-C 300ns in TR38.901, 3km/h, CDL optional |  |  |  |  |  |
| Max number of HARQ transmission | 1 as starting point. |  | 1 as starting point. More values, 2 for URLLC can be used. |  | 1 as starting point. |  |
| Channel estimation | Ideal channel estimation results should be reported for calibration Realistic channel estimation |  |  |  |  |  |
| MA signature allocation (for data and DMRS) | Fixed/Random |  |  |  |  | Proponents report the details of random MA signature allocation (whether without or with collision) |
| Distribution of avg. SNR | Both equal and unequal | Equal |  | Both equal and unequal |  | Uniform discrete values for unequal case,, range [x - a, x + a] (dB) with 1 dB step, where x is the average SNR among UEs, and the deviation [a=3] |
| Timing offset | 0 as starting point. For grant-free without perfect TA, value is TBD |  |  |  |  |  |
| Frequency error | 0 as starting point. The value(s) is TBD. |  |  |  |  |  |
| Traffic model for link level | Full buffer as starting point. Non-full-buffer model (like Poisson arrival of fixed packet size) is optional. |  |  |  |  |  |
| For link level calibration purpose only | OMA single user whose spectral efficiency is the same as per UE SE in NOMA. AWGN curves can be provided also. |  |  |  |  |  |

Note: for the case when a parameter has a “OR” condition, companies are encouraged to evaluate all the corresponding values.