3GPP TSG-RAN WG1 Meeting #95 			R1-1813147

Spokane, USA, 12 - 16 Nov 2018

Agenda item:	7.2.1.1

Title:	Sparse regular signature design for NOMA

Source:	Fraunhofer HHI, Fraunhofer IIS

Document for:	Discussion

1. Introduction

Non-orthogonal multiple access gained high interest in academia and standardization [1], as an alternative to orthogonal multiple access (OMA) for 5G-NR to support massive connectivity and increase system capacity. In the RAN#80 meeting, the study item on non-orthogonal multiple access for NR-Rel.16 has been approved [2]. The detailed objectives for this study have been identified as (1) transmitter / (2) receiver – side processing, (3) NOMA – related procedures and (4) evaluation methodology. Low-density code-domain (LDCD) NOMA is a prominent sub-category of signature-based multiplexing, which relies on low-density signatures (LDS).

Motivated by theoretical analyses [5][6], it was proposed in [3] and further discussed in [4] to study scalable symbol-to-resource mapping –designs, which can be parameterized by a large number of system parameters and network KPIs, (e.g. overload) or service requirement (e.g. latency) to support regular low-density spreading signatures for NOMA. In this document, we specify the design criteria of or regular sparse-spreading matrices and propose a specific mappings. Significant receiver complexity reduction can be achieved by utilizing e.g. message-passing algorithms (MPAs), which enable user separation even when the received powers are comparable (as opposed to power-domain NOMA). Sparse mapping comes with the possibility to trade-off the coding gain and multi-user interference mitigation capability by adjusting the size of the mapping block, and the UE-specific sparsity levels and patterns.

2. Regular-sparse resource mapping

A NOMA system with sparse-resource mapping, K users sharing N resources in a non-orthogonal fashion, where each user only access a fraction of the available resources. The pattern how the non-zeros are distributed can be generally expressed in terms of a matrix F, as depicted in Figure 1. It is noted that these pattern can relate to

- Resource elements (RE) considering a general signature (LDCD) - NOMA scheme, or

- Physical Resource Blocks (PRB), considering sparse coded-access - NOMA

As discussed in in [3] and [4], a design parameter of such a mapping is the “sparsity”, i.e. the number of non-zero elements (given as an integer value smaller than the number of resources N). The overloading factor is given as β=N/K.

In practice, it is important to have a flexible design of the matrix F, with appropriate dimension scaling, which allows to trade-off different service requirements such as latency, reliability and spectral efficiency in a flexible manner, making it appropriate for both unscheduled transmissions targeting mMTC scenarios, as well as scheduled transmission targeting eMBB and URLLC scenarios. An example is depicted in Fig. 1, where the explicit (sparse) resource mapping is represented as a binary matrix F.

Figure 1 Example of UE-specific sparse resource mapping. The rows correspond to the resources, and the columns represent the UE-specific signatures. In the example, 9 users in total are mapped to 6 resources, where 3 users access the same resource, and each user accesses 2 resources. The corresponding overloading factor is β=9/6.

A regular-sparse resource mapping is characterized by a fixed number of ones in each column of the matrix F, which is beneficial from both theoretic perspective and receiver complexity point of view. On top of this, it could be beneficial to fix the maximum number of users that simultaneously overlap on each resource. The presented examples have the special property that besides having the number k of ones per column fixed, they also have a fixed number n of ones per row. In addition the maximum overlap between any two columns is 1, meaning in practice that no two users will overlap simultaneously on more than one resource. Based on the desired properties, two regular-sparse mappings are specified in Table 1 and Table 2.

| PRB / RE | User / Layer ID |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 00 | 01 | 02 | 03 | 04 | 05 | 06 | 07 | 08 |
| 1 | 1 | 0 | 0 | 0 | 0 | 1 | 0 | 1 | 0 |
| 2 | 0 | 1 | 0 | 1 | 0 | 0 | 0 | 0 | 1 |
| 3 | 0 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 0 |
| 4 | 1 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 1 |
| 5 | 0 | 1 | 0 | 0 | 0 | 1 | 1 | 0 | 0 |
| 6 | 0 | 0 | 1 | 1 | 0 | 0 | 0 | 1 | 0 |

Table 1 Regular-sparse resource mapping / signature design: β=9/6; k = 2; n = 3.

| PRB / RE | User / Layer ID |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 00 | 01 | 02 | 03 | 04 | 05 | 06 | 07 | 08 | 09 | 10 | 11 | 12 | 13 | 14 | 15 |
| 1 | 1 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 1 |
| 2 | 0 | 1 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 1 | 0 |
| 3 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 1 | 1 | 0 | 0 | 0 | 0 | 1 | 0 | 0 |
| 4 | 0 | 0 | 0 | 1 | 0 | 0 | 1 | 0 | 0 | 1 | 0 | 0 | 1 | 0 | 0 | 0 |
| 5 | 1 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 1 | 0 | 1 | 0 | 0 |
| 6 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 1 | 0 | 1 | 0 | 0 | 0 |
| 7 | 0 | 0 | 1 | 0 | 1 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 1 |
| 8 | 0 | 0 | 0 | 1 | 0 | 1 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 1 | 0 |
| 9 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 1 | 0 | 0 | 0 | 0 | 1 | 0 |
| 10 | 0 | 1 | 0 | 0 | 0 | 0 | 1 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 1 |
| 11 | 0 | 0 | 1 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 0 | 0 | 0 |
| 12 | 0 | 0 | 0 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 1 | 0 | 0 |

Table 2 Regular-sparse resource mapping / signature design: β=16/12; k = 3; n = 4.

Observation 1: Regular-sparse designs for sparse-coded NOMA, which control both the number of collisions on the involved resources, as well as the overlap between any pair of users, are desirable.

Proposal 1: Further study regular-sparse designs / low-density signatures for NOMA.

3. Summary and Discussion

The sparse-regular property gives signatures with small density, which support decoding algorithms with low computational complexity. There are significant savings in storage requirements as the generation of the code signatures requires only the storage of cyclic permutations. Further, the construction can be combined with other code-domain NOMA, both with sparse spreading (such as SCMA and LDS), and dense spreading. The construction naturally incorporates user transmission with random activation, making it also amenable for a use as a grant-free scheme for non-orthogonal random access.

Observation 1: Regular-sparse designs for sparse-coded NOMA, which control both the number of collisions on the involved resources, as well the overlap between any pair of users, are desirable.

Proposal 1: Further study regular-sparse designs / low-density signatures for NOMA.

References

Y. Chen et al., "Toward the Standardization of Non-Orthogonal Multiple Access for Next Generation Wireless Networks," in IEEE Communications Magazine, vol. 56, no. 3, pp. 19-27, MARCH 2018.

RP-181403, “Revision of Study on 5G Non-orthogonal Multiple Access”, ZTE, June 2018

R1-1808955, “Discussion on General Symbol-to-Resource Mapping for NR-NOMA”, HHI, Aug 2018

R1- 1811098, “UE-Specific Regular-Sparse RE Mapping for NR-NOMA”, HHI, Okt. 2018

O. Shental, B. M. Zaidel and S. S. Shitz, "Low-density code-domain NOMA: Better be regular," 2017 IEEE International Symposium on Information Theory (ISIT), Aachen, 2017

B. M. Zaidel, O. Shental and S. S. Shitz, "Sparse NOMA: A Closed-Form Characterization," 2018 IEEE International Symposium on Information Theory (ISIT)

M. T. P. Le, G. C. Ferrante, T. Q. S. Quek and M. Di Benedetto, "Fundamental Limits of Low-Density Spreading NOMA With Fading," in IEEE Transactions on Wireless Communications, vol. 17, no. 7, pp. 4648-4659, July 2018

M. Taherzadeh, H. Nikopour, A. Bayesteh, and H. Baligh, "SCMA Codebook Design," in 2014 IEEE 80th Vehicular Technology Conference (VTC2014-Fall), Sept 2014, pp. 1–5.

H. Nikopour and H. Baligh, "Sparse code multiple access," in 2013 IEEE 24th Annual International Symposium on Personal, Indoor, and Mobile Radio Communications (PIMRC), Sept 2013, pp. 332–336.