3GPP TSG RAN WG1 Meeting 95		 R1-1813404

Spokane, WA, U.S.A, November 12th – 16th, 2018

Agenda item:	7.1.7

Source: 	Qualcomm Incorporated

Title: 	NR Features and Capabilities

Document for:	Discussion/Decision

# Discussion

In this document, we discuss proposed changes to the NR UE feature list.

## Beam correspondence

Optional/mandatory for beam correspondence is undecided.

2-20 Beam correspondence

| 2-20 | Beam correspondence | 1. Support Beam correspondence 2. When CA is configured, whether the same beam correspondence relationship for beam management is supported across CCs. Note: RAN4 to check the feasibility for component-2, e.g. intra-band or inter-band |  | Yes | Beam correspondence is not supported | Type 1 | N.A. | N.A. |  | Note: Beam correspondence means each Tx port can be beamformed in a desirable direction but does not imply setting phase across ports |  | Mandatory with capability signaling for FR2 Not supported in FR1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 1:

Beam correspondence is mandatory with capability signaling in FR2, not supported in FR1.

## SRS-based UL beam management

Feature 2-30 is currently named Uplink beam management, although it only covers the version that primarily relates to UEs without beam correspondence capability. This feature should be optional.

2-30 UL beam management

| 2-30 | SRS-based uplink beam management | 1 Support of SRS based beam management 2. Supported max number of SRS resource per set (SRS set use is configured as for beam management). 3. Supported max number of SRS resource sets (SRS set use is configured as for beam management). |  | Yes | Uplink beam management is not supported | Type1 | N.A. | N.A. |  |  |  | Optional, with capability signaling Component-2, candidate value set is {2, 4, 8, 16} Component-3, candidate value set is {from 1 to 8} |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 2:

SRS-based uplink beam management should be optional with capability signaling.

## No analog beamforming in FR1

2-26 and 2-27 are analog beamforming related and are not applicable to FR1.

| 2-26 | Receiving beam selection using CSI-RS resource repetition "ON" | 1. Support Rx beam switching procedure using CSI-RS resource repetition "ON" 2. Recommended CSI-RS resource repetition number per resource set, |  | Yes | Rx beam switching is not supported | Type 1 | N.A. | N.A. |  |  |  | Support Rx beam switching is mandatory for bands at least > 6GHz Componet-2: candidate value set {2, 3, 4, 5, 6, 7, 8} Not supported in FR1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2-27 | Beam switching | 1. Maximum number of Tx + Rx beam changes a UE can conduct during a slot across the whole band CC BB_Total,. This number is defined as per SCS Note: it is assumed that spec enable the possibility to restrict the same beam across intra-band CCs | 2-24 | Yes. | No restriction on the maximum number of Tx+Rx beam change for a slot | Type 1 | N.A. | N.A |  |  |  | Only applicable to FR2 Candidate value set: {4, 7, 14} |

Proposal 3:

For FR1, there is no need to define features 2-26 and 2-27.

## Number of TCI states for PDSCH

The number of configurable TCI states is not yet decided. Set the minimum supported to 64. In addition, add a note saying that when a TCI state for PDSCH includes QCL-TypeD in FR1, the UE should ignore the spatial QCL information.

| 2-4 | TCI states for PDSCH | 1. Support number of active TCI states per BWP per CC , including control and data 2. maximum number of configured TCI states per CC for PDSCH | 2-1 | Yes | Only one TCI state can be supported | Type 1 | N.A. | N.A. |  | Note: UE is required to track only the active TCI states Note: when a TCI state includes QCL-TypeD in FR1, the UE ignores the spatial QCL information |  | Mandatory with capability Component-1: Candidate value set: {1, 2, 4, 8} Component-2: candidate value set: {4, 8, 16, 32, 64, 128} UE is mandated to signal 64 for FR2. For FR1, UE is mandated to report at least the max number of allowed SSB in the band |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 4:

For FR2, the UE is mandated to signal at least 64 configured PDSCH TCI states per CC.  For FR1, when a TCI state includes QCL-TypeD in FR1, the UE ignores the spatial QCL information.

## CSI-RS reception

There is a conflict between the value range of Component-4 and Component-4a. For example, when Component-4a is set to 1 and the UE supports 1 DL CC, the value of Component-4 should be also 1. Correct this by ignoring Component-4a when conflict occurs. In addition, for FR1, when a CSI-RS resource configuration includes QCL-TypeD, the UE ignores the spatial QCL information, since analog beamforming restriction doesn’t apply.

| 2-33 | CSI-RS and CSI-IM reception for CSI feedback | 1. Supported max # of configured NZP-CSI-RS resources per CC, 2. Supported max # of ports across all configured NZP-CSI-RS resources per CC 3. Supported max # of configured CSI-IM resources per CC 4. Supported max # simultaneous NZP-CSI-RS resources in active BWPs across all CCs 4a. Supported max # simultaneous NZP-CSI-RS resources per CC 5. Supported max total # of CSI-RS ports in simultaneous NZP-CSI-RS resources in active BWPs across all CCs 5a. Supported max total # of CSI-RS ports in simultaneous NZP-CSI-RS resources per CC | 2-32 | Yes | CSI acquisition is not supported | Type 3 | N.A. | N.A. |  | Note: all the candidate values are the range of capability signaling which doesn’t determine whether UE is mandatory to support all the signaling values. Note: when a CSI-RS resource configuration includes QCL-TypeD in FR1, the UE ignores the spatial QCL information Note: Component-4 value is ignored when the number of configured CCs times Component-4a value is less than Component-4 |  | Component-1 candidate values: {from 1 to 32} Component-2 candidate values: {2, 4, 8, 12, 16, 24, 32, 40, 48 … ,256} Component-3: candidate values: {1,2,4,8,16,32} Component-4: candidate values {5, 6, 7, 8, 9, 10, 12, 14, 16, …, 62, 64} (includes all even numbers between 16 and 64) Component-4a: candidate values {1, 2, 3 … 32} Component-5: candidate values {8, 16, 24, …, 248, 256} Component-5a: candidate values {8, 16, 24, … 128 } |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 5:

There is a conflict between the value range of Component-4 and Component-4a.  When conflict occurs, ignore the Component-4 value.  For FR1, when a CSI-RS resource configuration includes QCL-TypeD, the UE ignores the spatial QCL information.

## Beam failure detection and recovery

A UE not supporting CSI-RS based RLM (FG 1-7) is not expected to perform CSI-RS based BFD, since the underlying functionality is identical.

| 2-31 | Beam failure recovery | 1. Maximal number of CSI-RS resources across all CCs for UE to monitor PDCCH quality 2. Maximal number of different SSBs across all CCs for UE to monitor PDCCH quality 3. Maximal number of different CSI-RS and/or SSB resources across all CCs for new beam identifications. 4. COREST recoveryControlResourceSetId if configured | 1-7 for CSI-RS based BFD/BFR | Yes | Beam failure recovery is not supported | Type 1 | N.A. | N.A. |  | Component 4) is in addition to component 1) of 3-1 |  | Component-1 candidate value set: {from 1 to 16} Component-2 candidate: {from 1 to 16} Component-3: Candidate value set is: {from 1 to 128} [UE is mandated to support at least 64.] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 6:

Add FG 1-7 as a prerequisite for CSI-RS based BFD and BFR.

## Rate matching

DM-RS and PT-RS rate matching was not included in the current wording.

2-33a Supported PDSCH RE-mapping patterns

| 2-33a | Supported PDSCH RE-mapping patterns | 1. Supported max # of RE mapping patterns, each pattern can be described as a resource (including NZP/ZP CSI-RS, DM-RS, PT-RS and CRS, CORESET and SSB and bitmap configured in 5-26/27/27a) Note: patterns are counted as per symbol per CC 2. Supported max # of RE mapping patterns, each pattern can be described as a resource (including NZP/ZP CSI-RS, DM-RS, PT-RS and CRS, CORESET and SSB and bitmap configured in 5-26/27/27a) Note: patterns are counted as per slot per CC |  | Yes | PDSCH RE mapping is not supported | Type 4 | No need | Yes |  |  |  | Mandatory with capability signaling candidate values: {10, 20} for FR1 {6, 20} for FR2 Compponent-2 candidate values: {from 16: 16: 256} for FR1 {16: 16: 256} for FR2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 7:

Include DM-RS, PT-RS and 5-27a in the number of rate matching patterns.

## DL mini-slot capability

With the last change, the inclusion of PDSCH for Message 4 was deleted. This should be added back.

5-11 Up to 2 unicast PDSCHs per slot for different TBs for UE processing time Capability 1

| 5-11 | Up to 2 unicast PDSCHs per slot for different TBs for UE processing time Capability 1 | Up to 2 unicast PDSCHs per slot only in TDM is supported for Capability 1 1) PDSCH(s) for Msg. 4 is included |  | Yes |  | Type 3 | N.A. | N.A. |  | This capability is necessary for each SCS |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 8:

Add back the condition that Message 4 is counted as part of the limit on number of TBs per slot.

## Remaining decisions on optional/mandatory

In the feature list, there are missing definitions for optional/mandatory for several features. We propose to add the following.

| 1-8 | RLM based on a mix of SS block and CSI-RS signals |  | 1-4 and 1-7 | Yes | UE does not support RLM based on a mix of SS block and CSI-RS signals | Type 4 | No need | No need |  |  | RAN1 | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2-34 | NZP-CSI-RS based interference measurement | 1. Support NZP-CSI-RS based interference measurement FFS whether to add the number of NZP-CSI-RS port for interference measurement | 2-33 | Yes | NZP-CSI-RS based interference measurement is not supported | Type 4 | No need | No need |  |  |  | Optional with capability signaling |
| 2-56 | SRS carrier switch | 1. Report inter-cell switching time capability | 2-53 | Yes | SRS carrier switch is not supported | Type 1 | No need | N.A. |  | RAN4 reply LS, R1-1805817, includes candidate value sets | RAN1/4 | Optional with capability signaling candidate values set is up to RAN4 |
| 6-16 | Supplemental uplink | 1) RACH, PUSCH, PUCCH, SRS operations in a band combination including SUL 2) Supplemental uplink with same numerology between SUL and non SUL carriers | 6-15 | Yes | UE will not be able to perform the RACH/PUSCH/PUCCH/SRS operation in a band combination including SUL | N.A. | N.A. | N.A. |  | This is conditioned on the support of SUL band combination(s). The band combination definition is up to RAN4. |  | Optional with capability signaling |
| 6-18 | Supplemental uplink with dynamic switch | DCI based selection of PUSCH carrier | 6-16 | Yes |  | Type 3 | N.A. | N.A. |  | This is conditioned on the support of SUL band combination(s). The band combination definition is up to RAN4. |  | Optional with capability signaling |

Proposal 9:

Resolve remaining definitions of optional/mandatory as noted in the tables in this contribution.

# Conclusions

In this contribution, the following proposals have been made:

Proposal 1:

Beam correspondence is mandatory with capability signaling in FR2, not supported in FR1.

Proposal 2:

SRS-based uplink beam management should be optional with capability signaling.

Proposal 3:

For FR1, there is no need to define features 2-26 and 2-27.

Proposal 4:

For FR2, the UE is mandated to signal at least 64 configured PDSCH TCI states per CC. For FR1, when a TCI state includes QCL-TypeD in FR1, the UE ignores the spatial QCL information.

Proposal 5:

There is a conflict between the value range of Component-4 and Component-4a. When conflict occurs, ignore the Component-4 value. For FR1, when a CSI-RS resource configuration includes QCL-TypeD, the UE ignores the spatial QCL information.

Proposal 6:

Add FG 1-7 as a prerequisite for CSI-RS based BFD and BFR.

Proposal 7:

Include DM-RS, PT-RS and 5-27a in the number of rate matching patterns.

Proposal 8:

Add back the condition that Message 4 is counted as part of the limit on number of TBs per slot.

Proposal 9:

Resolve remaining definitions of optional/mandatory as noted in the tables in this contribution.

# References

- R1-1805728, “NR UE feature list”, NTT DOCOMO, AT&T

- R1-1800886, “NR features and capabilities”, Qualcomm

- R1-1802849, “NR features and capabilities”, Qualcomm

- R1-1807374, “NR features and capabilities”, Qualcomm

- R1-1809998, “NR UE feature list”, NTT DOCOMO, AT&T

- R1-1809433, “NR Features and Capabilities”, Qualcomm

- R1-1811242, “NR Features and Capabilities”, Qualcomm

- RP-181707, “NR UE Features”, Qualcomm

- RP-182036, “NR UE Feature list”, NTT DOCOMO

- R1-1812064, “NR UE feature list”, NTT DOCOMO, AT&T