3GPP TSG-RAN WG1 #95	 R1- 1813869

Spokane, United States, November 12th – 16th 2018

Agenda Item:	7.2.10

Source:	Ericsson, Intel Corporation

Title:	Text proposal for deployment scenarios and evaluation methodologies for NR positioning

Document for:	Discussion, Decision

# 1	Discussion

The NR positioning study item [1] will address positioning solutions for both regulatory and commercial services. The outcome of the study will be documented in the technical report 38.855 [2], which version 0.0.1 has been already agreed in RAN1#94bis.

In this contribution, we provide text proposal for Section 5 “Rel. 16 NR Positioning Requirements” and Section  6 “Deployment Scenarios and Evaluation Methodologies for NR Positioning” of the TR, based on the already agreed discussions on requirements, simulation parameters and scenario descriptions agreed in RAN1#94bis.

Proposal 1	Add the text proposal in Appendix A.1 to the TR.

# Conclusion

We have the following proposal:

Proposal 1	Add the text proposal in Appendix A.1 to the TR.

# References

[1] RP-182155, “Study on NR positioning support”, RAN1, Intel Corporation, Ericsson

[2] TR 38.855, “Study on NR positioning support”, v.0.0.1, Rel.16.

# Appendix

## A.1 	Text Proposal to the TR 38.855

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1] 	RP-182155, “SID, “Study on NR positioning support”

[2]	3GPP TR 21.905, “Vocabulary for 3GPP Specifications”.

[3]	3GPP TS 22.071, “Location Services (LCS); Service description; Stage 1”.

[4]	3GPP TR 22.261, “Service requirements for next generation new services and markets”

[5]	3GPP TR 22.804, “Study on Communication for Automation in Vertical domains (CAV)”

[6]	SP-170589, “New SID on Study on positioning use cases”

[7]	3GPP TR 22.872, “Study on positioning use cases”

[8]	3GPP TR 38.913, “Study on scenarios and requirements for next generation access technologies”

[9]	3GPP TR 22.862, “Feasibility study on new services and markets technology enablers for critical communications; Stage 1”

# 3	Definitions and Abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [2] and the following apply. 
A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [2].

Definition format (Normal)

<defined term>: <definition>.

example: text used to clarify abstract rules by applying them literally.

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [2] and the following apply. 
An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [2].

CID	Cell ID

CDF	Cumulative Distribution Function

ECID	Enhanced Cell ID

GNSS	Global Navigation Satellite System

LCS	LoCation Services

LPP	LTE Positioning Protocol

OTDOA	Observed Time Of Arrival

TBS	Terrestrial Beacon System

TTFF	Time To First Fix

UTDOA	Uplink Time Of Arrival

WLAN	Wireless Local Area Network

# Rel. 16 NR Positioning Requirements

Regulatory requirements are considered as a minimum performance targets for NR Positioning studies. Additional requirements based on commercial use cases can be used as input performance targets that are subject to further analysis in terms of performance/ complexity tradeoffs in different evaluation scenarios. For regulatory use cases, the following requirements are considered as a minimum performance targets for NR positioning​:

- Horizontal positioning error <= 50m for 80% of Ues

- Vertical positioning error [<5 m] for [80%] of UEs​ [Note: The regulatory requirements refer to floor level vertical accuracy

- End to end latency and TTFF < 30 seconds​

As a starting point for commercial use cases, the following requirements are considered as performance targets for RAT dependent solutions, which are subject to further analysis in terms of performance/ complexity tradeoffs of NR positioning radio-layer solutions​:

- Horizontal positioning error < [3]m for [80]% of UEs in indoor deployment scenarios

- Vertical positioning error < [3]m for [80]% of UEs in indoor deployment scenarios​

- Horizontal positioning error < [10]m for [80]% of UEs in outdoor deployments scenarios

- Vertical positioning error < [3]m for [80]% of UEs in outdoor deployment scenarios​

- End to end latency < [1]s​

Note: This does not eliminate more or less demanding commercial use cases.

# 6 Deployment Scenarios and Evaluations Methodologies for NR Positioning

## 6.1	Scenarios and System Parameters for Positioning Evaluations

### 6.1.1	Evaluation scenarios

For evaluating baseline performance, scenarios (with various options/configurations) are defined below for RAT-dependent positioning techniques for NR positioning study:

Scenario 1. Indoor Office for FR1 and FR2 (Open office and Mixed Office)​

Scenario 2. UMi street canyon for FR1 and FR2 (ISD 200m)​

Scenario 3. UMa (ISD 500m, TBD: ISD > 500m) for FR1 only (Macro cell only deployment scenario)

Table 6.1.1-1: Common scenario parameters applicable for all scenarios

|  | FR1 Specific Values | FR2 Specific Values |
| --- | --- | --- |
| Carrier frequency, GHz | 2GHz, 4GHz – Note 1 | 30 GHz – Note 1 |
| Bandwidth, MHz | 5MHz, 50MHz for 2GHz 100MHz for 4GHz | 100MHz, 400MHz |
| Subcarrier spacing, kHz | 15kHz for 5MHz and 50MHz 30kHz for 100MHz | 120kHz |
| gNB model parameters |  |  |
| gNB noise figure, dB | 5dB | 7dB |
| UE max. TX power, dBm | 23dBm – Note 1 | 23dBm – Note 1 EIRP should not exceed 43 dBm. |
| UE model parameters |  |  |
| UE noise figure, dB | 9dB – Note 1 | 13dB – Note 1 |
| UE antenna configuration | Panel model 1 – Note 1 Mg = 1, Ng = 1, P = 2, dH = 0.5λ, ​ (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1) | Multi-panel Configuration 1 and Panel Configuration a – Note 1 - Multi-panel Configuration 1: (Mg, Ng) = (1, 2); Θmg,ng=90°; Ω0,1=Ω0,0+180°; (dg,H, dg,V)=(0,0)​ - Panel Configuration a:​ - Each antenna array has shape dH=dV=0.5λ​ - Config a: (M, N, P) = (2, 4, 2), ​ - the polarization angles are 0° and 90°​ - The antenna elements of the same polarization of the same panel is virtualized into one TXRU​ - Optional: Provided by company |
| UE antenna radiation pattern | Omni, 0dBi | Antenna model according to Table 6.1.1-2 |
| PHY/link level abstraction | Explicit simulation of all links, individual parameters estimation is applied. Companies to provide description of applied algorithms for estimation of signal location parameters. |  |
| Network synchronization | The network synchronization error, per UE dropping, is defined as a truncated Gaussian distribution of (T1 ns) rms values between an eNB and a timing reference source which is assumed to have perfect timing, subject to a largest timing difference of T2 ns, where T2 = 2*T1 – That is, the range of timing errors is [-T2, T2] – T1: 0ns (perfectly synchronized), 50ns |  |
| Note 1: According to 3GPP TR 38.802 Note 2: According to 3GPP TR 38.901 |  |  |

Table 6.1.1-2: UE antenna radiation pattern model 1 (FR2)

| Parameter | Values |
| --- | --- |
| Antenna element radiation pattern in θ'' dim (dB) |  |
| Antenna element radiation pattern in φ'' dim (dB) |  |
| Combining method for 3D antenna element pattern (dB) |  |
| Maximum directional gain of an antenna element GE,max | 5dBi |
|  | Note: are in local coordinate system. |

Table 6.1.1-3: Scenario 1. Indoor office scenario parameters

|  | FR1 Specific Values | FR2 Specific Values |
| --- | --- | --- |
| Layout | Indoor floor: (12BSs per 120m x 50m), TRP number per floor:12, Inter-gNB distance = 20m - Note 1 |  |
| Total gNB TX power, dBm | 24dBm | 24dBm EIRP should not exceed 58 dBm |
| gNB antenna configuration | (M, N, P, Mg, Ng) = (4, 4, 2, 1, 1), dH=dV=0.5λ – Note 1 | (M, N, P, Mg, Ng) = (4, 8, 2, 1, 1), dH=dV=0.5λ – Note 1​ One TXRU per polarization per panel is assumed |
| gNB antenna radiation pattern | Single sector – Note 1 | 3-sector antenna configuration – Note 1 |
| Channel model | Indoor open office – Note 2 |  |
| Peneteration loss | 0dB |  |
| Number of floors | 1 |  |
| UE drop procedure | 100% indoor, uniformly distributed over the horizontal area |  |
| UE mobility | 3km/h |  |
| UE antenna height | 1.5m |  |
| Min gNB-UE distance (2D), m | 0m |  |
| gNB antenna height | 3m |  |
| Note 1: According to 3GPP TR 38.802 Note 2: According to 3GPP TR 38.901 |  |  |

Table 6.1.1-4: Scenario 2: Urban micro (UMi) scenario parameters

|  | FR1 Specific Values | FR2 Specific Values |
| --- | --- | --- |
| Layout | Hexagonal grid, 19 or 7 macro sites, 3 sectors per site, ISD = 200m, Note 1 Wrap-around is applied​, Note 2 |  |
| Total gNB TX power, dBm | 44dBm | 37dBm per panel EIRP should not exceed 73dBm |
| gNB antenna configuration | (M, N, P, Mg, Ng) = (8, 8, 2, 1, 1), (dH, dV) = (0.5, 0.8)λ, - Note 1 | (M, N, P, Mg, Ng) = (4, 8, 2, 2, 2), (dH, dV) = (0.5, 0.5)λ, (dg,H,dg,V) = (4.0, 2.0)λ, - Note 1 |
| gNB antenna radiation pattern | Directional, 8dBi – Note 1 Table 6.1.1-5 | Directional, 8dBi – Note 1 Table 6.1.1-5 |
| Channel model | UMi Street Canyon – Note 3 |  |
| Penetration loss | For outdoor UEs: 0dB |  |
| Number of floors | All UEs are on the ground. |  |
| UE drop procedure | 100% outdoor uniformly distributed over the horizontal area |  |
| UE mobility (for modeling Doppler effects) | Outdoor: 3km/h |  |
| UE height, m | 1.5m |  |
| Min. gNB-UE distance (2D), m | 10m |  |
| gNB antenna height | 10 m by default. Companies can bring results with uniform distribution [5-20]m​ |  |
| Note 1: According to 3GPP TR 38.802 Note 2: In case if interference considerations are not properly taken into account for 7 sites companies are encouraged to provide results for 19 sites. Note 3: According to 3GPP TR 38.901 |  |  |

Table 6.1.1-5: gNB antenna patterns for UMi

| Parameter | Values |
| --- | --- |
| Antenna element vertical radiation pattern (dB) |  |
| Antenna element horizontal radiation pattern (dB) |  |
| Combining method for 3D antenna element pattern (dB) |  |
| Maximum directional gain of an antenna element GE,max | 8dBi |

Table 6.1.1-6: Scenario 3: Outdoor macro (UMa) scenario parameters

|  | FR1 Specific Values |
| --- | --- |
| Layout | Hexagonal grid, 3 sectors per site, 7 or 19 macro sites, ISD = 500m – Note 1 Wrap-around is applied. Note 2 |
| Total gNB TX power, dBm | 49dBm |
| gNB antenna configuration | (M, N, P, Mg, Ng) = (8, 8, 2, 1, 1), (dH, dV) = (0.5, 0.8)λ – Note 1 Applicable for 2GHz and 4 GHz carrier frequency. |
| gNB antenna radiation pattern | Directional, 8dBi – Note 1, Table 6.1.1-5 |
| Channel model | UMa scenario – Note 3 |
| Penetration loss | For outdoor UEs: 0dB For indoor UEs: 20dB+0.5d2D-in – Note 3 |
| Number of floors, (floor height) | 8, (3m) |
| Antenna Height: | Uniformly distributed [20-50] m – Note 4 25m + α, where α~uniform[-5, 25] |
| UE Height | 3(nfl – 1) + 1.5 m – Note 4 where, nfl ~ uniform(1,Nfl) and Nfl = 8 |
| UE dropping procedure | 50% indoor and 50% outdoor uniformly distributed over the horizontal area (separate statistic) |
| Min. gNB-UE distance (2D), m | 35m |
| UE mobility (for modeling Doppler effects) | For indoor UEs: 3km/h For outdoor UEs: 60km/h |
| Note 1: According to 3GPP TR 38.802 Note 2: In case if interference considerations are not properly taken into account for 7 sites companies are encouraged to provide results for 19 sites. Note 3: According to 3GPP TR 38.901 Note 4: According to 3GPP TR 37.857 |  |

  - Performance Metrics

For evaluating performance of NR positioning technologies, the following metrics should apply.

### 6.2.1	Horizontal accuracy

Horizontal accuracy is the difference between the calculated horizontal position and the actual horizontal position of a UE.   At least CDFs of horizontal positioning errors are used as a performance metrics in NR positioning evaluations​. At least the following percentiles of positioning error are analyzed 50%, 67%, 80%, 90%.

### 6.2.2	Vertical accuracy

Vertical accuracy is the difference between the calculated vertical position and the actual vertical position of a UE.  Vertical error is not necessarily applicable to all solutions and/or scenarios. In terms of applicability, the CDFs of vertical positioning errors are used as a performance metrics in NR positioning evaluations​. At least the following percentiles of positioning error are analyzed 50%, 67%, 80%, 90%.

### 6.2.3	Other metrics

Physical layer latency, UE power consumption, scalability/capacity, network deployment complexity, availability, UE and gNB complexity can be considered as important design factors for NR positioning solutions and can be evaluated analytically for proposed solutions.