3GPP TSG RAN1 #96		R1-1901707

Athens, Greece, 25th February – 1st March, 2019

Source:	vivo

Title:	Performance evaluation on multi PDCCH based multi TRP transmission

Agenda Item:	7.2.8.6

Document for:	Discussion and Decision

- Introductions

In RAN1 #94bis [1], the following agreements were achieved on performance evaluation for eMBB.

Agreement

For multi-TRP/panel performance evaluation:

  - For eMBB in FR1, 10MHz BW and 15kHz SCS are baseline.

  - For eMBB in FR1, 20MHz BW and 30kHz SCS are optional.

  - For eMBB in FR2, 80MHz BW and 120kHz SCS are baseline.

Agreement

For eMBB multi-TRP/panel performance evaluation, FTP traffic model 1 with packet size 0.5Mbytes as a baseline, and other traffic model is not precluded. RU=20/40/60% are baseline, and optional low RU (e.g. 5/10) can be considered.

Agreement

For eMBB multi-TRP/panel performance evaluation, MMSE IRC is the baseline, and advanced receiver is not precluded. Practical channel estimation and feedback model are used.

Agreement

- For eMBB multi-TRP performance evaluation, ideal and non-ideal backhaul are considered, the following delay values are assumed:

  - Ideal backhaul: 0ms

  - Non-ideal backhaul: 2ms, 5ms, 50ms(optional)

- Companies to provide the delay values used in their evaluations

Agreement:  SLS assumption for eMBB multi-TRP/panel enhancement

| Parameters | Dense urban (Macro Only) | Indoor hotspot |
| --- | --- | --- |
| Carrier frequency | 2GHz/4GHz is baseline (each company to choose 1 or more) 30GHz is optional | 4GHz is baseline, 30GHz is optional |
| Channel model | TR38.901 |  |
| TP antenna configuration | 4 ports: (M, N, P, Mg, Ng, Mp, Np) = (8,2,2,1,1,1,2) 16 ports: (M, N, P, Mg, Ng Mp, Np) = (8,4,2,1,1,2,4) (dH, dV) = (0.5, 0.8)λ for FR1 2 ports (8,8,2,1,1) and 8 ports (4,8,2,2,2) for 30GHz Other antenna configurations is not precluded (such as 32 ports) | 2 ports: (M, N, P, Mg, Ng) = (1,1,2,1,1) for 4GHz 2 ports: (4,4,2,1,1) for 30GHz Other antenna configurations is not precluded. |
| UE antenna configuration | 4Rx Port: (Baseline) (M,N,P,Mg,Ng,Mp,Np) = =(1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for FR1 For 30 GHz: (M, N, P, Mg, Ng) = (2, 4, 2, 1, 2); (dV, dH) = (0.5, 0.5)λ. (dg,V, dg,H) = (0, 0)λ. * Θmg,ng=90°; Ω0,1=Ω0,0+180 | 4Rx Port: (Baseline) (M,N,P,Mg,Ng,Mp,Np) = =(1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for 4GHz For 30 GHz: (M, N, P, Mg, Ng) = (2, 4, 2, 1, 2); (dV, dH) = (0.5, 0.5)λ. (dg,V, dg,H) = (0, 0)λ. * Θmg,ng=90°; Ω0,1=Ω0,0+180 |
| Coordination assumptions | Each company to provide details on cluster size, coordination scheme, etc |  |

Agreement

Baseline scheme to evaluate eMBB multi-TRP enhancements is DPS or single TRP

- Each company to provide the details on backhaul delay, CSI reporting, transmission scheme, scheduling, etc.

In RAN1 NR_AH1901 meeting [2], the following alternatives on resource allocation from multiple TRPs were listed

Agreement

For a UE supporting multiple-PDCCH based multi-TRP/panel transmission and each PDCCH schedules one PDSCH, at least for eMBB with non-ideal backhaul, down-select one alternative from following in RAN1 96

- Alt 1: the UE may be scheduled with full/partially/non-overlapped PDSCHs at time and frequency domain by multiple PDCCHs

- Alt 2:  the UE can be only scheduled with full/non-overlapped PDSCHs at time and frequency domain by multiple PDCCHs

- Alt 3: the UE may be scheduled with full/partially/non-overlapped PDSCHs at time and frequency domain by multiple PDCCHs with following restrictions:

  - Same DMRS configuration with respect to actual number of front loaded DMRS symbol(s), the actual number of additional DMRS, the DMRS symbol location and DMRS configuration type shall be assumed by the UE for full/partially overlapping PDSCHs.

  - The UE is not expected to have more than one TCI state with DMRS ports within the same CDM group for full/partially overlapping PDSCHs

  - Full scheduling information for receiving a PDSCH is indicated and carried only by the corresponding PDCCH.

Other restrictions are not excluded, for example BWP switching

In this paper we provide system-level simulation (SLS) results to compare DPS and DPS+NCJT in non-ideal backhaul cases.

- SLS evaluation

2.1 Simulation methodology

We perform performance evaluation for eMBB in FR1 with 10MHz BW and 15kHz SCS, 4GHz carrier frequency. Multi-TRP transmission with non-ideal backhaul is evaluated such that independent scheduling is assumed in each TRP per cluster. DPS is the baseline. SU-MIMO is applied for both DPS and DPS+NC-JT cases.

For Indoor Hotspot scenario of multi-TRP transmission, four neighboring TRPs forming a cluster. A UE with its associated TRP in the cluster measures the RSRP of all TRPs in the cluster and selects at most one candidate coordinating TRP, with the RSRP gap lower than a predefined threshold compared to the serving TRP.

- DPS

The UE satisfying the threshold compares the estimated throughput among the two possible DPS transmissions by its associated TRP and coordinating TRP. And then, UE reports the CSI with maximal estimated throughput to its recommended transmitting TRP. Rank 1 or 2 is reported for either TRP.

The scheduler per TRP in the cluster schedules one UE according to proportional fair algorithm without semi-statically configured resource allocation. With one optimal DPS CSI feedback, non-overlapping in time domain is achieved.

- DPS+NC-JT

The UE satisfying the threshold compares the estimated throughput among the two possible DPS transmissions, and one NC-JT from the two TRPs. And then, UE reports the CSI with maximal estimated throughput to its recommended transmitting TRP, i.e., one TRP for DPS transmission or both TRPs for NC-JT. If UE reports a NC-JT CSI, rank 1 or 2 is chosen per TRP to maximize the NC-JT estimated overall throughput.

The scheduler per TRP in the cluster schedules one UE according to proportional fair algorithm without semi-statically configured resource allocation. According to the non-ideal backhaul assumption, the scheduler of a TRP is not aware of the scheduling results of another TRP at the same time. With this dynamic independent scheduling, partially-overlapped PDSCHs can be achieved. One codeword per TRP is transmitted if the UE is scheduled as NC-JT.

For a UE receiving data from two TRPs in NC-JT manner, its equivalent channel is given by

He=H1, H2W100W2=H1W1, H2W2

where H1, H2 are channels from the two TRPs, and W1, W2 are the precoders of the two TRPs. Then the post-SINR per layer can be calculated assuming MIMO detection of the equivalent channel He.

Other simulation assumptions can be found in the Appendix.

2.2 Performance evaluation results and discussion

We provide UPT comparison for FTP model 1 with target RU set to 20% in Table 1. It can be seen compared to baseline DPS, DPS+NC-JT can achieve about 7% gain in mean UPT and 2.7% gain in 5% UPT.

However, we can observe that the cell average throughput of DPS+NC-JT is about 34% worse than that of DPS, although the gain of UPT is achieved by DPS+NC-JT. This is because in our simulation if two TRPs are transmitting to one UE in NC-JT manner, the resource is double-counted. This would affect the UPT of NC-JT, because doubled resource in NC-JT may not always obtain halved time duration to transmit the same size of packets which would increase RU. To keep the RU at the level of 20%, one have to set a lower packet arrival rate which surely lead to less packets and higher UPT.

To justify our understanding, we set the same packet arrival rate (λ) for DPS as that of DPS+NC-JT when RU is about 20%. The performance comparison are shown in Table 2. With the same packet arrival rate, RU of DPS is reduced to 13.3%. Almost same cell average throughput can be achieved by both schemes, considerable UPT gain can be observed at 50% UPT.

Table 1: DPS+NC-JT vs. DPS for Indoor Hotspot with non-ideal backhaul (same RU)

|  | Cell average throughput | Mean UPT | 5% UPT | 50% UPT | 95% UPT | RU |
| --- | --- | --- | --- | --- | --- | --- |
| DPS | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 19.8% |
| DPS+NC-JT | -34.0% | 7.0% | 2.7% | 17.1% | 0.0% | 19.1% |

Table 2: DPS+NC-JT vs. DPS for Indoor Hotspot with non-ideal backhaul (same λ)

|  | Cell average throughput | Mean UPT | 5% UPT | 50% UPT | 95% UPT | RU |
| --- | --- | --- | --- | --- | --- | --- |
| DPS | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 13.3% |
| DPS+NC-JT | -0.7% | 2.0% | -4.3% | 16.4% | 0.0% | 19.1% |

In order to fairly compare NC-JT with other single TRP schemes, RU statistics for NC-JT in the simulation needs to be clarified. If two TRPs are transmitting to one UE in the same time-frequency resource, whether the used resource is double counted or not?

Observation 1: With target RU ≈ 20%, DPS+NC-JT can achieve about 7% gain in mean UPT and 2.7% gain in 5% UPT for non-ideal backhaul, but the cell average throughput of DPS+NC-JT is about 34% worse than that of DPS.

Observation 2: With the same packet arrival rate for DPS as that of DPS+NC-JT when RU is about 20%, RU of DPS is reduced to 13.3%. Almost same cell average throughput can be achieved by both schemes, considerable UPT gain can be observed at 50% UPT.

Proposal: Clarify RU statistics for NC-JT: whether the used resource is double counted or not?

- Conclusion

In this contribution, we have the following observations according to our evaluation results for multi-TRP transmission with non-ideal backhaul. We have following observations and proposal.

Observation 1: With target RU ≈ 20%, DPS+NC-JT can achieve about 7% gain in mean UPT and 2.7% gain in 5% UPT for non-ideal backhaul, but the cell average throughput of DPS+NC-JT is about 34% worse than that of DPS.

Observation 2: With the same packet arrival rate for DPS as that of DPS+NC-JT when RU is about 20%, RU of DPS is reduced to 13.3%. Almost same cell average throughput can be achieved by both schemes, considerable UPT gain can be observed at 50% UPT.

Proposal: Clarify RU statistics for NC-JT: whether the used resource is double counted or not?

References

- RAN1 #94bis Chairman’s Notes.

- RAN1 AdHoc 1901 Chairman's Notes.

Appendix

Table 1: SLS assumption for eMBB multi-TRP enhancement

| Parameters |  | Value |
| --- | --- | --- |
| Duplex, Waveform |  | FDD, OFDM |
| Multiple access |  | OFDMA |
| Scenario |  | Indoor hotspot (InH) |
| Frequency Range |  | FR1 only, 4GHz |
| Inter-BS distance |  | 20m for InH |
| Channel model |  | According to the TR 38.901 |
| Antenna setup and port layouts at TRP |  | 2 Tx ports: (M, N, P, Mg, Ng) = (1,1,2,1,1) for InH 4 Tx ports: (M, N, P, Mg, Ng, Mp, Np) = (8,2,2,1,1,1,2) (dH, dV) = (0.5, 0.8)λ for UMa |
| Antenna setup and port layouts at UE |  | 4Rx Port: (M, N, P, Mg, Ng, Mp,Np) = (1,2,2,1,1,1,2) (dH,dV) = (0.5, 0.5)λ |
| BS Tx power |  | 23dBm for InH |
| BS antenna height |  | 3m for InH |
| UE antenna height & gain |  | Follow TR36.873 |
| UE receiver noise figure |  | 9dB |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | 15kHz |
| Number of RBs |  | 52 |
| Simulation bandwidth |  | 10 MHz |
| Frame structure |  | Slot Format 0 (all downlink) for all slots |
| Configuration for multi-TRP | Cluster | 4 neighboring TRPs for InH |
|  | Maximal number of coordinating TRPs | 2 |
|  | Backhaul assumption | Ideal |
| Transmission scheme |  | Baseline: SU-MIMO, rank 1 or rank 2 per TRP DPS: rank 1 or rank 2 per TRP Multi-TRP: dynamic switching between DPS and NCJT, rank 1 or rank 2 per TRP |
| CSI feedback |  | Feedback assumption - CSI feedback periodicity: 5 ms - Scheduling delay (from CSI feedback to time to apply in scheduling): 4 ms - Subband PMI, subband CQI - Type I codebook - For DPS: DPS CSI of recommended TRP; for DPS+NC-JT: CSI of recommended transmission scheme: DPS or NCJT |
| Traffic model |  | FTP model 1 |
| UE receiver |  | MMSE-IRC |
| Channel estimation |  | Ideal |