3GPP TSG RAN WG1 Meeting #96 R1-1901928 Athens, Greece, 25th February – 1st March, 2019 CR-Form-v11.2. CHANGE REQUEST. 38.213; CR; DRAFT; rev; -; Current version:; 15.4.0. For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests.. Proposed change affects:; UICC apps; ME; X; Radio Access Network; X; Core Network. Title:; Draft CR on obtaining P0_Nominal for virtual PHR on SCell without RACH. Source to WG:; ZTE. Source to TSG:; RAN1. Work item code:; NR_newRAT-Core; Date:; 2019-2-15. Category:; F; Release:; Rel-15. Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900.; Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16). Reason for change:; In TS38.213, for virtual type1 PHR, cell specific target received power is obtained via which is used for Msg3 and defined as , where the parameter preambleReceivedTargetPower [11, TS 38.321] (for ) and msg3-DeltaPreamble (for ) are provided by higher layers, or dB if msg3-DeltaPreamble is not provided, for carrier of serving cell . Since Msg3 is the message for RACH, there is no need to configure RACH on all serving cells. For those serving cells which have no RACH configured, the UE CANNOT get the parameter preambleReceivedTargetPower [11, TS 38.321] (for ). It is proposed to address the issue as follows: - For those serving cells which have no preambleReceivedTargetPower configured, the UE uses the parameter for non SUL carrier f and primary cell c for virtual PHR calculation.. -. Summary of change:; Adding the following sentence for the serving cells which have no preambleReceivedTargetPower configured. - When the parameter preambleReceivedTargetPower is not provided for serving cell c, the UE uses the parameter for non SUL carrier f and primary cell c.. Consequences if not approved:; For those serving cells which have no RACH configured, the UE CANNOT get the parameter to calculate virtual PHR.. Clauses affected:; 7.7.1. Y; N. Other specs; X; Other core specifications; TS/TR ... CR .... affected:; X; Test specifications; TS/TR ... CR .... (show related CRs); X; O&M Specifications; TS/TR ... CR .... Other comments:.

<Unchanged part omitted>

### 7.7.1	Type 1 PH report

<Unchanged part omitted>

If the UE determines that a Type 1 power headroom report for an activated serving cell is based on a reference PUSCH transmission then, for PUSCH transmission occasion  on active UL BWP  of carrier  of serving cell , the UE computes the Type 1 power headroom report as

[dB]

where  is computed assuming MPR=0 dB, A-MPR=0 dB, P-MPR=0 dB. TC = 0 dB. MPR, A-MPR, P-MPR and TC are defined in [8-1, TS 38.101-1] and [8-2, TS38.101-2]. The remaining parameters are defined in Subclause 7.1.1 where  and  are obtained using  and p0-PUSCH-AlphaSetId = 0,  is obtained using PathlossReferenceRS-Id = 0, and . When the parameter preambleReceivedTargetPower is not provided for serving cell c, the UE uses the parameter  for non SUL carrier f and primary cell c.

If a UE is configured with two UL carriers for a serving cell and the UE determines a Type 1 power headroom report for the serving cell based on a reference PUSCH transmission, the UE computes a Type 1 power headroom report for the serving cell assuming a reference PUSCH transmission on the UL carrier provided by pusch-Config. If the UE is provided pusch-Config for both UL carriers, the UE computes a Type 1 power headroom report for the serving cell assuming a reference PUSCH transmission on the UL carrier provided by pucch-Config. If pucch-Config is not provided to the UE for any of the two UL carriers, the UE computes a Type 1 power headroom report for the serving cell assuming a reference PUSCH transmission on the non-supplementary UL carrier.

<Unchanged part omitted>