3GPP TSG RAN WG1 #96	R1-1902295

# Athens, Greece, 25th February – 1st March, 2019

Agenda item:	7.2.5.4

Source:	Samsung

Title:	OAM functions to support RIM operation

Document for:	Discussion and decision

# Introduction

This contribution discusses corresponding OAM functions to support RIM operation.

# OAM functions to support RIM operation

The following OAM functions were agreed in RAN1 AH#1901:

Agreement - OAM configures gNB to turn ON or OFF RIM-RS monitoring - OAM configures gNB with information required for periodic RIM-RS monitoring Agreement - The OAM configures gNBs to report the all necessary information derived from the detected RIM-RS

According to the SI progress, following additional OAM functions can be considered:

- OAM function 1: OAM sends remote interference mitigation scheme to aggressor

- OAM function 2: OAM restores original configuration at aggressor side

- OAM function 3: OAM stops RS transmission at victim side

It is our understanding that above OAM functions 1 and 2 can be implemented without specification support. On the other hand, above OAM function 3 would require standardized way.

Observation: gNB can apply RIM scheme and restore original configuration by implementation.

Proposal 1: The OAM configures gNB to stop RIM-RS transmission.

In RAN1 AH#1901, some specific OAM configurations were agreed which includes:

- RIM-RS configurations including sequence, time-frequency resource, periodicity, etc.

- Reference point for RIM-RS sequence mapping in frequency

- The initialization seed for RIM-RS

Therefore, they should be included in OAM configuration.

Proposal 2: As per RAN1 AH#1901 agreements, OAM configuration includes at least following information:

- RIM-RS configurations including sequence, time-frequency resource, periodicity, etc.

- Reference point for RIM-RS sequence mapping in frequency domain

- The initialization seed for RIM-RS

For backhaul-based coordination, RAN3 have provided their recommendation. Some aspects including gNB set configuration, intra-set and inter-set coordination would require OAM function. The TR [2] states following with respect to OAM:

- gNB set configuration: - The grouping of gNBs into sets, the set ID, the RIM-RS configuration and the associated RIM-RS radio resources for sending and/or receiving the RIM-RS are performed by the OAM system - Intra-set coordination: - For the normative phase, intra-set gNB-to-gNB coordination inside victim and aggressor gNB sets is not recommended by RAN3. - Inter-set coordination: The level of impact in the 5GC (AMF) and core network OAM system depends on the level of required functionality. - Solution 1: The solution involves gNBs registering to the AMF using Set IDs configured by the OAM system. - Solution 2: the local RAN OAM system enables mapping in gNBs from received RIM-RS to a globally unique set ID, and a DNS solution (out of 3GPP scope) is used to retrieve TAI / Global gNB ID of one or more gNBs of the set.

Proposal 3: For backhaul-based coordination, consider RAN3 recommendation including gNB set configuration, intra-set and inter-set coordination where details are RAN3 work scope.

# Conclusion

The following is proposed based on above discussion:

Observation: gNB can apply RIM scheme and restore original configuration by implementation.

Proposal 1: The OAM configures gNB to stop RIM-RS transmission.

Proposal 2: As per RAN1 AH#1901 agreements, OAM configuration includes at least following information:

- RIM-RS configurations including sequence, time-frequency resource, periodicity, etc.

- Reference point for RIM-RS sequence mapping in frequency domain

- The initialization seed for RIM-RS

Proposal 3: For backhaul-based coordination, consider RAN3 recommendation including gNB set configuration, intra-set and inter-set coordination where details are RAN3 work scope.

# References

[1] RP-182864, Revised WID on Cross Link Interference (CLI) handling and Remote Interference Management (RIM) for NR, LGE

[2] 3GPP TR38.866 Study on remote interference management for NR