3GPP TSG RAN WG1 Meeting #96		                 R1-1902533

Athens, Greece, 25th February – 1st March, 2019

Agenda Item:	7.2.1.1

Source: 	LG Electronics

Title: 	Channel Structure for Two-Step RACH

Document for:	Discussion and Decision

- Introduction

In this contribution, we discuss on channel structure for 2-step RACH.

Discussion

  - Mapping between the PRACH preamble and the time-frequency resource of PUSCH in msgA+DMRS

Multiplexing scheme for PUSCH

For 4-step RACH, PUSCH for msg3 is assigned by UL grant in RAR message. The resource is UE specifically assigned in FDM/TDM manner. Also, for PUSCH for msg3, since single symbol front-loaded DMRS of configured type 1 on DMRS port 0 is used and the remaining REs in the symbols are not used for any PUSCH transmission, the PUSCH is allowed for single UE.

For 2-step RACH procedure, msgA is comprised with PRACH preamble and PUSCH. Similarly, for PUSCH in msgA, FDM/TDM resource assignment can be allowed. In figure 1, the example of PUSCH multiplexing schemes are depicted.

(a) FDM                                (b) TDM

(c) FDM/TDM                (d) FDM/CDM

Figure 1. Example of multiplexing scheme for PUSCH in msgA

In FDM case, resources of PUSCH with narrow bandwidth and many OFDM symbols could be assigned. Main advantage is to enjoy UL coverage enhancement using several OFDM symbols. On the other hand, the PUSCH resource including DMRS RE and data RE could suffer inter-carrier interference from adjacent PUSCH resources.

In TDM case, resource of PUSCH with wide bandwidth and small OFDM symbols could be assigned. Main advantage is to obtain frequency diversity gain from wide bandwidth. However, since the number of OFDM symbols is small, UL coverage is limited. Also, PUSCH resource may suffer inter-symbol interference from adjacent PUSCH symbols.

Also, in both FDM and TDM cases, since PUSCH resources are orthogonally designated, if the reserved resource of PUSCH is not selected by UE, the non-selected resource(s) is/are wasted.

In CDM case, resources of PUSCH with wide bandwidth and many OFDM symbols could be assigned. In this case, UL coverage enhancement can be expected. Also, impact of inter-carrier and inter-symbol could be reduced. Also, resource utilization efficiency could be increased. On the other hand, effect of inter-layer interference should be carefully investigated, especially in time misalignment case.

Proposal 1:

- Investigate the pros and cons of the multiplexing schemes (e.g., FDM, TDM, CDM) for PUSCH in msgA

  - In terms of UL coverage, channel estimation, inter-symbol/inter-carrier interference, resource utilization, etc.

  - For CDM case, effect of inter-layer interference should be carefully investigated, especially in time misalignment case.

Multiple Preamble to Single PUSCH resource

For 4-step RACH, PUSCH for msg3 is assigned by UL grant which is associated with RARIP. Similarly, for 2-step RACH, PUSCH in msgA can be designated for each RAPID for the case of FDM and TDM. Furthermore, if CDM is allowed, it is need to allow that single PUSCH resource is corresponded with multiple RAPIDs. In this case, multiple DMRS resources/sequences and multiple scrambling sequences are required for decoupling the mixed multiple data on a PUSCH resource. Also, a certain DMRS resource/sequence and scrambling sequences can be associated with a RAPID.

Proposal 2:

- If CDM is allowed for PUSCH in msgA, multiple RAPIDs are corresponded to single PUSCH resource.

  - Multiple DMRS resources/sequences and multiple scrambling sequences can be used.

  - A certain DMRS resource/sequence and scrambling sequences can be associated with a RAPID.

Rel-15 NR UL DMRS sequence is designed using base sequence (i.e. Gold sequence for CP-OFDM and low-PAPR sequence for with transform precoding) and orthogonal code cover (i.e. frequency domain OCC and time domain OCC). For PUSCH in msgA, both base sequence and OCC can be applied. However, when it comes to time misalignment, we need to study whether OCC is applicable for DMRS sequence of PUSCH in msgA. Also, it is necessity to discuss how to select initialization seed value for base sequence.

For Rel-15 NR UL DMRS, two types of physical layer resource are defined, i.e., dmrs-TypeA-Position (comb-2) and dmrs-TypeB-Position. Same with msg3 for 4-step RACH, dmrs-TypeA-Position can be applied for PUSCH in msgA. In this case, we need to discuss whether all of two sets of frequency resource (i.e. REs with even index and REs with odd index) are used as DMRS physical layer resource of PUSCH in msgA. We can see some benefits of using two sets of frequency resource for DMRS of PUSC in msgA. One advantage is that if it is allowed to use all of two sets of frequency resource, larger number of UE can be multiplexed within PUSCH resource. Other benefit is that when (if allowed) two PUSCH resources which the length of frequency resource is different are multiplexed, different set of frequency resource can be assigned as DMRS physical resource for each PUSCH resource, which provides better channel estimation performance.

Proposal 3:

- If CDM is allowed for PUSCH in msgA, study how to utilize DMRS resource and sequence.

  - Whether OCC is applicable for DMRS sequence of PUSCH in msgA

  - How to select initialization seed value for base sequence (i.e. Gold sequence for CP-OFDM and low-PAPR sequence for with transform precoding)

  - Whether all of two sets of frequency resource (i.e. REs with even index and REs with odd index) are used as DMRS physical layer resource of PUSCH in msgA.

RO mapping for 2-step RACH

One possible discussion point is whether RO is shared between 2-step RACH and 4-step RACH. If RO sharing is allowed, contention-based PRACH preamble for 2-step RACH can be configured among the remaining PRACH preambles in the configured RO for 4-step RACH, hence the purpose of PRACH transmission (i.e., 2-step RACH or 4-step RACH) can be identified to gNB. Also, in RO sharing case, PUSCH in msgA can be assigned at the PUSCH slot following RACH slot.

If RO sharing is not allowed, RO is configured for 2-step RACH. For RO configuration for 2-step RACH, we can consider two alternatives. (1) First alternative is to reuse RACH configuration table for 4-step RACH. This configuration table is designed based on the assumption that most of OFDM symbols within RACH slot or within second half RACH slot are used as ROs. Hence, for this case, it is possible that RO and PUSCH can be multiplexed at different slots. In figure 2 (a), slot level time multiplexing is depicted as an example of multiplexing of RACH and PUSCH of msgA.

(2) Second alternative is to allow new configuration that OFDM symbols within a first half RACH slot are used as RO. The OFDM symbols following RO can be assigned for PUSCH of msgA. In figure 2 (b), symbol level time multiplexing is depicted as an example of multiplexing of RACH and PUSCH of msgA.

(a) Slot level time multiplexing

(b) Symbol level multiplexing within a slot

Figure 2. Example of multiplexing between RACH and PUSCH of msgA

Proposal 4:

- For RO mapping for 2-step RACH, study whether RO is shared between 2-step RACH and 4-step RACH.

- For RO configuration for 2-step RACH, study the following two alternatives:

  - Alt.1: Reuse RACH configuration table for 4-step RACH

  - Alt.2: Allow new configuration that OFDM symbols within a first half RACH slot are used as RO

  - Supported MCS(s) and time-frequency resource size(s) of PUSCH in msgA etc.

In 4-step RACH, MCS for msg3 is assigned by UL grant in RAR message. So, depending on UE channel state, gNB can designate MCS from low index to high index. Also, time/frequency resource for PUSCH can be assigned depending on the selected MCS level and required coverage. On the other hand, for 2-step RACH, it is hard to allow flexible MCS selection. If UE selects MCS level for UL transmission depending on DL measurement result, it could not applicable for UL transmission because not only channel state but also interference level are quite different between DL channel and UL channel. Furthermore, depending on the MCS level, the required amount of resource of PUSCH for msgA is changed. So, if several MCS levels are allowed, many type of PUSCH resource are defined and pre-assigned. It is not good in terms of resource utilization. Therefore, for PUSCH in msgA, it is desired to use very limited number of MCS level (e.g. one or two MCS levels). For example, only QPSK for CP-OFDM is applied for PUSCH in msgA, and two types of coding rate are used.

Proposal 5:

- For PUSCH in msgA, use very limited number of MCS level (e.g. one or two MCS levels).

  - E.g., only QPSK for CP-OFDM is applied for PUSCH in msgA, and two types of coding rate are used.

As discussed, if multiple MCS levels are allowed for PUSCH transmission, multiple types of PUSCH resource are defined depending on MCS level. Consequently, if PUSCH resource is corresponded with RAPID, the RAPID can be also associated with MCS level. So, if UE decide suitable MCS level for PUSCH transmission, UE may select RAPID which is associated with the MCS level.

Furthermore, if multiple sets of DMRS frequency resource is allowed, it can be defined that each DMRS frequency resource is associated with MCS level. For example, if two different PUSCH resources (e.g., one is larger frequency resource for lower MCS level, and another is smaller frequency resource for higher MCS level) are assumed, two different frequency resource sets are designated for each PUSCH resources. In figure 2, an example of PUSCH resource and DMRS RE designation depending on MCS level is depicted.

Figure 2. An Example of PUSCH resource and DMRS RE designation depending on MCS level

Proposal 6:

- If multiple MCS levels are allowed for PUSCH transmission, multiple types of PUSCH resource are defined depending on MCS level.

  - RAPID can be associated with MCS level.

- If multiple sets of DMRS frequency resource is allowed, it can be defined that each DMRS frequency resource is associated with MCS level.

Conclusion

In this contribution, we discuss on channel structure for two-step RACH. We proposed as follows:

Mapping between the PRACH preamble and the time-frequency resource of PUSCH in msgA+DMRS

Proposal 1:

- Investigate the pros and cons of the multiplexing schemes (e.g., FDM, TDM, CDM) for PUSCH in msgA

  - In terms of UL coverage, channel estimation, inter-symbol/inter-carrier interference, resource utilization, etc.

  - For CDM case, effect of inter-layer interference should be carefully investigated, especially in time misalignment case.

Proposal 2:

- If CDM is allowed for PUSCH in msgA, multiple RAPIDs are corresponded to single PUSCH resource.

  - Multiple DMRS resources/sequences and multiple scrambling sequences can be used.

  - A certain DMRS resource/sequence and scrambling sequences can be associated with a RAPID.

Proposal 3:

- If CDM is allowed for PUSCH in msgA, study how to utilize DMRS resource and sequence.

  - Whether OCC is applicable for DMRS sequence of PUSCH in msgA

  - How to select initialization seed value for base sequence (i.e. Gold sequence for CP-OFDM and low-PAPR sequence for with transform precoding)

  - Whether all of two sets of frequency resource (i.e. REs with even index and REs with odd index) are used as DMRS physical layer resource of PUSCH in msgA.

Proposal 4:

- For RO mapping for 2-step RACH, study whether RO is shared between 2-step RACH and 4-step RACH.

- For RO configuration for 2-step RACH, study the following two alternatives:

  - Alt.1: Reuse RACH configuration table for 4-step RACH

  - Alt.2: Allow new configuration that OFDM symbols within a first half RACH slot are used as RO

Supported MCS(s) and time-frequency resource size(s) of PUSCH in msgA etc.

Proposal 5:

- For PUSCH in msgA, use very limited number of MCS level (e.g. one or two MCS levels).

  - E.g., only QPSK for CP-OFDM is applied for PUSCH in msgA, and two types of coding rate are used.

Proposal 6:

- If multiple MCS levels are allowed for PUSCH transmission, multiple types of PUSCH resource are defined depending on MCS level.

  - RAPID can be associated with MCS level.

- If multiple sets of DMRS frequency resource is allowed, it can be defined that each DMRS frequency resource is associated with MCS level.

Reference

[1] RP-182894, “New work item: 2-step RACH for NR”, ZTE Corporation, Sanechips

- Annex

Work Scope for NR 2-step RACH [1] - 2-step RACH [RAN1, RAN2] - 2-step RACH shall be able operate regardless of whether the UE has valid TA or not. - 2-step RACH is applicable to any cell size supported in Rel-15 NR; - 2-step RACH is applied for RRC_INACTIVE , RRC_CONNECTED and RRC_IDLE state - Specify contention-based 2-step RACH procedure (RAN2) - Channel structure of msgA is Preamble and PUSCH carrying payload (RAN1) - Only reuse the Rel-15 NR PRACH Preambles design. - Only reuse the Rel-15 NR PUSCH including Rel-15 DMRS for transmission of payload of msgA) - No new CP length and no sub-PRB guard subcarrier(s) Note 1: The above sub-bullet is to ensure that signal structure optimizations for any specific cell size (e.g. cells with RTT larger than Rel-15 PUSCH CP duration) are not pursued. - Specify the mapping between the PRACH preamble and the time-frequency resource of PUSCH in msgA+ DMRS - PRACH Preamble and PUSCH in a msgA is TDMed - Specify the supported MCS(s) and time-frequency resource size(s) of PUSCH in msgA - Consider the msgA payload contents determined by RAN2 - Specify power control of PUSCH of msgA - Specify msgA’s content: to include the equivalent contents of msg3 of 4-step RACH (RAN2/RAN1) - Inclusion of UCI in msgA is not precluded - Specify msgB’s content: to include the equivalent contents of msg2 and msg4 of 4-step RACH (RAN1/RAN2) - Contention resolution for 2-step RACH (RAN2) - Design of RNTI for msgB of 2-step RACH (RAN2) - Specify the fall back procedure from 2-step RACH to 4-step RACH (RAN2/RAN1) - All triggers for Rel-15 NR 4-step RACH are applied for 2-step RACH except for SI Request and BFR which are up to RAN2 discussion - No new triggers for 2 step RACH