3GPP TSG RAN WG1 #96	R1-1902692

Athens, Greece, 25th February – 1st March, 2019

Agenda Item:	7.2.4.1.1

Source: 	NEC

Title:	Physical layer structure for NR V2X

Document for:	Discussion/Decision

Introduction

In the last RAN1 meeting, issues related to physical layer structure for NR V2X were discussed with following agreements and working assumptions [1]:

Agreements:

- Confirm the working assumption

  - Working assumption: only one SL BWP is configured in a carrier for a NR V2X UE

Agreements:

- Configuration for SL BWP is separated from Uu BWP configuration signalling.

  - UE is not expected to use different numerology in the configured SL BWP and active UL BWP in the same carrier at a given time.

    - FFS the time scale

    - FFS relation to DL BWP including initial Uu BWP

    - FFS relation in terms of frequency location and bandwidth

Agreements:

- For time domain resources of a resource pool for PSSCH,

  - Support the case where the resource pool consists of non-contiguous time resources

    - FFS details including granularity

- For frequency domain resources of a resource pool for PSSCH,

  - Down select following options:

    - Option 1: The resource pool always consists of contiguous PRBs

    - Option 2: The resource pool can consist of non-contiguous PRBs

Agreements:

- Multiple DMRS patterns in time domain are supported for PSSCH

  - FFS: Whether a DMRS pattern is selected based on the subcarrier spacing

  - FFS: Single or multiple DMRS pattern(s) per a resource pool

  - FFS: How TX UE and RX UE can be aligned in terms of the DMRS pattern used for PSSCH

  - FFS: RE mapping, sequence generation

- Continue to study DMRS pattern in frequency domain for PSSCH

  - E.g. whether multiple patterns are supported, whether PDSCH/PUSCH DMRS configuration 1 or 2 is reused.

Agreements:

- Support PT-RS for PSSCH for FR2

In this contribution, we will discuss and give our views on resource pool, control and data multiplexing, DMRS, AGC settling, and other physical layer structure related issues.

Discussion

Resource pool

The structure of resource pool for PSSCH was discussed last meeting. As to time domain granularity of a PSSCH resource pool, in our view the symbol level should be supported, this can facilitate the co-existence between sidelink and Uu on the shared carrier, considering that symbol level resource mapping has already been supported for PUSCH. Furthermore, as CP-OFDM is supported for NR sidelink, resources for PSSCH has no need to be continuous in frequency domain, frequency domain resources for a resource pool are not necessarily to be contiguous any more, this can increase the flexibility on resource pool configuration and also increase the frequency diversity of PSSCH.

Proposal 1:

- Symbol level granularity for PSSCH resource pool should be supported in NR V2X;

- The resource pool for PSSCH should be able to consist of non-contiguous resources in frequency domain.

Control and data multiplexing

If transient period is not needed for Option 3 is confirmed by RAN4 later, there are two FFS issues on control and data multiplexing. The first one is how to determine the starting symbol of PSCCH and associated PSSCH in Option 3. In our view, PSCCH should start from symbol#0. As some UEs may settle AGC fast in the first symbol, and these UEs are able to receive PSCCH transmitted in the symbol, which can increase the decoding ratio of PSCCH. On the contrary, if PSSCH is transmitted in symbol#0, depending on the DMRS design of PSCCH and PSSCH, the PSSCH transmitted in this symbol may not be used for decoding of the whole PSSCH.

Proposal 2:

- Some redundancy of PSCCH should be transmitted in symbol#0 of a slot.

The other FFS issue is if additional multiplexing options need to be supported to increase PSCCH coverage. In our view, the number of symbols that can be used for one PSCCH can be configurable in Option 3. In other words, if larger coverage is needed then more symbols can be configured for PSCCH, and vice versa. Hence, there is no need to support other options just for the purpose of PSCCH coverage enhancement.

Proposal 3:

- Only Option 3 with configurable number of PSCCH symbols should be supported for PSCCH and PSSCH multiplexing.

DMRS for PSCCH and PSSCH

For the CP-OFDM waveform, DMRS for PDCCH in NR Uu can be reused for PSCCH, the DMRS is inserted continuously in time domain and have ¼ density in frequency domain, which should be sufficient to guarantee the performance of PSCCH.

Proposal 4:

- NR PSCCH reuses DMRS design for NR PDCCH.

Physical data channel in NR Uu supports two DMRS configurations, and for each DMRS configuration, DMRS pattern can be either single-symbol or double-symbol. For single-symbol DMRS configuration 1 or 2, up to 4 or 6 DMRS ports are supported, and the number of supported DMRS ports are double for double-symbol DMRS pattern. Considering the multi-layers transmission requirement on NR sidelink and also the DMRS overhead, single-symbol DMRS pattern should be used for PSSCH. And for CP-OFDM waveform, DMRS configuration 2 is preferable than DMRS configuration 1, since the former has lower frequency domain density and could further reduce the DMRS overhead given DMRS and data can be multiplexed in the same symbol.

Proposal 5:

- At least for NR PSSCH using CP-OFDM waveform, configuration 2 DMRS with single-symbol should be used.

As agreed in the last meeting, multiple DMRS patterns are supported in time domain. To completely exploit the benefit of multiple DMRS patterns, a Tx UE should be allowed to select the appropriate DMRS pattern dynamically according to instant channel status. For this purpose, in our view multiple DMRS patterns should be supported in one resource pool, and the DMRS pattern used by the transmitter should be indicated in the associated PSCCH to enable PSSCH decoding and sensing at the receiver side.

Proposal 6:

- Multiple DMRS patterns should be supported in one resource pool, DMRS pattern used by transmitter should be indicated in the associated PSCCH.

Time domain location of data channel DMRS in NR Uu is related to the mapping type of data channel, i.e. for mapping type A, the first DMRS is placed in the 2nd or 3rd OFDM symbol within the slot, and for mapping type B, the first DMRS is placed in the first data channel symbol. If Option 3 is used for PSCCH and PSSCH multiplexing, both mapping type A and mapping type B may occur for PSSCH in one slot, to guarantee the accuracy of channel estimation for each part of PSSCH, it is preferable to insert DMRS separately for each PSSCH part.

Figure 1 DMRS for PSCCH and different mapping types of PSSCH in Option 3

Proposal 7:

- In Option 3, DMRS for PSSCH part with mapping type A and PSSCH part mapping type B should be inserted separately.

AGC settling and Gap

In LTE V2X the first whole symbol of a subframe is used for AGC settling, and the last whole symbol of a subframe is reserved for Tx/Rx switching (Gap). According to the agreements in RAN1#94[2], the time needed for AGC settling and Tx/Rx switching may be (up to RAN4 confirmation) shorter than half symbol for 15 kHz and 30 kHz SCS. Hence, for 15 kHz and 30 kHz SCS, there is possibility to further reduce the overhead of AGC and gap in NR V2X.

In case of the duration of one symbol is longer than the time for AGC settling plus the time for Tx/Rx switching, one example for AGC settling and Tx/Rx switching is given in Figure 2. As Tx/Rx switching is only necessary for the UE switching Tx/Rx behavior in next slot, there is no gap for Tx/Rx switching from system point of view. For a UE is transmitting in the current slot and going to receiving in the next slot, it can perform Tx to Rx switching at the beginning of the next slot, and then use the remaining part of the first symbol for AGC settling. On the contrary, if the UE is receiving at the current slot and going to transmitting in the next slot, if can perform the Rx to Tx switching at the end of the current slot. For a UE receiving continuously, it only needs to settle AGC at the beginning of the first symbol. In this way, all symbols in a slot can be used for transmission.

In case of the duration of one symbol cannot accommodate both AGC and Tx/Rx switching, LTE sidelink design can be reused, i.e. reserving one symbol for AGC and Tx/Rx switching respectively.

Furthermore, as not all UEs need to settle AGC at the beginning of a slot, in another word, some UE is capable to receive and decoding the first symbol, it is beneficial to transmit data or control information rather than an AGC signal in the first symbol.

Figure 2 AGC and/or Tx/Rx switching time for a UE switching from Tx to Rx, for a switching from Rx to Tx, and for a UE receiving continuously

Observation 1:

- Tx/Rx switching time is only necessary for a UE switching Tx/Rx behaviour in the next slot, in case of the duration of one symbol is longer than the time for AGC plus the time for Tx/Rx switching, there is no need to configure system gap for Tx/Rx switching.

Conclusion

In this contribution, we discussed the issues related to physical layer structure in NR-V2X, we have following observation and proposals:

Observation 1:

- Tx/Rx switching time is only necessary for a UE switching Tx/Rx behaviour in the next slot, in case of the duration of one symbol is longer than the time for AGC plus the time for Tx/Rx switching, there is no need to configure system gap for Tx/Rx switching.

Proposal 1:

- Symbol level granularity for PSSCH resource pool should be supported in NR V2X;

- The resource pool for PSSCH should be able to consist of non-contiguous resources in frequency domain.

Proposal 2:

- Some redundancy of PSCCH should be transmitted in symbol#0 of a slot.

Proposal 3:

- Only Option 3 with configurable number of PSCCH symbols should be supported for PSCCH and PSSCH multiplexing.

Proposal 4:

- NR PSCCH reuses DMRS design for NR PDCCH.

Proposal 5:

- At least for NR PSSCH using CP-OFDM waveform, configuration 2 DMRS with single-symbol should be used.

Proposal 6:

- Multiple DMRS patterns should be supported in one resource pool, DMRS pattern used by transmitter should be indicated in the associated PSCCH.

Proposal 7:

- In Option 3, DMRS for PSSCH part with mapping type A and PSSCH part mapping type B should be inserted separately.

References

- Chairman's Notes RAN1 Ad-Hoc Meeting 1901;

- Chairman's Notes RAN1#94;