3GPP TSG RAN WG1 Meeting #96	                                                                          R1-1902989

Athens, Greece

Feb. 25th – Mar. 1st, 2019

Agenda item:    7.2.2.2.5

Source:             Qualcomm Incorporated

Title:                  Wideband operation for NR-U

Document for:  Discussion and Decision

# Introduction

From the NR-U study item phase, the following has been agreed for NR-U wideband operation:

For DL operation, the following options for BWP-based operation within a carrier with bandwidth larger than 20 MHz can be considered.

- Option 1a: Multiple BWPs configured, multiple BWPs activated, transmission of PDSCH on one or more BWPs

- Option 1b: Multiple BWPs configured, multiple BWPs activated, transmission of PDSCH on single BWP

- Option 2: Multiple BWPs can be configured, single BWP activated, gNB transmits PDSCH on a single BWP if CCA is successful at gNB for the whole BWP

- Option 3: Multiple BWPs can be configured, single BWP activated, gNB transmits PDSCH on parts or whole of single BWP where CCA is successful at gNB

For UL operation, the following options for BWP-based operation within a carrier with bandwidth larger than 20 MHz can be considered.

- Option 1a: Multiple BWPs configured, multiple BWPs activated, transmission of PUSCH on one or more BWPs

- Option 1b: Multiple BWPs configured, multiple BWPs activated, transmission of PUSCH on single BWP

- Option 2: Multiple BWPs can be configured, single BWP activated, UE transmits PUSCH on a single BWP if CCA is successful at UE for the whole BWP

- Option 3: Multiple BWPs can be configured, single BWP activated, UE transmits PUSCH on parts or whole of single BWP where CCA is successful at UE

In RAN1 #Adhoc1901, the following agreements have been reached:

Agreement:

- For wideband operation in DL with a single serving cell operation within a carrier with bandwidth larger than 20 MHz

  - Multiple BWPs can be configured, single BWP activated, gNB may transmit PDSCH on parts or whole of single active BWP where CCA is successful at gNB (i.e., option 2 and 3 from previous agreement)

    - FFS: Restrictions on supportable gaps and combinations of gaps between discontiguous blocks where

      - each block spans contiguous (one or) multiple successful LBT sub-bands

      - each gap spans one or multiple contiguous unsuccessful LBT sub-bands

    - FFS: Transmission bandwidth adaptation delay, potentially different delay for e.g., different number of supported gaps, different transmission bandwidths and different positions of the LBT sub-bands where transmissions occur

    - FFS: Limit on the occupied LBT sub-bands due to regulation and coexistence considerations (not intended to imply that regulation and coexistence considerations will not be addressed)

    - FFS: Whether/how to indicate gNB’s transmitted LBT sub-bands

    - FFS: Enhancements to PDCCH/PDSCH configuration/transmission for the parts of BWP where gNB does not transmit due to CCA failure

Agreement:

- Operation with multiple active BWPs for a carrier on unlicensed bands is not supported for DL or UL at least in Rel-16 NR-U WI.

In this paper, we discuss further details on wideband BWP operation.

# DL wideband BWP operation

For DL wideband BWP operation, option 2 and option 3 have been agreed, and the detailed restrictions are being studied in RAN4.

The option 2 design is straight-forward. However, option 3 is more complex, and at least the following has to be considered:

- How to monitor the coreset

- How to transmit PDSCH

- Subband usage indication

For coreset monitoring for Option 3, the requirement is, the UE can be schedulable if a subset of subbands in the BWP pass LBT. Note that before LBT, the gNB does not know which subband(s) can be used in the COT, and UE does not know which subband(s) will be active and carry PDCCH as well. A simple solution is to define multiple coresets, one in each subband (Figure 1). However, Rel.15 NR has limitations on the number of coresets and the number of search space sets. These limitations will be quickly reached if there are multiple subbands and coresets are defined for each subband.

It turns out Rel.15 NR coreset design is quite flexible to support dynamic subband usage.

One way to configure coreset is forming the coreset by RBs in multiple clusters, where each cluster is over a continuous set of RBs. For example, to support Option 3 over 80MHz, the gNB can configure a coreset with 4 clusters, where each cluster is localized in one subband (Figure 2, Figure 3). The coreset can be interleaved (Figure 2) or localized (Figure 3). When a subset of subbands pass LBT, only the clusters of the coreset within those subbands will be transmitted.

Figure 1. Coreset configuration example with one coreset per subband

In interleaved coreset case, each PDCCH will have some REGs in each cluster (the parameters of the coreset and search space may need to be picked carefully to achieve this goal). In this case, even if some of the clusters are punctured, the UE can still decode the PDCCH using the remaining REGs for the PDCCH.

Figure 2. Coreset configuration example with one interleaved discontinuous coreset over all subbands

In localized coreset case, each PDCCH will be localized in one cluster (the parameters of the coreset and search space may need to be picked carefully to achieve this goal). If some of the clusters are punctured, the PDCCH in the remaining clusters will not be affected and can be decoded.

Figure 3. Coreset configuration example with one localized discontinuous coreset over all subbands

Proposal 1: Rel.15 NR wideband multi-cluster coreset with possible extensions can be used to support subband based access of wideband BWP.

For the PDSCH transmission, since NR uses frequency first mapping, if a PDSCH crosses multiple subbands and some of the subbands are punctured due to LBT failure, the decoding for each CB will be affected. However, NR has natural HARQ retransmission to handle the failures. On the other hand, if there are multiple UEs to schedule in DL, and each UE’s PDSCH is restricted to one subband, then the PDSCH to the UE is either fully punctured or fully transmitted in frequency domain. For example, in Figure 4, we show a case where gNB plans to schedule three UEs. One on subband 0, one on subband 1 and another on subband 2 and 3. LBT passes on subband 0,1,3 and fails on subband 2. PDSCH for UE1 and UE2 are sent without any issues. However, for UE3, since gNB does not have too much time to react to the LBT failure, it continues to transmit portion of the PDSCH on subband 3 but punctures the portion of the PDSCH on subband 2. At a later time, it starts avoiding scheduling on subband 2. More advanced gNBs may have different sets of PDSCH(s) prepared for different LBT outcomes and can even avoid such puncturing.

Figure 4: DL operation with option 3

After the LBT, the gNB knows which subbands can be used in the current COT. It will be beneficial to eventually indicate to the UE the subband usage. For example, when the subband usage is known, the UE can adjust the PDCCH monitoring to allocate the blind decodings to the coreset clusters in the used subbands only. We propose the include the subband usage information in the COT structure indication as well, as similar to duration, the bandwidth is part of the COT structure after all.

Proposal 2. Include the subband usage information as part of the COT structure indication.

# UL wideband BWP operation

For UL transmission, similar to DL transmission, we have Option 2 and Option 3 as well. We believe the definition of Option 2 and Option 3 is not very clear. For example if the UE is granted with a PUSCH in one subband only out of a BWP covers two subbands, the transmission of the PUSCH should happen if the LBT for the subband contains PUSCH passes. Therefore, we propose to update the language for Option 2 and Option 3 as follows:

Instead of a symmetric design, we propose to support Option 2 for UL for the following reasons.

- The UL is scheduled, UE autonomous puncturing in frequency domain may lead to some complexity at gNB receiver.

- UL transmission within gNB COT is subject to Cat 1 immediate transmission or Cat 2 LBT. When Cat 1 immediate transmission is used, the UE will follow the UL grant to transmit in the RBs allocated, which are in the subbands that have passed LBT at gNB side. When Cat 2 LBT is used, since the RBs allocated for UL transmission are in the subbands that passed LBT on gNB side recently, the chances of different LBT outcomes across these subbands are not high.

- The UL transmission is a single PUSCH targeting the gNB. If there is frequency domain puncturing, due to the frequency first mapping of the PUSCH, all CBs in the PUSCH are affected, and likely a retransmission is needed. The similar issue applies for DL as well if single wideband PDSCH is granted. But the DL has the option to serve multiple UEs in separate subbands with different PDSCH.

Proposal 3: NR-U supports Option 2 for UL wideband BWP operation.

# Summary

The proposals and observations made in this contribution are summarized below.

Proposal 1: Rel.15 NR wideband multi-cluster coreset with possible extensions can be used to support subband based access of wideband BWP.

Proposal 2. Include the subband usage information as part of the COT structure indication.

Proposal 3: NR-U supports Option 2 for UL wideband BWP operation.