3GPP TSG RAN WG1 Meeting #96	R1-1903207

# Athens, Greece, February 25th- March 1st, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [DRAFT] CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | XX | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on DCI format 2_3 for SUL cell in TS 38.213 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-02-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In current version of TS 38.213, the description for DCI format 2_3 in Subclause 11.4 is not consistent with TS 38.212 and TS 38.331. According to TS 38.331 and Subclause 7.3.1.3.4 of TS 38.212, each uplink of a serving cell configured with two uplinks can be associated with separate DCI block in DCI format 2_3, whose first bit is indicated by separate field, i.e., startingBitOfFormat2-3 and startingBitOfFormat2-3SUL-v1530, respectively. But only the field “startingBitOfFormat2-3” is captured in TS 38.213. Moreover, the parameter “srs-pcadjustment-state-config” captured in TS 38.213 is not aligned with TS 38.331, and the field “srs-SwitchFromCarrier” contained in “SRS-CarrierSwitching” is not captured in TS 38.213. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add the description for the field “startingBitOfFormat2-3SUL-v1530” in Subclause 11.4 of TS 38.213. - Replace “srs-pcadjustment-state-config” in Subclause 11.4 of TS 38.213 by “srs-PowerControlAdjustmentStates” - Add the description for the field “srs-SwitchFromCarrier” in Subclause 11.4 of TS 38.213. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TS 38.213 is inconsistent with TS 38.212 and TS 38.331. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated Impact Analysis: This CR removes some inconsistency between TS38.213 and TS38.331 on SRS switching. There is no functional change and therefore there is no impact on gNB/UE implementation due to this CR. |  |  |  |  |  |  |  |  |

11.4	SRS switching

DCI format 2_3 is applicable for uplink carrier(s) of serving cells where a UE is not configured for PUSCH/PUCCH transmission or for uplink carrier(s) of a serving cell where srs-PowerControlAdjustmentStates indicates a separate power control adjustment state between SRS transmissions and PUSCH transmissions.

A UE configured by higher layers with parameter SRS-CarrierSwitching is provided

-	a TPC-SRS-RNTI for a DCI format 2_3 by tpc-SRS-RNTI

-	an index of a serving cell where the UE interrupts transmission in order to transmit SRS on one or more other serving cells by srs-SwitchFromServCellIndex

-	an indication of an uplink carrier where the UE interrupts transmission in order to transmit SRS on one or more other serving cells by srs-SwitchFromCarrier, if the serving cell indicated by srs-SwitchFromServCellIndex has two uplink carriers

-	a DCI format 2_3 field configuration type by typeA or typeB

-	for typeA, an index for a set of serving cells is provided by cc-SetIndex, indexes of serving cells in the set of serving cells are provided by cc-IndexInOneCC-Set, and a DCI format 2_3 field includes a TPC command for each serving cell from the set of serving cells and can also include a SRS request for SRS transmission on the set of serving cells

-	for typeB, DCI format 2_3 field includes a TPC command for a serving cell index and can also include a SRS request for SRS transmission on the serving cell

-	an indication for a serving cell for whether or not a field in DCI format 2_3 includes a SRS request by fieldTypeFormat2-3 where a value of 0/1 indicates absence/presence of the SRS request – a mapping for a 2 bit SRS request to SRS resource sets is as provided in [6, TS 38.214]

-	an index for a location in DCI format 2_3 of a first bit for a field for a non-supplementary uplink carrier of the serving cell by startingBitOfFormat2-3

-	an index for a location in DCI format 2_3 of a first bit for a field for a supplementary uplink carrier of the serving cell by startingBitOfFormat2-3SUL-v1530