3GPP TSG RAN WG1 96	R1-1903433

Athens, Greece, February 25th – 1st March 2019

Agenda Item:	7.2.10.1.4

Source:	Ericsson

Title:	offline session notes for AI 7.2.10.1.4 System-level Performance Evaluation for RAT-Dependent Positioning Techniques (Monday session)

Document for:	Discussion, Decision

# 2 Summary of offline discussion

### 2.1 summary for Downlink evaluations

#### 2.1.1 Summary for Scenario 1 – Indoor Open Office

Several companies commented on the suitability of DL-TDOA and angle based methods for Scenario 1[1][3][4][7][11][13][15][9][2][6][8][17]. There is consensus on the fact that DL-TDOA is suitable to meet the regulatory requirements. There are different results as to whether DL-TDOA can meet some of the commercial requirements, using different parameters. Several RS have been studied, with CSI-RS (TRS), LTE-PRS, and NR PRS patterns.

The topic of synchronization is raised by observations in 4 contributions, and there it was observed that for DL-TDOA, synchronization may impact the fulfilment of commercial targets[1][3][4][15].

The companies that have evaluated angle based methods[1][10] observed that DL-AoD positioning technique can meet the regulatory requirements. commercial requirements have also been evaluated.

3 contributions[1][11][17] mentioned the benefits of high resolution estimators (either by interpolation or super-resolution approaches)

Two contributions[11][17] mentioned the importance of maintaining orthogonality/having low interference on the reference signal for positioning.

Based on these observations, the following consensus is proposed. Yellow markings shall be resolved online prior to agreements.

Offline consensus proposal:

Endorse the following text proposal for section 8.4.1.1 in TR 38.855

---------------------------begin TP for 8.4.1.1  in 38.855:-----------------------

based on the agreed scenarios, channel models and simulation assumptions, the following can be observed for horizontal accuracy as described in section 5 of the TR for regulatory and commercial requirements:

- For scenario 1,  12 sources evaluated that DL-TDOA can meet the regulatory requirements.

- For scenario 1,  12 sources evaluated that DL-TDOA can meet the commercial requirements, when  no synchronization error is included in the evaluation. When  synchronization error is modeled, the commercial requirements as described in section 5 of the TR are not met.

- For scenario 1,  1 source evaluated  DL AoD for FR2, and observed that it can meet the regulatory requirements.

- For scenario 1, 1 source evaluated DL AoD for FR2, and observed that it can meet the commercial requirements.

- For scenario 1, 1 source evaluated DL-TDOA+AoD for FR2, and observed that it can meet the regulatory requirements.

- For scenario 1,  1 source evaluated DL-TDOA+AoD for FR2, and observed that it can meet the commercial requirements, when  no synchronization error is included in the evaluation. When  synchronization error is modeled, the commercial requirements for horizontal accuracy are not met.

- For scenario 1, 3 sources evaluated resolution enhancement techniques (interpolation, super resolution) can improve  DL-TDOA methods.

-----End TP------

# 4 References

- R1-1901577	Performance evaluations for NR positioning	Huawei, HiSilicon

- R1-1901717	Evaluation for RAT-Dependent Positioning Techniques	vivo

- R1-1901781	System-level evaluations on Rat-dependent positioning	ZTE Corporation,Sanechips

- R1-1901819	System level performance with explicit link level simulation for downlink DL-TDOA technique MediaTek Inc.

- R1-1901820	System level performance with explicit link level simulation for downlink angle based technique MediaTek Inc.

- R1-1901850	Updated system level simulation results for NR Positioning	Nokia, Nokia Shanghai Bell

- R1-1901983	Updated System-level Performance Evaluation for NR Positioning	CATT

- R1-1902102	Discussion on Performance Evaluation for RAT-Dependent Positioning Techniques	LG Electronics

- R1-1902246	System-level Performance Evaluation for RAT-Dependent Positioning Techniques	Samsung

- R1-1902397	System-level Performance Evaluation for SSB Angle Based Positioning 	Polaris Wireless

- R1-1902514	NR Positioning Evaluation Results	Intel Corporation

- R1-1902550	System level simulations for DL-TDOA	ESA

- R1-1902698	System-level Performance Evaluation of Co-band TBS for TR 38.855	BUPT, ZTE, CAICT

- R1-1902837	Evaluation results for NR positioning	Mitsubishi Electric Co.

- R1-1903021	Evaluation results for RAT-dependent positioning Techniques	Qualcomm Incorporated

- R1-1903054	SLS results for NR positioning potential solutions	 Fraunhofer IIS, Fraunhofer HHI

- R1-1903142	System-level Performance Evaluation for RAT-Dependent Positioning Techniques	Ericsson

- R-19NNNN text proposal for section 8.4 of TR 36.855, Ericsson